# Rustdoc Export: device_envoy_rp

Source directory: /home/carlk/programs/mcu/device-envoy/target/thumbv8m.main-none-eabihf/doc/device_envoy_rp

Page count: 332

# Crate device_ envoy_ rp Copy item path

Path: index.html

# Crate device_ envoy_ rp Copy item path

Source

Expand description

## § device-envoy-rp

Build Pico applications with LED panels, easy WiFi, and composable device abstractions.

device-envoy-rp is a library for building embedded applications in Rust, built on the Embassy framework. It organizes hardware around device abstractions .

A device abstraction is a software encapsulation of hardware that manages timing, tasks, control flow, interrupts, channels, and state within the abstraction.

Rather than replacing HALs or drivers, device-envoy-rp builds on them. It defines device abstractions that expose a small set of simple operations to the rest of the program.

Currently targeting Raspberry Pi Pico 1 and Pico 2 (ARM cores) in device-envoy-rp , and ESP32-C6 and ESP32-S3 in device-envoy-esp . RISC-V core support on RP exists but is not actively tested.

### § Start From a Template

Want a minimal starting project?

device-envoy-rp-blinky on GitHub

### § Status

⚠️ Alpha / Experimental

The API is actively evolving. Not recommended for production use, but excellent for experimentation, learning, and exploratory projects.

### § Features

LED Strips & Panels - NeoPixel-style (WS2812) LED arrays with 2D text rendering, animation, embedded-graphics support. Provides efficient options for power limiting and color correction.

WiFi (Pico W) - Connect to the Internet with automatic credentials management. On boot, opens a web form if WiFi credentials aren’t saved, then connects seamlessly to a stored network. Requires Pico W; WiFi is not supported on non-W boards.

Audio Player - Play audio clips over I²S hardware with runtime sequencing, volume control, and compression.

Button Input - Button handling with debouncing

Servo Control - Servo positioning and animation

Flash Storage - Type-safe, on-board persistent storage

LCD Display - Text display (HD44780)

IR Remote - Remote control decoder (NEC protocol)

RFID Reader - Card detection and reading (MFRC522)

Clock Sync - Network time synchronization utilities

LED4 Display - 4-digit, 7-segment LED display control with optional animation and blinking

Single LED - Single LED control with animation support

### § Forum

Using Embassy to build applications A place to talk about writing embedded applications with Embassy: sharing code, asking practical questions, and learning what works in practice. Not limited to Pico or ESP boards, or to device-envoy .

### § Videos and Articles

device-envoy: Making Embedded Fun with Rust, Embassy, and Composable Device Abstractions – versions: article or video

How Rust & Embassy Shine on Embedded Devices by Carl M. Kadie and Brad Gibson.

More Rust articles

### § Thanks

Special thanks to Brad Gibson , organizer of the Seattle Rust User Group . He introduced me to Rust programming on microcontrollers, suggested the term device abstraction , and encouraged thinking in terms of shared traits across controller families. Those conversations helped set the goals for device-envoy.

### § Examples & Demos

The project includes examples (single-device tests) in examples/ and demo applications in demos/ showing integration patterns:

#### § Example: animated LED strip

This example cycles a 96-LED strip through red, green, and blue frames.

It shows how device-envoy generates a struct (device abstraction) for an LED strip and then animates a sequence of frames.

use
device_envoy_rp::{
Result
, led_strip::{LedStrip
as _
, Frame1d, colors}};
use
device_envoy_rp::led_strip;
led_strip!
{
    LedStripAnimated {
        pin: PIN_4,
        len:
96
,
    }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
led_strip_animated = LedStripAnimated::new(p.PIN_4, p.PIO0, p.DMA_CH0, spawner)
?
;
// Create a sequence of frames and durations and then animate them (looping, until replaced).
let
frame_duration = embassy_time::Duration::from_millis(
300
);
    led_strip_animated.animate([
        (Frame1d::filled(colors::RED), frame_duration),
        (Frame1d::filled(colors::GREEN), frame_duration),
        (Frame1d::filled(colors::BLUE), frame_duration),
    ]);

    core::future::pending().
await
// run forever
}

For complete, runnable examples (including wiring and setup), see the examples/ and demos/ directories.

Basic LED Examples : Simple on/off control with blinky pattern

LED Strip Examples : Simple animations, color control, text rendering

LED Panel Examples : 12×4, 12×8, and multi-panel configurations with graphics

Button Examples : Debouncing and state handling

Servo Examples : Position sweeps and animation playback

WiFi Examples : WiFi setup, time sync, DNS

Flash Examples : Configuration persistence and data reset

See the examples/ and demos/ directories for complete runnable code.

### § Building & Running

If you just want to use this library, start from the template project: device-envoy-rp-blinky .

If you want to edit this project, see the Development Guide .

### § Policy on AI-assisted development and contributions

The use of AI tools is permitted for development and contributions to this repository. AI may be used as a productivity aid for drafting, exploration, and refactoring.

All code and documentation contributed to this repository must be reviewed, edited, and validated by a human contributor. AI tools are not a substitute for design judgment, testing, or responsibility for correctness.

AGENTS.md contains the general instructions and constraints given to AI tools used during development of this repository.

### § License

Licensed under either:

MIT license (see the repository root LICENSE-MIT file)

Apache License, Version 2.0 (see the repository root LICENSE-APACHE file)

at your option.

## § Glossary

Resources available on the Pico 1 and Pico 2:

PIO ( Programmable I/O ): Pico 1 has 2. Pico 2 has 3.

DMA ( Direct Memory Access ): Both Pico 1 and 2 have 12 channels.

PWM ( Pulse Width Modulation ) Slices: Both Pico 1 and 2 have 8 slices (& 16 channels). These “slices” are unrelated Rust slices.

I2C ( Inter-Integrated Circuit ): Serial control bus used by abstractions like lcd_text . Pico 1 and Pico 2 each provide 2 controllers ( I2C0 , I2C1 ).

## Modules §

audio_ player A device abstraction for playing audio clips over I²S hardware, with runtime sequencing, volume control, and compression. button A device abstraction for buttons with debouncing and press duration detection. clock_ sync A device abstraction that combines NTP time synchronization with a local clock. docs Documentation-only pages for this crate. flash_ block A device abstraction for type-safe persistent storage in flash memory. ir A device abstraction for infrared receivers using the NEC protocol. lcd_ text A device abstraction for HD44780-compatible character LCDs (e.g., 16x2, 20x2, 20x4). led A device abstraction for a single digital LED with animation support. led4 A device abstraction for a 4-digit, 7-segment LED display for text with optional animation and blinking. led2d A device abstraction for rectangular NeoPixel-style (WS2812) LED panel displays. For 1-dimensional LED strips, see the led_strip module. led_ strip A device abstraction for 1-dimensional NeoPixel-style (WS2812) LED strips. For 2-dimensional panels, see the led2d module. rfid A device abstraction for RFID readers using the MFRC522 chip. servo A device abstraction for hobby servos. wifi_ auto A device abstraction that connects a Pico with WiFi to the Internet and, when needed, creates a temporary WiFi network to enter credentials.

## Enums §

Error Define a unified error type for this crate.

## Type Aliases §

Result A specialized Result where the error is this crate’s Error type.

# List of all items

Path: all.html

# List of all items

### Structs

audio_player::AdpcmClip

audio_player::Gain

audio_player::PcmClip

audio_player::SilenceClip

audio_player::Volume

audio_player::audio_player_generated::AudioPlayerGenerated

button::ButtonRp

button::button_watch_generated::ButtonWatchGenerated

clock_sync::ClockSyncRp

clock_sync::ClockSyncStaticRp

clock_sync::ClockSyncTick

clock_sync::UnixSeconds

flash_block::FlashBlockRp

ir::ir_generated::IrGenerated

ir::ir_generated::IrKeplerGenerated

ir::ir_generated::IrMappingGenerated

lcd_text::lcd_text_generated::I2csGenerated

lcd_text::lcd_text_generated::LcdTextGenerated

lcd_text::lcd_text_generated::LcdTextGenerated20x4

led2d::Frame2d

led2d::LedLayout

led2d::Point

led2d::Size

led2d::layout::LedLayout

led2d::led2d_generated::Led2dGenerated

led4::AnimationFrame

led4::Led4Rp

led4::Led4RpStatic

led4::OutputArray

led::led_generated::LedGenerated

led_strip::Frame1d

led_strip::Rgb888

led_strip::led_strip_generated::LedStripGenerated

rfid::RfidRp

rfid::RfidStatic

servo::ServoRp

servo::servo_player_generated::ServoPlayerGenerated

wifi_auto::WifiAutoRp

wifi_auto::fields::TextField

wifi_auto::fields::TextFieldStatic

wifi_auto::fields::TimezoneField

wifi_auto::fields::TimezoneFieldStatic

### Enums

Error

audio_player::AtEnd

button::PressDuration

button::PressedTo

clock_sync::CoreError

ir::IrEvent

ir::KeplerKeys

ir::ir_generated::RemoteKeysGenerated

led2d::Led2dFont

led4::BlinkState

led::LedLevel

led::OnLevel

led_strip::Current

led_strip::Gamma

rfid::RfidEvent

servo::AtEnd

servo::Direction

wifi_auto::WifiAutoEvent

### Traits

audio_player::AudioPlayer

audio_player::Playable

button::Button

clock_sync::ClockSync

flash_block::FlashBlock

ir::Ir

ir::IrKepler

ir::IrMapping

lcd_text::LcdText

led2d::Led2d

led4::Led4

led::Led

led_strip::LedStrip

led_strip::ToRgb8

led_strip::ToRgb888

rfid::Rfid

servo::Servo

servo::ServoPlayer

wifi_auto::WifiAuto

wifi_auto::WifiAutoField

### Macros

audio_player::adpcm_clip

audio_player::audio_player

audio_player::pcm_clip

audio_player::tone

button::button_watch

ir::ir

ir::ir_kepler

ir::ir_keplers

ir::ir_mapping

ir::ir_mappings

ir::irs

lcd_text::i2cs

lcd_text::lcd_text

led2d::led2d

led::led

led_strip::led_strip

led_strip::led_strips

servo::combine

servo::servo

servo::servo_player

### Functions

audio_player::adpcm_clip_generated::AdpcmClipGenerated::adpcm_clip

audio_player::adpcm_clip_generated::AdpcmClipGenerated::pcm_clip

audio_player::pcm_clip_generated::PcmClipGenerated::adpcm_clip

audio_player::pcm_clip_generated::PcmClipGenerated::pcm_clip

clock_sync::h12_m_s

led2d::bit_matrix3x4_font

led2d::render_text_to_frame

led4::circular_outline_animation

servo::combine

servo::linear

### Type Aliases

Result

audio_player::AdpcmClipBuf

audio_player::PcmClipBuf

audio_player::audio_player_generated::AudioPlayerGeneratedPlayable

clock_sync::CoreResult

led4::Animation

led_strip::RGB8

wifi_auto::WifiStack

### Constants

audio_player::CD_44100_HZ

audio_player::NARROWBAND_8000_HZ

audio_player::PRO_48000_HZ

audio_player::VOICE_16000_HZ

audio_player::VOICE_22050_HZ

audio_player::adpcm_clip_generated::AdpcmClipGenerated::ADPCM_DATA_LEN

audio_player::adpcm_clip_generated::AdpcmClipGenerated::PCM_SAMPLE_COUNT

audio_player::adpcm_clip_generated::AdpcmClipGenerated::SAMPLE_RATE_HZ

audio_player::pcm_clip_generated::PcmClipGenerated::ADPCM_DATA_LEN

audio_player::pcm_clip_generated::PcmClipGenerated::PCM_SAMPLE_COUNT

audio_player::pcm_clip_generated::PcmClipGenerated::SAMPLE_RATE_HZ

clock_sync::ONE_DAY

clock_sync::ONE_MINUTE

clock_sync::ONE_SECOND

led4::ANIMATION_MAX_FRAMES

led_strip::colors::ALICE_BLUE

led_strip::colors::ANTINQUE_WHITE

led_strip::colors::AQUA

led_strip::colors::AQUAMARINE

led_strip::colors::AZURE

led_strip::colors::BEIGE

led_strip::colors::BISQUE

led_strip::colors::BLACK

led_strip::colors::BLANCHED_ALMOND

led_strip::colors::BLUE

led_strip::colors::BLUE_VIOLET

led_strip::colors::BROWN

led_strip::colors::BURLYWOOD

led_strip::colors::CADET_BLUE

led_strip::colors::CHARTREUSE

led_strip::colors::CHOCOLATE

led_strip::colors::CORAL

led_strip::colors::CORNFLOWER_BLUE

led_strip::colors::CORNSILK

led_strip::colors::CRIMSON

led_strip::colors::CYAN

led_strip::colors::DARK_BLUE

led_strip::colors::DARK_CYAN

led_strip::colors::DARK_GOLDENROD

led_strip::colors::DARK_GRAY

led_strip::colors::DARK_GREEN

led_strip::colors::DARK_KHAKI

led_strip::colors::DARK_MAGENTA

led_strip::colors::DARK_OLIVE_GREEN

led_strip::colors::DARK_ORANGE

led_strip::colors::DARK_ORCHID

led_strip::colors::DARK_RED

led_strip::colors::DARK_SALMON

led_strip::colors::DARK_SEA_GREEN

led_strip::colors::DARK_SLATE_BLUE

led_strip::colors::DARK_SLATE_GRAY

led_strip::colors::DARK_TURQUOISE

led_strip::colors::DARK_VIOLET

led_strip::colors::DEEP_PINK

led_strip::colors::DEEP_SKY_BLUE

led_strip::colors::DIM_GRAY

led_strip::colors::DODGER_BLUE

led_strip::colors::FIREBRICK

led_strip::colors::FLORAL_WHITE

led_strip::colors::FOREST_GREEN

led_strip::colors::FUCHSIA

led_strip::colors::GAINSBORO

led_strip::colors::GHOST_WHITE

led_strip::colors::GOLD

led_strip::colors::GOLDENROD

led_strip::colors::GRAY

led_strip::colors::GREEN

led_strip::colors::GREEN_YELLOW

led_strip::colors::HONEYDEW

led_strip::colors::HOT_PINK

led_strip::colors::INDIAN_RED

led_strip::colors::INDIGO

led_strip::colors::IVORY

led_strip::colors::KHAKI

led_strip::colors::LAVENDER

led_strip::colors::LAVENDER_BLUSH

led_strip::colors::LAWN_GREEN

led_strip::colors::LEMON_CHIFFON

led_strip::colors::LIGHT_BLUE

led_strip::colors::LIGHT_CORAL

led_strip::colors::LIGHT_CYAN

led_strip::colors::LIGHT_GOLDENROD_YELLOW

led_strip::colors::LIGHT_GRAY

led_strip::colors::LIGHT_GREEN

led_strip::colors::LIGHT_PINK

led_strip::colors::LIGHT_SALMON

led_strip::colors::LIGHT_SEA_GREEN

led_strip::colors::LIGHT_SKY_BLUE

led_strip::colors::LIGHT_SLATE_GRAY

led_strip::colors::LIGHT_STEEL_BLUE

led_strip::colors::LIGHT_YELLOW

led_strip::colors::LIME

led_strip::colors::LIME_GREEN

led_strip::colors::LINEN

led_strip::colors::MAGENTA

led_strip::colors::MAROON

led_strip::colors::MEDIUM_AQUAMARINE

led_strip::colors::MEDIUM_BLUE

led_strip::colors::MEDIUM_ORCHID

led_strip::colors::MEDIUM_PURPLE

led_strip::colors::MEDIUM_SEA_GREEN

led_strip::colors::MEDIUM_SLATE_BLUE

led_strip::colors::MEDIUM_SPRING_GREEN

led_strip::colors::MEDIUM_TURQUOISE

led_strip::colors::MEDIUM_VIOLET_RED

led_strip::colors::MIDNIGHT_BLUE

led_strip::colors::MINT_CREAM

led_strip::colors::MISTY_ROSE

led_strip::colors::MOCCASIN

led_strip::colors::NAVAJO_WHITE

led_strip::colors::NAVY

led_strip::colors::OLD_LACE

led_strip::colors::OLIVE

led_strip::colors::OLIVE_DRAB

led_strip::colors::ORANGE

led_strip::colors::ORANGE_RED

led_strip::colors::ORCHID

led_strip::colors::PALE_GOLDENROD

led_strip::colors::PALE_GREEN

led_strip::colors::PALE_TURQUOISE

led_strip::colors::PALE_VIOLET_RED

led_strip::colors::PAPAYA_WHIP

led_strip::colors::PEACH_PUFF

led_strip::colors::PERU

led_strip::colors::PINK

led_strip::colors::PLUM

led_strip::colors::POWDER_BLUE

led_strip::colors::PURPLE

led_strip::colors::RED

led_strip::colors::ROSY_BROWN

led_strip::colors::ROYAL_BLUE

led_strip::colors::SADDLE_BROWN

led_strip::colors::SALMON

led_strip::colors::SANDY_BROWN

led_strip::colors::SEASHELL

led_strip::colors::SEA_GREEN

led_strip::colors::SIENNA

led_strip::colors::SILVER

led_strip::colors::SKY_BLUE

led_strip::colors::SLATE_BLUE

led_strip::colors::SLATE_GRAY

led_strip::colors::SNOW

led_strip::colors::SPRING_GREEN

led_strip::colors::STEEL_BLUE

led_strip::colors::TAN

led_strip::colors::TEAL

led_strip::colors::THISTLE

led_strip::colors::TOMATO

led_strip::colors::TURQUOISE

led_strip::colors::VIOLET

led_strip::colors::WHEAT

led_strip::colors::WHITE

led_strip::colors::WHITE_SMOKE

led_strip::colors::YELLOW

led_strip::colors::YELLOW_GREEN

servo::SERVO_MAX_US_DEFAULT

servo::SERVO_MIN_US_DEFAULT

# Constant ADPCM_ DATA_ LEN Copy item path

Path: audio_player/adpcm_clip_generated/AdpcmClipGenerated/constant.ADPCM_DATA_LEN.html

device_envoy_rp

::

audio_player :: adpcm_clip_generated :: AdpcmClipGenerated

# Constant ADPCM_ DATA_ LEN Copy item path

Source

pub const ADPCM_DATA_LEN:
usize
= _; // 46_848usize

Expand description

Byte length for compressed (ADPCM) encoding this clip.

See the audio_player module documentation for usage examples.

# Constant PCM_ SAMPLE_ COUNT Copy item path

Path: audio_player/adpcm_clip_generated/AdpcmClipGenerated/constant.PCM_SAMPLE_COUNT.html

device_envoy_rp

::

audio_player :: adpcm_clip_generated :: AdpcmClipGenerated

# Constant PCM_ SAMPLE_ COUNT Copy item path

Source

pub const PCM_SAMPLE_COUNT:
usize
= 92_160;

Expand description

Number of samples for uncompressed (PCM) version of this clip.

# Constant SAMPLE_ RATE_ HZ Copy item path

Path: audio_player/adpcm_clip_generated/AdpcmClipGenerated/constant.SAMPLE_RATE_HZ.html

device_envoy_rp

::

audio_player :: adpcm_clip_generated :: AdpcmClipGenerated

# Constant SAMPLE_ RATE_ HZ Copy item path

Source

pub const SAMPLE_RATE_HZ:
u32
= VOICE_22050_HZ; // 22_050u32

Expand description

Sample rate in hertz for this generated clip.

See the audio_player module documentation for usage examples.

# Function adpcm_ clip Copy item path

Path: audio_player/adpcm_clip_generated/AdpcmClipGenerated/fn.adpcm_clip.html

device_envoy_rp

::

audio_player :: adpcm_clip_generated :: AdpcmClipGenerated

# Function adpcm_ clip Copy item path

Source

pub const fn adpcm_clip() ->
AdpcmClipBuf
<SAMPLE_RATE_HZ, ADPCM_DATA_LEN>

Expand description

const function that returns the compressed (ADPCM) encoding for this clip.

See the audio_player module documentation for usage examples.

# Function pcm_ clip Copy item path

Path: audio_player/adpcm_clip_generated/AdpcmClipGenerated/fn.pcm_clip.html

device_envoy_rp

::

audio_player :: adpcm_clip_generated :: AdpcmClipGenerated

# Function pcm_ clip Copy item path

Source

pub const fn pcm_clip() ->
PcmClipBuf
<SAMPLE_RATE_HZ, PCM_SAMPLE_COUNT>

Expand description

const function that returns the uncompressed (PCM) version of this clip.

See the audio_player module documentation for usage examples.

# Module Adpcm Clip Generated Copy item path

Path: audio_player/adpcm_clip_generated/AdpcmClipGenerated/index.html

device_envoy_rp

::

audio_player :: adpcm_clip_generated

# Module Adpcm Clip Generated Copy item path

Source

Expand description

Sample module generated by the adpcm_clip! macro, showing the generated items.

This page serves as the reference for what a generated ADPCM clip module provides. For first-time readers, start with the audio_player module documentation, then return here for the generated items.

The generated items are in a module (not a struct type) because stable Rust does not support inherent associated types on structs.

Auto-generated.

## Constants §

ADPCM_ DATA_ LEN Byte length for compressed (ADPCM) encoding this clip. PCM_ SAMPLE_ COUNT Number of samples for uncompressed (PCM) version of this clip. SAMPLE_ RATE_ HZ Sample rate in hertz for this generated clip.

## Functions §

adpcm_ clip const function that returns the compressed (ADPCM) encoding for this clip. pcm_ clip const function that returns the uncompressed (PCM) version of this clip.

# Module adpcm_ clip_ generated Copy item path

Path: audio_player/adpcm_clip_generated/index.html

device_envoy_rp

::

audio_player

# Module adpcm_ clip_ generated Copy item path

Source

Expand description

Module containing AdpcmClipGenerated , the sample module generated by the adpcm_clip! macro.

Auto-generated.

## Modules §

Adpcm Clip Generated Sample module generated by the adpcm_clip! macro, showing the generated items.

# Module audio_ player_ generated Copy item path

Path: audio_player/audio_player_generated/index.html

device_envoy_rp

::

audio_player

# Module audio_ player_ generated Copy item path

Source

Expand description

Module containing AudioPlayerGenerated , the sample struct type generated by the audio_player! macro.

Auto-generated.

## Structs §

Audio Player Generated Sample struct type generated by the audio_player! macro, showing methods and associated constants.

## Type Aliases §

Audio Player Generated Playable Trait-object clip source type at VOICE_22050_HZ .

# Struct Audio Player Generated Copy item path

Path: audio_player/audio_player_generated/struct.AudioPlayerGenerated.html

device_envoy_rp

::

audio_player :: audio_player_generated

# Struct Audio Player Generated Copy item path

Source

pub struct AudioPlayerGenerated;

Expand description

Sample struct type generated by the audio_player! macro, showing methods and associated constants.

This page serves as the reference for what a generated audio player type provides. For first-time readers, start with the audio_player module documentation, then return here for a complete list of available methods and associated constants.

The macro also generates AudioPlayerGeneratedPlayable , a trait-object alias for clip sources at the same sample rate.

Auto-generated.

## Implementations §

Source

§

### impl AudioPlayerGenerated

Source

#### pub const SAMPLE_RATE_HZ : u32 = VOICE_22050_HZ

Sample rate used for playback by this generated player type.

See the audio_player module docs for usage.

Source

#### pub const MAX_CLIPS : usize = 16

Maximum number of clips accepted by play(...) .

Source

#### pub const INITIAL_VOLUME : Volume = Volume::MAX

Initial runtime volume relative to Self::MAX_VOLUME .

Source

#### pub const MAX_VOLUME : Volume = Volume::MAX

Runtime volume ceiling for this generated player type.

Source

#### pub fn new ( data_pin: Peri<'static, PIN_8>, bit_clock_pin: Peri<'static, PIN_9>, word_select_pin: Peri<'static, PIN_10>, pio: Peri<'static, PIO1>, dma: Peri<'static, DMA_CH0>, spawner: Spawner, ) -> Result <&'static Self>

Creates and spawns the generated audio player instance.

See the audio_player module docs for usage.

## Trait Implementations §

Source

§

### impl AudioPlayer <VOICE_22050_HZ> for AudioPlayerGenerated

Source

§

#### const SAMPLE_RATE_HZ : u32 = VOICE_22050_HZ

Sample rate in hertz for this generated player type.

Source

§

#### const MAX_CLIPS : usize = 16

Maximum number of clips accepted by

play(...)

for this generated type.

Source

§

#### const INITIAL_VOLUME : Volume = Volume::MAX

Initial runtime volume relative to

Self::MAX_VOLUME

.

Source

§

#### const MAX_VOLUME : Volume = Volume::MAX

Runtime volume ceiling for this generated player type.

Source

§

#### fn play <I>(&self, audio_clips: I, at_end: AtEnd ) where I: IntoIterator <Item = &'static dyn Playable <VOICE_22050_HZ>>,

Starts playback of one or more static audio clips.

Read more

Source

§

#### fn stop (&self)

Stops current playback as soon as possible.

Read more

Source

§

#### async fn wait_until_stopped (&self)

Waits until playback is stopped.

Read more

Source

§

#### fn set_volume (&self, volume: Volume )

Sets runtime playback volume relative to the generated player’s max volume.

Read more

Source

§

#### fn volume (&self) -> Volume

Returns the current runtime playback volume relative to max volume.

Read more

## Auto Trait Implementations §

§

### impl Freeze for AudioPlayerGenerated

§

### impl RefUnwindSafe for AudioPlayerGenerated

§

### impl Send for AudioPlayerGenerated

§

### impl Sync for AudioPlayerGenerated

§

### impl Unpin for AudioPlayerGenerated

§

### impl UnwindSafe for AudioPlayerGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Type Alias Audio Player Generated Playable Copy item path

Path: audio_player/audio_player_generated/type.AudioPlayerGeneratedPlayable.html

device_envoy_rp

::

audio_player :: audio_player_generated

# Type Alias Audio Player Generated Playable Copy item path

Source

pub type AudioPlayerGeneratedPlayable = dyn
Playable
<VOICE_22050_HZ>;

Expand description

Trait-object clip source type at VOICE_22050_HZ .

See the audio_player module documentation for usage examples.

# Constant CD_ 44100_ HZ Copy item path

Path: audio_player/constant.CD_44100_HZ.html

device_envoy_rp

::

audio_player

# Constant CD_ 44100_ HZ Copy item path

pub const CD_44100_HZ:
u32
= 44_100; // 44_100u32

Expand description

Compact-disc sample rate.

# Constant NARROWBAND_ 8000_ HZ Copy item path

Path: audio_player/constant.NARROWBAND_8000_HZ.html

device_envoy_rp

::

audio_player

# Constant NARROWBAND_ 8000_ HZ Copy item path

pub const NARROWBAND_8000_HZ:
u32
= 8_000; // 8_000u32

Expand description

Narrowband telephony sample rate.

# Constant PRO_ 48000_ HZ Copy item path

Path: audio_player/constant.PRO_48000_HZ.html

device_envoy_rp

::

audio_player

# Constant PRO_ 48000_ HZ Copy item path

pub const PRO_48000_HZ:
u32
= 48_000; // 48_000u32

Expand description

Pro-audio sample rate.

# Constant VOICE_ 16000_ HZ Copy item path

Path: audio_player/constant.VOICE_16000_HZ.html

device_envoy_rp

::

audio_player

# Constant VOICE_ 16000_ HZ Copy item path

pub const VOICE_16000_HZ:
u32
= 16_000; // 16_000u32

Expand description

Wideband voice sample rate.

# Constant VOICE_ 22050_ HZ Copy item path

Path: audio_player/constant.VOICE_22050_HZ.html

device_envoy_rp

::

audio_player

# Constant VOICE_ 22050_ HZ Copy item path

pub const VOICE_22050_HZ:
u32
= 22_050; // 22_050u32

Expand description

Common low-memory voice/music sample rate.

Convenience constant: any sample rate supported by your hardware setup may be used.

# Enum AtEnd Copy item path

Path: audio_player/enum.AtEnd.html

device_envoy_rp

::

audio_player

# Enum AtEnd Copy item path

pub enum AtEnd {
    Loop,
    Stop,
}

Expand description

End-of-sequence behavior for playback.

Generated audio player types support looping or stopping at the end of a clip sequence.

See the audio_player module documentation for usage examples.

## Variants §

§

### Loop

Repeat the full clip sequence forever.

§

### Stop

Stop after one full clip sequence pass.

## Auto Trait Implementations §

§

### impl Freeze for AtEnd

§

### impl RefUnwindSafe for AtEnd

§

### impl Send for AtEnd

§

### impl Sync for AtEnd

§

### impl Unpin for AtEnd

§

### impl UnwindSafe for AtEnd

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module audio_ player Copy item path

Path: audio_player/index.html

device_envoy_rp

# Module audio_ player Copy item path

Source

Expand description

A device abstraction for playing audio clips over I²S hardware, with runtime sequencing, volume control, and compression.

This page provides the primary documentation for generated audio player types and clip utilities.

Audio clip sample data is defined at compile time as static values. At runtime, you select which clips to play and in what order. Playback runs in the background while the application does other work. Volume can be adjusted on the fly, and playback can be stopped or interrupted mid-clip. Audio samples can be compressed and are stored in flash. Only a small DMA buffer is used at runtime.

Supported audio formats

Any sample rate supported by your hardware

Either: Uncompressed: 16-bit PCM (s16le) Compressed: IMA ADPCM in WAV (mono; ~25% the size of PCM; ideal for speech)

Uncompressed: 16-bit PCM (s16le)

Compressed: IMA ADPCM in WAV (mono; ~25% the size of PCM; ideal for speech)

Mono input audio (duplicated to left/right on I²S output)

For ffmpeg conversion commands, see “Preparing audio files” in pcm_clip! and adpcm_clip! .

After reading the examples below, see also:

audio_player! - Macro to generate an audio player struct type (includes syntax details). See AudioPlayerGenerated for sample generated methods and associated constants.

AudioPlayerGenerated - Sample generated audio player struct type, showing constructor/associated constants (for example new ).

AudioPlayer - Trait providing playback operations ( play , stop , wait_until_stopped , runtime volume controls) for generated types.

pcm_clip! - Macro to “compile in” an uncompressed (PCM) clip from an external file (includes syntax details). See PcmClipGenerated for sample generated items.

adpcm_clip! - Macro to “compile in” a compressed (ADPCM) WAV clip from an external file (includes syntax details). See AdpcmClipGenerated for sample generated items.

tone! - Macro to generate tone audio clips.

SilenceClip - An audio clip of silence for a specific duration. Memory-efficient because it stores no audio sample data.

PcmClip and PcmClipBuf - Unsized and sized const-friendly uncompressed (PCM) clip types.

AdpcmClip and AdpcmClipBuf - Unsized and sized const-friendly compressed (ADPCM) clip types.

## § Example: Play “Mary Had a Little Lamb” (Phrase) Once

This example plays the opening phrase ( E D C D E E E ) and then stops.

use
device_envoy_rp::{
Result
,
    audio_player::{AudioPlayer
as _
,AtEnd, SilenceClip, VOICE_22050_HZ, Volume, audio_player},
    tone,
};
use
core::time::Duration
as
StdDuration;
// Generate `AudioPlayer8`, a struct type with the specified configuration.
audio_player!
{
    AudioPlayer8 {
        data_pin: PIN_8,
        bit_clock_pin: PIN_9,
        word_select_pin: PIN_10,
        sample_rate_hz: VOICE_22050_HZ,
// Convenience constant for this example; any hardware-supported sample rate can be used.
max_volume: Volume::percent(
50
),
    }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
// REST is 80 ms of silence. It stores only duration
    // (no PCM/ADPCM sample data in flash).
const
REST:
&
AudioPlayer8Playable =
&
SilenceClip::new(StdDuration::from_millis(
80
));
// Define each note as a static clip of a sine wave at the appropriate frequency, 220 ms long.
const
SAMPLE_RATE_HZ: u32 = AudioPlayer8::SAMPLE_RATE_HZ;
const
NOTE_DURATION: StdDuration = StdDuration::from_millis(
220
);
const
NOTE_E4:
&
AudioPlayer8Playable =
&
tone!
(
330
, SAMPLE_RATE_HZ, NOTE_DURATION);
const
NOTE_D4:
&
AudioPlayer8Playable =
&
tone!
(
294
, SAMPLE_RATE_HZ, NOTE_DURATION);
const
NOTE_C4:
&
AudioPlayer8Playable =
&
tone!
(
262
, SAMPLE_RATE_HZ, NOTE_DURATION);
let
p = embassy_rp::init(Default::default());
// Create an `AudioPlayer8` instance with the specified pins and resources.
let
audio_player8 = AudioPlayer8::new(p.PIN_8, p.PIN_9, p.PIN_10, p.PIO0, p.DMA_CH0, spawner)
?
;

    audio_player8.play(
        [
            NOTE_E4, REST, NOTE_D4, REST, NOTE_C4, REST, NOTE_D4, REST, NOTE_E4, REST,
            NOTE_E4, REST, NOTE_E4,
        ],
        AtEnd::Stop,
    );
// Audio plays in the background while we can do other things here, like blink an LED or read a button.
pending().
await
// run forever
}

## § Example: Compiling in an External Audio Clip and Runtime Volume Changes

This example shows how to “compile in” an audio clip from an external file, compress it at compile time, and then play it in a loop while changing the volume while it plays. This also demonstrates how to stop playback and reset the volume.

use
device_envoy_rp::{
Result
,
    audio_player::{AudioPlayer
as _
,
        AtEnd, Gain, SilenceClip, Volume, pcm_clip, audio_player, VOICE_22050_HZ,
    },
    button::{Button
as _
, ButtonRp, PressedTo},
    tone,
};
use
core::time::Duration
as
StdDuration;
use
embassy_futures::select::{Either, select};
use
embassy_time::{Duration, Timer};
audio_player!
{
    AudioPlayer8 {
        data_pin: PIN_8,
        bit_clock_pin: PIN_9,
        word_select_pin: PIN_10,
        sample_rate_hz: VOICE_22050_HZ,
        pio: PIO0,
// optional, defaults to PIO0
dma: DMA_CH1,
// optional, defaults to DMA_CH0
max_clips:
8
,
// optional, defaults to 16
max_volume: Volume::spinal_tap(
11
),
// optional, defaults to Volume::MAX
initial_volume: Volume::spinal_tap(
5
),
// optional, defaults to Volume::MAX
}
}
// Define a `const` function that returns audio from this PCM file.
// If unused, it adds nothing to the firmware image.
pcm_clip!
{
    Nasa {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/nasa_22k.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
    }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
const fn
ms(milliseconds: u64) -> StdDuration {
        StdDuration::from_millis(milliseconds)
    }
const
SAMPLE_RATE_HZ: u32 = AudioPlayer8::SAMPLE_RATE_HZ;
// Only the final transformed clips are stored in flash.
    // Intermediate compile-time temporaries (such as compression and gain steps) are not stored.

    // Read the uncompressed (PCM) NASA clip in compressed (ADPCM) format.
const
NASA:
&
AudioPlayer8Playable =
&
Nasa::adpcm_clip();
// 80ms of silence
const
GAP:
&
AudioPlayer8Playable =
&
SilenceClip::new(ms(
80
));
// 100ms of a pure 880Hz tone, at 20% loudness.
const
CHIME:
&
AudioPlayer8Playable =
&
tone!
(
880
, SAMPLE_RATE_HZ, ms(
100
)).with_gain(Gain::percent(
20
));
let
p = embassy_rp::init(Default::default());
let
mut
button = ButtonRp::new(p.PIN_13, PressedTo::Ground);
let
audio_player8 =
        AudioPlayer8::new(p.PIN_8, p.PIN_9, p.PIN_10, p.PIO0, p.DMA_CH1, spawner)
?
;
const
VOLUME_STEPS_PERCENT: [u8;
7
] = [
50
,
25
,
12
,
6
,
3
,
1
,
0
];
loop
{
// Wait for user input before starting.
button.wait_for_press().
await
;
// Start playing the NASA clip, over and over.
audio_player8.play([CHIME, NASA, GAP], AtEnd::Loop);
// Lower runtime volume over time, unless the button is pressed.
for
volume_percent
in
VOLUME_STEPS_PERCENT {
match
select(
                button.wait_for_press(),
                Timer::after(Duration::from_secs(
1
)),
            )
            .
await
{
                Either::First(()) => {
// Button pressed: leave inner loop.
break
;
                }
                Either::Second(()) => {
// Timer elapsed: lower volume and keep looping.
audio_player8.set_volume(Volume::percent(volume_percent));
                }
            }
        }
        audio_player8.stop();
        audio_player8.set_volume(<AudioPlayer8
as
device_envoy_rp::audio_player::AudioPlayer<{ AudioPlayer8::SAMPLE_RATE_HZ }>>::INITIAL_VOLUME);

    }
}

## § Example: Resample and Play Countdown Once

This example “compiles in” three 22.05 kHz clips ( 2 , 1 , 0 ) and NASA. It changes them to 8 kHz at compile time ( resample means changing how many audio samples are stored per second), compresses them, and plays them once.

Only the final 8 kHz compressed clips are stored in flash.

sample_rate_hz means samples per second. The clip sample rate is part of the clip type, so using the wrong rate is a compile-time error.

use
device_envoy_rp::{
Result
,
    audio_player::{AudioPlayer
as _
,
        AtEnd, Gain, NARROWBAND_8000_HZ, VOICE_22050_HZ, Volume, pcm_clip,
        audio_player,
    },
};
// To save memory, we use a lower sample rate.
audio_player!
{
    AudioPlayer8 {
        data_pin: PIN_8,
        bit_clock_pin: PIN_9,
        word_select_pin: PIN_10,
        sample_rate_hz: NARROWBAND_8000_HZ,
        max_volume: Volume::percent(
50
),
    }
}
// We resample each clip from the original 22KHz to the 8KHz sample rate of our audio player.
pcm_clip!
{
    Digit0 {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/0_22050.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: AudioPlayer8::SAMPLE_RATE_HZ,
    }
}
pcm_clip!
{
    Digit1 {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/1_22050.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: AudioPlayer8::SAMPLE_RATE_HZ,
    }
}
pcm_clip!
{
    Digit2 {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/2_22050.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: AudioPlayer8::SAMPLE_RATE_HZ,
    }
}
pcm_clip!
{
    Nasa {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/nasa_22k.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: AudioPlayer8::SAMPLE_RATE_HZ,
    }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
// We convert, at compile-time, to compressed (ADPCM) format.
const
DIGITS: [
&
AudioPlayer8Playable;
3
] = [
&
Digit0::adpcm_clip(),
&
Digit1::adpcm_clip(),
&
Digit2::adpcm_clip(),
    ];
// We read the uncompressed (PCM) NASA clip, change its loudness, and then convert it to compressed (ADPCM) format.
const
NASA:
&
AudioPlayer8Playable =
&
Nasa::pcm_clip()
        .with_gain(Gain::percent(
25
))
        .with_adpcm::<{ Nasa::ADPCM_DATA_LEN }>();
let
p = embassy_rp::init(Default::default());
let
audio_player8 = AudioPlayer8::new(p.PIN_8, p.PIN_9, p.PIN_10, p.PIO0, p.DMA_CH0, spawner)
?
;

    audio_player8.play([DIGITS[
2
], DIGITS[
1
], DIGITS[
0
], NASA], AtEnd::Stop);
    pending().
await
// run forever
}

## Modules §

adpcm_ clip_ generated Module containing AdpcmClipGenerated , the sample module generated by the adpcm_clip! macro. audio_ player_ generated Module containing AudioPlayerGenerated , the sample struct type generated by the audio_player! macro. pcm_ clip_ generated Module containing PcmClipGenerated , the sample module generated by the pcm_clip! macro.

## Macros §

adpcm_ clip Macro to “compile in” a compressed (ADPCM) WAV clip from an external file (includes syntax details). audio_ player Macro to generate an audio player struct type (includes syntax details). pcm_ clip Macro to “compile in” an uncompressed (PCM) clip from an external file (includes syntax details). tone Macro to create an audio clip of a musical tone.

## Structs §

Adpcm Clip Unsized view of static compressed (ADPCM) clip data. Gain Relative loudness adjustment for audio clips. PcmClip Unsized view of static uncompressed (PCM) audio clip data. Silence Clip An audio clip of silence for a specific duration. Memory-efficient because it stores no audio sample data. Volume Absolute playback loudness setting for the whole player.

## Enums §

AtEnd End-of-sequence behavior for playback.

## Constants §

CD_ 44100_ HZ Compact-disc sample rate. NARROWBAND_ 8000_ HZ Narrowband telephony sample rate. PRO_ 48000_ HZ Pro-audio sample rate. VOICE_ 16000_ HZ Wideband voice sample rate. VOICE_ 22050_ HZ Common low-memory voice/music sample rate.

## Traits §

Audio Player Platform-agnostic audio player device contract. Playable A clip source trait for generated audio player play(...) methods.

## Type Aliases §

Adpcm Clip Buf Sized, const-friendly storage for compressed (ADPCM) clip data. PcmClip Buf Sized, const-friendly storage for uncompressed (PCM) audio clip data.

# macro.adpcm_clip!.html

Path: audio_player/macro.adpcm_clip!.html

Redirecting to macro.adpcm_clip.html ...

# Macro adpcm_ clip Copy item path

Path: audio_player/macro.adpcm_clip.html

device_envoy_rp

::

audio_player

# Macro adpcm_ clip Copy item path

macro_rules! adpcm_clip {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to “compile in” a compressed (ADPCM) WAV clip from an external file (includes syntax details).

markdownlint-disable MD041

See AdpcmClipGenerated for a sample of generated items.

At compile time, you can read the clip as compressed ADPCM with Name::adpcm_clip() or uncompressed PCM with Name::pcm_clip() . You can also modify the PCM data at compile time (for example with Gain via with_gain(...) ), and only the final transformed clip is stored in firmware. Additionally, you can decode to PCM first with with_pcm , process it, and still store the final clip in compressed ADPCM form with with_adpcm .

Syntax:

adpcm_clip! {
    [<visibility>] <Name> {
        file: <path_expr>,
        target_sample_rate_hz: <sample_rate_expr>, // optional, defaults to WAV sample_rate_hz
    }
}

Inputs:

$vis - Optional generated module visibility.

$name - Module name for the generated module.

Required fields:

file - Path to an ADPCM WAV file.

Optional fields:

target_sample_rate_hz - Output sample rate in hertz for generated clips (default: the WAV file sample rate).

Generated items:

Name::pcm_clip() - const function that returns the uncompressed (PCM) version of this clip.

Name::adpcm_clip() - const function that returns the compressed (ADPCM) encoding for this clip.

Name::SAMPLE_RATE_HZ - sample rate for generated clips

Name::ADPCM_DATA_LEN - byte length for compressed (ADPCM) encoding this clip

Name::PCM_SAMPLE_COUNT - number of samples for uncompressed (PCM) version of this clip

See the audio_player module documentation for usage examples.

## § Preparing audio files for adpcm_clip!

This macro expects mono IMA ADPCM WAV input.

### § 1) Download an example clip (NASA)

Download the MP3:

curl -L -o nasa.mp3 "https://www.nasa.gov/wp-content/uploads/2015/01/640149main_Computers20are20in20Control.mp3"

(Windows 10/11 includes curl by default.)

### § 2) Install ffmpeg (or confirm it is installed)

General: see the official download page:

https://ffmpeg.org/download.html

#### § Ubuntu / Debian

sudo apt update
sudo apt install ffmpeg
ffmpeg -version

#### § Windows (recommended: pixi )

Install pixi :

https://pixi.sh

Install ffmpeg globally:

pixi global install ffmpeg
ffmpeg -version

### § 3) Convert to ADPCM WAV ( .wav ) for adpcm_clip!

This produces a mono IMA ADPCM WAV that adpcm_clip! can compile in:

ffmpeg -y -i nasa.mp3 -vn -ac 1 -ar 22050 -c:a adpcm_ima_wav -block_size 256 nasa_22k_adpcm.wav

What the extra arguments mean:

-c:a adpcm_ima_wav - encode IMA ADPCM in WAV

-block_size 256 - use 256-byte ADPCM blocks (matches pcm_clip! ADPCM output blocks)

nasa_22k_adpcm.wav - output file (ready for adpcm_clip! )

Tip: omit target_sample_rate_hz in adpcm_clip! to keep the WAV sample rate, or set it to resample at compile time.

# macro.audio_player!.html

Path: audio_player/macro.audio_player!.html

Redirecting to macro.audio_player.html ...

# Macro audio_ player Copy item path

Path: audio_player/macro.audio_player.html

device_envoy_rp

::

audio_player

# Macro audio_ player Copy item path

Source

macro_rules! audio_player {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate an audio player struct type (includes syntax details).

See AudioPlayerGenerated for a sample of a generated type.

See the audio_player module documentation for usage examples.

Syntax:

audio_player! {
    [<visibility>] <Name> {
        data_pin: <pin_ident>,
        bit_clock_pin: <pin_ident>,
        word_select_pin: <pin_ident>,
        sample_rate_hz: <sample_rate_expr>,
        pio: <pio_ident>,                 // optional
        dma: <dma_ident>,                 // optional
        max_clips: <usize_expr>,          // optional
        max_volume: <Volume_expr>,        // optional
        initial_volume: <Volume_expr>,    // optional
    }
}

Field order is flexible. Optional fields may be omitted or specified in any order.

Inputs:

$vis - Optional generated type visibility (for example: pub , pub(crate) , pub(self) ). Defaults to private visibility when omitted.

$name - Generated type name (for example: AudioPlayer10 )

Required fields:

data_pin - GPIO pin carrying I²S data ( DIN )

bit_clock_pin - GPIO pin carrying I²S bit clock ( BCLK )

word_select_pin - GPIO pin carrying I²S word-select / LR clock ( LRC / LRCLK )

sample_rate_hz - Playback sample rate in hertz (for example: VOICE_22050_HZ )

Optional fields:

pio - PIO resource (default: PIO0 )

dma - DMA channel (default: DMA_CH0 )

max_clips - Maximum clips per queued play request (default: 16 )

max_volume - Runtime volume ceiling (default: Volume::MAX )

initial_volume - Initial runtime volume relative to max_volume (default: Volume::MAX )

Generated items:

<Name> - generated player struct type

<Name>Playable - trait-object clip source alias at this player’s sample rate

associated constants and constructor on <Name> (for example: SAMPLE_RATE_HZ , new(...) )

playback operations via AudioPlayer trait ( play(...) , stop() , wait_until_stopped(...) , volume controls)

The generated type contains static resources and spawns its background device task from new(...) .

# macro.pcm_clip!.html

Path: audio_player/macro.pcm_clip!.html

Redirecting to macro.pcm_clip.html ...

# Macro pcm_ clip Copy item path

Path: audio_player/macro.pcm_clip.html

device_envoy_rp

::

audio_player

# Macro pcm_ clip Copy item path

macro_rules! pcm_clip {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to “compile in” an uncompressed (PCM) clip from an external file (includes syntax details).

markdownlint-disable MD041

See PcmClipGenerated for a sample of generated items.

See the audio_player module documentation for usage examples.

At compile time, you can read the clip as uncompressed PCM with Name::pcm_clip() or compressed ADPCM with Name::adpcm_clip() . You can also modify the PCM data at compile time (for example with Gain via with_gain(...) ), and only the final transformed clip is stored in firmware. Additionally, you can process PCM first and still store the final clip in compressed ADPCM form with with_adpcm .

Syntax:

pcm_clip! {
    [<visibility>] <Name> {
        file: <file_path_expr>,
        source_sample_rate_hz: <sample_rate_expr>,
        target_sample_rate_hz: <sample_rate_expr>, // optional, defaults to source_sample_rate_hz
    }
}

Inputs:

$vis - Optional module visibility for the generated module (for example: pub , pub(crate) , pub(self) ). Defaults to private visibility when omitted.

$name - Module name for the generated module (for example: Nasa )

Required fields:

file - Path to an external audio file (for example: "nasa_22k.s16" )

source_sample_rate_hz - Source sample rate in hertz for the input file (for example: VOICE_22050_HZ )

Optional fields:

target_sample_rate_hz - Output sample rate in hertz for generated clips (default: source_sample_rate_hz )

Generated items:

Name::pcm_clip() - const function that returns the uncompressed (PCM) version of this clip.

Name::adpcm_clip() - const function that returns the compressed (ADPCM) encoding for this clip.

Name::SAMPLE_RATE_HZ - sample rate for generated clips

Name::PCM_SAMPLE_COUNT - number of samples for uncompressed (PCM) version of this clip

Name::ADPCM_DATA_LEN - byte length for compressed (ADPCM) encoding this clip

See PcmClipGenerated and the audio_player module documentation .

## § Preparing audio files for pcm_clip!

This macro expects audio in a simple raw format:

mono

16-bit signed samples

little-endian

a fixed sample rate (for example, 22050 Hz)

The easiest way to produce that format is with ffmpeg .

### § 1) Download an example clip (NASA)

Download the MP3:

curl -L -o nasa.mp3 "https://www.nasa.gov/wp-content/uploads/2015/01/640149main_Computers20are20in20Control.mp3"

(Windows 10/11 includes curl by default.)

### § 2) Install ffmpeg (or confirm it is installed)

General: see the official download page:

https://ffmpeg.org/download.html

#### § Ubuntu / Debian

sudo apt update
sudo apt install ffmpeg
ffmpeg -version

#### § Windows (recommended: pixi )

Install pixi :

https://pixi.sh

Install ffmpeg globally:

pixi global install ffmpeg
ffmpeg -version

### § 3) Convert to raw PCM ( .s16 ) for pcm_clip!

This produces a file you can embed and process at compile time:

ffmpeg -y -i nasa.mp3 -vn -ac 1 -ar 22050 -f s16le nasa_22k.s16

What the arguments mean:

-i nasa.mp3 - input file

-vn - ignore any video track (safe even for audio-only inputs)

-ac 1 - force mono

-ar 22050 - set the sample rate (Hz)

-f s16le - write raw 16-bit little-endian PCM (no WAV header)

nasa_22k.s16 - output file (ready for pcm_clip! )

Tip: pass the file’s native rate as source_sample_rate_hz: and optionally resample at compile time with target_sample_rate_hz: .

# macro.tone!.html

Path: audio_player/macro.tone!.html

Redirecting to macro.tone.html ...

# Macro tone Copy item path

Path: audio_player/macro.tone.html

device_envoy_rp

::

audio_player

# Macro tone Copy item path

macro_rules! tone {
    ($frequency_hz:expr, $sample_rate_hz:expr, $duration:expr) => { ... };
}

Expand description

Macro to create an audio clip of a musical tone.

Examples:

tone!(440, VOICE_22050_HZ, Duration::from_millis(500))

tone!(440, AudioPlayer8::SAMPLE_RATE_HZ, Duration::from_millis(500))

The result is an uncompressed (PCM) clip. (It does not use compressed because ADPCM sounds poor for pure sine tones.)

See the audio_player module documentation for usage examples.

# Constant ADPCM_ DATA_ LEN Copy item path

Path: audio_player/pcm_clip_generated/PcmClipGenerated/constant.ADPCM_DATA_LEN.html

device_envoy_rp

::

audio_player :: pcm_clip_generated :: PcmClipGenerated

# Constant ADPCM_ DATA_ LEN Copy item path

Source

pub const ADPCM_DATA_LEN:
usize
= _; // 46_848usize

Expand description

Byte length for compressed (ADPCM) encoding this clip.

See the audio_player module documentation for usage examples.

# Constant PCM_ SAMPLE_ COUNT Copy item path

Path: audio_player/pcm_clip_generated/PcmClipGenerated/constant.PCM_SAMPLE_COUNT.html

device_envoy_rp

::

audio_player :: pcm_clip_generated :: PcmClipGenerated

# Constant PCM_ SAMPLE_ COUNT Copy item path

Source

pub const PCM_SAMPLE_COUNT:
usize
= 92_160;

Expand description

Number of samples for uncompressed (PCM) version of this clip.

# Constant SAMPLE_ RATE_ HZ Copy item path

Path: audio_player/pcm_clip_generated/PcmClipGenerated/constant.SAMPLE_RATE_HZ.html

device_envoy_rp

::

audio_player :: pcm_clip_generated :: PcmClipGenerated

# Constant SAMPLE_ RATE_ HZ Copy item path

Source

pub const SAMPLE_RATE_HZ:
u32
= VOICE_22050_HZ; // 22_050u32

Expand description

Sample rate in hertz for this generated clip.

See the audio_player module documentation for usage examples.

# Function adpcm_ clip Copy item path

Path: audio_player/pcm_clip_generated/PcmClipGenerated/fn.adpcm_clip.html

device_envoy_rp

::

audio_player :: pcm_clip_generated :: PcmClipGenerated

# Function adpcm_ clip Copy item path

Source

pub const fn adpcm_clip() ->
AdpcmClipBuf
<SAMPLE_RATE_HZ, ADPCM_DATA_LEN>

Expand description

const function that returns the compressed (ADPCM) encoding for this clip.

See the audio_player module documentation for usage examples.

# Function pcm_ clip Copy item path

Path: audio_player/pcm_clip_generated/PcmClipGenerated/fn.pcm_clip.html

device_envoy_rp

::

audio_player :: pcm_clip_generated :: PcmClipGenerated

# Function pcm_ clip Copy item path

Source

pub const fn pcm_clip() ->
PcmClipBuf
<SAMPLE_RATE_HZ, PCM_SAMPLE_COUNT>

Expand description

const function that returns the uncompressed (PCM) version of this clip.

See the audio_player module documentation for usage examples.

# Module PcmClip Generated Copy item path

Path: audio_player/pcm_clip_generated/PcmClipGenerated/index.html

device_envoy_rp

::

audio_player :: pcm_clip_generated

# Module PcmClip Generated Copy item path

Source

Expand description

Sample module generated by the pcm_clip! macro, showing the generated items.

This page serves as the reference for what a generated audio clip module provides. For first-time readers, start with the audio_player module documentation, then return here for the generated pcm_clip() / adpcm_clip() items.

The generated items are in a module (not a struct type) because stable Rust does not support inherent associated types on structs.

Auto-generated.

## Constants §

ADPCM_ DATA_ LEN Byte length for compressed (ADPCM) encoding this clip. PCM_ SAMPLE_ COUNT Number of samples for uncompressed (PCM) version of this clip. SAMPLE_ RATE_ HZ Sample rate in hertz for this generated clip.

## Functions §

adpcm_ clip const function that returns the compressed (ADPCM) encoding for this clip. pcm_ clip const function that returns the uncompressed (PCM) version of this clip.

# Module pcm_ clip_ generated Copy item path

Path: audio_player/pcm_clip_generated/index.html

device_envoy_rp

::

audio_player

# Module pcm_ clip_ generated Copy item path

Source

Expand description

Module containing PcmClipGenerated , the sample module generated by the pcm_clip! macro.

Auto-generated.

## Modules §

PcmClip Generated Sample module generated by the pcm_clip! macro, showing the generated items.

# Struct Adpcm Clip Copy item path

Path: audio_player/struct.AdpcmClip.html

device_envoy_rp

::

audio_player

# Struct Adpcm Clip Copy item path

pub struct AdpcmClip<const SAMPLE_RATE_HZ:
u32
, T = [
u8
]>
where
    T: ?
Sized
,
{
/* private fields */
}

Expand description

Unsized view of static compressed (ADPCM) clip data.

For fixed-size, const-friendly storage, see AdpcmClipBuf .

## Implementations §

§

### impl<const SAMPLE_RATE_HZ: u32 , T> AdpcmClip <SAMPLE_RATE_HZ, T> where T: ? Sized ,

#### pub fn block_align (&self) -> u16

Returns the ADPCM block size in bytes.

#### pub fn samples_per_block (&self) -> u16

Returns the number of decoded samples per ADPCM block.

#### pub fn pcm_sample_count (&self) -> usize

Returns the decoded PCM sample count represented by this ADPCM clip.

#### pub fn data (&self) -> &T

Returns a reference to the raw ADPCM byte data.

§

### impl<const SAMPLE_RATE_HZ: u32 , const DATA_LEN: usize > AdpcmClip <SAMPLE_RATE_HZ, [ u8 ; DATA_LEN ]>

Implementation for fixed-size clips ( AdpcmClipBuf ).

This impl applies to AdpcmClip with array-backed storage: AdpcmClip<SAMPLE_RATE_HZ, [u8; DATA_LEN]> (which is what AdpcmClipBuf aliases).

#### pub const fn with_pcm <const SAMPLE_COUNT: usize >( &self, ) -> PcmClip <SAMPLE_RATE_HZ, [ i16 ; SAMPLE_COUNT ]>

Returns the uncompressed (PCM) version of this clip.

SAMPLE_COUNT is the number of samples in the resulting PCM clip. Typically, use the generated clip-module constant: AdpcmClipGenerated::PCM_SAMPLE_COUNT .

#### pub const fn with_gain ( self, gain: Gain , ) -> AdpcmClip <SAMPLE_RATE_HZ, [ u8 ; DATA_LEN ]>

Returns this fixed-size ADPCM clip with linear sample gain applied.

This operation decodes ADPCM to PCM, applies gain, then re-encodes ADPCM. The extra ADPCM encode pass can be more lossy than applying gain once on PCM before a single ADPCM encode.

## Auto Trait Implementations §

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Freeze for AdpcmClip <SAMPLE_RATE_HZ, T> where T: Freeze + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> RefUnwindSafe for AdpcmClip <SAMPLE_RATE_HZ, T> where T: RefUnwindSafe + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Send for AdpcmClip <SAMPLE_RATE_HZ, T> where T: Send + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Sync for AdpcmClip <SAMPLE_RATE_HZ, T> where T: Sync + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Unpin for AdpcmClip <SAMPLE_RATE_HZ, T> where T: Unpin + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> UnwindSafe for AdpcmClip <SAMPLE_RATE_HZ, T> where T: UnwindSafe + ? Sized ,

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Playable <SAMPLE_RATE_HZ> for T where T: PlayableSealed<SAMPLE_RATE_HZ> + ? Sized ,

# Struct Gain Copy item path

Path: audio_player/struct.Gain.html

device_envoy_rp

::

audio_player

# Struct Gain Copy item path

pub struct Gain(
/* private fields */
);

Expand description

Relative loudness adjustment for audio clips.

Use Gain with PcmClipBuf::with_gain to make a clip louder or quieter before playback.

with_gain is intended for const clip definitions, so the adjusted samples are precomputed at compile time with no extra runtime work.

You can set gain by percent or by dB:

Gain::percent where 100 means unchanged and values above 100 are louder.

Gain::db where positive dB is louder and negative dB is quieter.

This is different from Volume used by max_volume , initial_volume , and set_volume , which set the absolute playback loudness behavior for the whole player.

See the audio_player module documentation for usage examples.

## Implementations §

§

### impl Gain

#### pub const MUTE : Gain

Silence.

#### pub const fn percent (percent: u16 ) -> Gain

Creates a gain from percentage.

100 is unity gain. Values above 100 boost the signal.

See the audio_player module documentation for usage examples.

#### pub const fn db (db: i8 ) -> Gain

Creates gain from dB with a bounded boost range.

Values above +12 dB clamp to +12 dB . Values below -96 dB clamp to -96 dB .

See PcmClipBuf::with_gain for usage.

## Trait Implementations §

§

### impl Clone for Gain

§

#### fn clone (&self) -> Gain

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Gain

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl PartialEq for Gain

§

#### fn eq (&self, other: & Gain ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for Gain

§

### impl Eq for Gain

§

### impl StructuralPartialEq for Gain

## Auto Trait Implementations §

§

### impl Freeze for Gain

§

### impl RefUnwindSafe for Gain

§

### impl Send for Gain

§

### impl Sync for Gain

§

### impl Unpin for Gain

§

### impl UnwindSafe for Gain

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct PcmClip Copy item path

Path: audio_player/struct.PcmClip.html

device_envoy_rp

::

audio_player

# Struct PcmClip Copy item path

pub struct PcmClip<const SAMPLE_RATE_HZ:
u32
, T = [
i16
]>
where
    T: ?
Sized
,
{
/* private fields */
}

Expand description

Unsized view of static uncompressed (PCM) audio clip data.

For fixed-size, const-friendly storage, see PcmClipBuf .

See the audio_player module documentation for usage examples.

## Implementations §

§

### impl<const SAMPLE_RATE_HZ: u32 , T> PcmClip <SAMPLE_RATE_HZ, T> where T: ? Sized ,

#### pub fn samples (&self) -> &T

Returns a reference to the raw PCM sample data.

§

### impl<const SAMPLE_RATE_HZ: u32 , const SAMPLE_COUNT: usize > PcmClip <SAMPLE_RATE_HZ, [ i16 ; SAMPLE_COUNT ]>

Implementation for fixed-size clips ( PcmClipBuf ).

This impl applies to PcmClip with array-backed storage: PcmClip<SAMPLE_RATE_HZ, [i16; SAMPLE_COUNT]> (which is what PcmClipBuf aliases).

#### pub const fn with_gain ( self, gain: Gain , ) -> PcmClip <SAMPLE_RATE_HZ, [ i16 ; SAMPLE_COUNT ]>

Returns a new clip with linear sample gain applied.

This is intended to be used in const clip definitions so the adjusted samples are computed ahead of time.

Gain multiplication uses i32 math and saturates to i16 sample bounds. Large boosts can hard-clip peaks and introduce distortion.

See the audio_player module documentation for usage examples.

#### pub const fn with_adpcm <const DATA_LEN: usize >( &self, ) -> AdpcmClip <SAMPLE_RATE_HZ, [ u8 ; DATA_LEN ]>

Returns the compressed (ADPCM) encoding for this clip.

See the audio_player module documentation for usage examples.

## Auto Trait Implementations §

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Freeze for PcmClip <SAMPLE_RATE_HZ, T> where T: Freeze + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> RefUnwindSafe for PcmClip <SAMPLE_RATE_HZ, T> where T: RefUnwindSafe + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Send for PcmClip <SAMPLE_RATE_HZ, T> where T: Send + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Sync for PcmClip <SAMPLE_RATE_HZ, T> where T: Sync + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Unpin for PcmClip <SAMPLE_RATE_HZ, T> where T: Unpin + ? Sized ,

§

### impl<const SAMPLE_RATE_HZ: u32 , T> UnwindSafe for PcmClip <SAMPLE_RATE_HZ, T> where T: UnwindSafe + ? Sized ,

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Playable <SAMPLE_RATE_HZ> for T where T: PlayableSealed<SAMPLE_RATE_HZ> + ? Sized ,

# Struct Silence Clip Copy item path

Path: audio_player/struct.SilenceClip.html

device_envoy_rp

::

audio_player

# Struct Silence Clip Copy item path

pub struct SilenceClip {
/* private fields */
}

Expand description

An audio clip of silence for a specific duration. Memory-efficient because it stores no audio sample data.

This clip type is sample-rate agnostic. It can be used with any generated player sample rate because silence is rendered at playback time.

See the audio_player module documentation for usage examples.

## Implementations §

§

### impl SilenceClip

#### pub const fn new (duration: Duration ) -> SilenceClip

Creates a silence clip for a specific duration. This uses core::time::Duration .

#### pub const fn duration (self) -> Duration

Returns the silence duration. This returns core::time::Duration .

## Trait Implementations §

§

### impl Clone for SilenceClip

§

#### fn clone (&self) -> SilenceClip

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for SilenceClip

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl PartialEq for SilenceClip

§

#### fn eq (&self, other: & SilenceClip ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for SilenceClip

§

### impl Eq for SilenceClip

§

### impl StructuralPartialEq for SilenceClip

## Auto Trait Implementations §

§

### impl Freeze for SilenceClip

§

### impl RefUnwindSafe for SilenceClip

§

### impl Send for SilenceClip

§

### impl Sync for SilenceClip

§

### impl Unpin for SilenceClip

§

### impl UnwindSafe for SilenceClip

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Playable <SAMPLE_RATE_HZ> for T where T: PlayableSealed<SAMPLE_RATE_HZ> + ? Sized ,

# Struct Volume Copy item path

Path: audio_player/struct.Volume.html

device_envoy_rp

::

audio_player

# Struct Volume Copy item path

pub struct Volume(
/* private fields */
);

Expand description

Absolute playback loudness setting for the whole player.

Volume is used by the player-level controls max_volume , initial_volume , and set_volume , which set the absolute playback loudness behavior for the whole player.

This is different from Gain and PcmClipBuf::with_gain , which adjust the relative loudness of individual clips.

See the audio_player module documentation for usage examples.

## Implementations §

§

### impl Volume

#### pub const MUTE : Volume

Silence.

#### pub const MAX : Volume

Maximum playback volume.

#### pub const fn percent (percent: u8 ) -> Volume

Creates a volume from a percentage of full scale.

Values above 100 are clamped to 100 .

See the audio_player module documentation for usage examples.

#### pub const fn spinal_tap (spinal_tap: u8 ) -> Volume

Creates a humorous “goes to 11” demo volume scale.

0..=11 maps to 0..=100% using a perceptual curve (roughly logarithmic, but not mathematically exact).

Values above 11 clamp to 11 .

See the audio_player module documentation for usage examples.

## Trait Implementations §

§

### impl Clone for Volume

§

#### fn clone (&self) -> Volume

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Volume

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl PartialEq for Volume

§

#### fn eq (&self, other: & Volume ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for Volume

§

### impl Eq for Volume

§

### impl StructuralPartialEq for Volume

## Auto Trait Implementations §

§

### impl Freeze for Volume

§

### impl RefUnwindSafe for Volume

§

### impl Send for Volume

§

### impl Sync for Volume

§

### impl Unpin for Volume

§

### impl UnwindSafe for Volume

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Audio Player Copy item path

Path: audio_player/trait.AudioPlayer.html

device_envoy_rp

::

audio_player

# Trait Audio Player Copy item path

pub trait AudioPlayer<const SAMPLE_RATE_HZ:
u32
> {
    const
SAMPLE_RATE_HZ
:
u32
;
    const
MAX_CLIPS
:
usize
;
    const
INITIAL_VOLUME
:
Volume
;
    const
MAX_VOLUME
:
Volume
;

    // Required methods
    fn
play
<I>(&self, audio_clips: I, at_end:
AtEnd
)
where I:
IntoIterator
<Item = &'static dyn
Playable
<SAMPLE_RATE_HZ>>
;
fn
stop
(&self);
async fn
wait_until_stopped
(&self);
fn
set_volume
(&self, volume:
Volume
);
fn
volume
(&self) ->
Volume
;
}

Expand description

Platform-agnostic audio player device contract.

Platform crates implement this trait for generated audio player types so playback operations resolve through trait methods instead of inherent methods.

## § Example: Play “Mary Had a Little Lamb” (Phrase) Once

This example plays the opening phrase ( E D C D E E E ) and then stops.

use
device_envoy_core::audio_player::{
    AtEnd, AudioPlayer, Playable, SilenceClip, VOICE_22050_HZ, Volume, tone,
};
use
core::time::Duration
as
StdDuration;
const
SAMPLE_RATE_HZ: u32 = VOICE_22050_HZ;
fn
play_mary_phrase(audio_player:
&
impl
AudioPlayer<SAMPLE_RATE_HZ>) {
type
PlayableRef =
&
'static
dyn
Playable<SAMPLE_RATE_HZ>;
const
REST: PlayableRef =
&
SilenceClip::new(StdDuration::from_millis(
80
));
const
NOTE_DURATION: StdDuration = StdDuration::from_millis(
220
);
const
NOTE_E4: PlayableRef =
&
tone!
(
330
, SAMPLE_RATE_HZ, NOTE_DURATION);
const
NOTE_D4: PlayableRef =
&
tone!
(
294
, SAMPLE_RATE_HZ, NOTE_DURATION);
const
NOTE_C4: PlayableRef =
&
tone!
(
262
, SAMPLE_RATE_HZ, NOTE_DURATION);

    audio_player.play(
        [
            NOTE_E4, REST, NOTE_D4, REST, NOTE_C4, REST, NOTE_D4, REST, NOTE_E4, REST,
            NOTE_E4, REST, NOTE_E4,
        ],
        AtEnd::Stop,
    );
}

## § Example: Compiling in an External Audio Clip and Runtime Volume Changes

This example shows how to compile in an external clip, play it in a loop, change volume at runtime, and then stop/reset playback settings.

use
device_envoy_core::audio_player::{
    AtEnd, AudioPlayer, Gain, Playable, SilenceClip, VOICE_22050_HZ, Volume, pcm_clip, tone,
};
use
device_envoy_core::button::Button;
use
core::time::Duration
as
StdDuration;
use
embassy_futures::select::{Either, select};
use
embassy_time::{Duration, Timer};
pcm_clip!
{
    Nasa {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/nasa_22k.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
    }
}
async fn
play_nasa_with_runtime_volume(
    audio_player:
&
impl
AudioPlayer<VOICE_22050_HZ>,
    button:
&mut
impl
Button,
) -> ! {
type
PlayableRef =
&
'static
dyn
Playable<VOICE_22050_HZ>;
const fn
ms(milliseconds: u64) -> StdDuration {
        StdDuration::from_millis(milliseconds)
    }
const
NASA: PlayableRef =
&
Nasa::adpcm_clip();
const
GAP: PlayableRef =
&
SilenceClip::new(ms(
80
));
const
CHIME: PlayableRef =
&
tone!
(
880
, VOICE_22050_HZ, ms(
100
)).with_gain(Gain::percent(
20
));
const
VOLUME_STEPS_PERCENT: [u8;
7
] = [
50
,
25
,
12
,
6
,
3
,
1
,
0
];
let
initial_volume = audio_player.volume();
loop
{
        audio_player.play([CHIME, NASA, GAP], AtEnd::Loop);
for
volume_percent
in
VOLUME_STEPS_PERCENT {
match
select(button.wait_for_press(), Timer::after(Duration::from_secs(
1
))).
await
{
                Either::First(()) =>
break
,
                Either::Second(()) => audio_player.set_volume(Volume::percent(volume_percent)),
            }
        }

        audio_player.stop();
        audio_player.set_volume(initial_volume);
        button.wait_for_press().
await
;
    }
}
fn
main() {
let
mut
button = ButtonMock;
let
audio_player = DemoAudioPlayer;
let
_future = play_nasa_with_runtime_volume(
&
audio_player,
&mut
button);
}

## § Example: Resample and Play Countdown Once

This example compiles in 2 , 1 , 0 , and NASA at 22.05 kHz , resamples them to narrowband ( 8 kHz ) at compile time, and then plays the sequence.

use
device_envoy_core::audio_player::{
    AtEnd, AudioPlayer, Gain, NARROWBAND_8000_HZ, Playable, VOICE_22050_HZ, Volume, pcm_clip,
};
use
device_envoy_core::button::Button;
pcm_clip!
{
    Digit0 {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/0_22050.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: NARROWBAND_8000_HZ,
    }
}
pcm_clip!
{
    Digit1 {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/1_22050.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: NARROWBAND_8000_HZ,
    }
}
pcm_clip!
{
    Digit2 {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/2_22050.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: NARROWBAND_8000_HZ,
    }
}
pcm_clip!
{
    Nasa {
        file:
concat!
(
env!
(
"CARGO_MANIFEST_DIR"
),
"/examples/data/audio/nasa_22k.s16"
),
        source_sample_rate_hz: VOICE_22050_HZ,
        target_sample_rate_hz: NARROWBAND_8000_HZ,
    }
}
fn
play_resampled_countdown(audio_player:
&
impl
AudioPlayer<NARROWBAND_8000_HZ>) {
type
PlayableRef =
&
'static
dyn
Playable<NARROWBAND_8000_HZ>;
const
DIGITS: [PlayableRef;
3
] =
        [
&
Digit0::adpcm_clip(),
&
Digit1::adpcm_clip(),
&
Digit2::adpcm_clip()];
const
NASA: PlayableRef =
&
Nasa::pcm_clip()
        .with_gain(Gain::percent(
25
))
        .with_adpcm::<{ Nasa::ADPCM_DATA_LEN }>();

    audio_player.play([DIGITS[
2
], DIGITS[
1
], DIGITS[
0
], NASA], AtEnd::Stop);
}
fn
main() {
let
mut
button = ButtonMock;
let
audio_player = DemoAudioPlayer;
    play_resampled_countdown(
&
audio_player);
let
_future = button.wait_for_press();
}

## Required Associated Constants §

#### const SAMPLE_RATE_HZ : u32

Sample rate in hertz for this generated player type.

#### const MAX_CLIPS : usize

Maximum number of clips accepted by play(...) for this generated type.

#### const INITIAL_VOLUME : Volume

Initial runtime volume relative to Self::MAX_VOLUME .

#### const MAX_VOLUME : Volume

Runtime volume ceiling for this generated player type.

## Required Methods §

#### fn play <I>(&self, audio_clips: I, at_end: AtEnd ) where I: IntoIterator <Item = &'static dyn Playable <SAMPLE_RATE_HZ>>,

Starts playback of one or more static audio clips.

Accepts any array-like or iterator input. The maximum number of clips is determined by the generated type configuration.

See the AudioPlayer trait documentation for usage examples.

#### fn stop (&self)

Stops current playback as soon as possible.

See the AudioPlayer trait documentation for usage examples.

#### async fn wait_until_stopped (&self)

Waits until playback is stopped.

See the AudioPlayer trait documentation for usage examples.

#### fn set_volume (&self, volume: Volume )

Sets runtime playback volume relative to the generated player’s max volume.

See the AudioPlayer trait documentation for usage examples.

#### fn volume (&self) -> Volume

Returns the current runtime playback volume relative to max volume.

See the AudioPlayer trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl AudioPlayer <VOICE_22050_HZ> for AudioPlayerGenerated

Source

§

#### const SAMPLE_RATE_HZ : u32 = VOICE_22050_HZ

Source

§

#### const MAX_CLIPS : usize = 16

Source

§

#### const INITIAL_VOLUME : Volume = Volume::MAX

Source

§

#### const MAX_VOLUME : Volume = Volume::MAX

# Trait Playable Copy item path

Path: audio_player/trait.Playable.html

device_envoy_rp

::

audio_player

# Trait Playable Copy item path

pub trait Playable<const SAMPLE_RATE_HZ:
u32
>: PlayableSealed<SAMPLE_RATE_HZ> { }

Expand description

A clip source trait for generated audio player play(...) methods.

This trait let’s us pass audio clips of different types (PCM, ADPCM, or silence) in a single heterogeneous sequence to play .

This trait is object-safe, so mixed clips are passed as: &'static dyn Playable<SAMPLE_RATE_HZ> .

## Implementors §

§

### impl<const SAMPLE_RATE_HZ: u32 , T> Playable <SAMPLE_RATE_HZ> for T where T: PlayableSealed<SAMPLE_RATE_HZ> + ? Sized ,

# Type Alias Adpcm Clip Buf Copy item path

Path: audio_player/type.AdpcmClipBuf.html

device_envoy_rp

::

audio_player

# Type Alias Adpcm Clip Buf Copy item path

pub type AdpcmClipBuf<const SAMPLE_RATE_HZ:
u32
, const DATA_LEN:
usize
> =
AdpcmClip
<SAMPLE_RATE_HZ, [
u8
;
DATA_LEN
]>;

Expand description

Sized, const-friendly storage for compressed (ADPCM) clip data.

## Aliased Type §

pub struct AdpcmClipBuf<const SAMPLE_RATE_HZ:
u32
, const DATA_LEN:
usize
> {
/* private fields */
}

# Type Alias PcmClip Buf Copy item path

Path: audio_player/type.PcmClipBuf.html

device_envoy_rp

::

audio_player

# Type Alias PcmClip Buf Copy item path

pub type PcmClipBuf<const SAMPLE_RATE_HZ:
u32
, const SAMPLE_COUNT:
usize
> =
PcmClip
<SAMPLE_RATE_HZ, [
i16
;
SAMPLE_COUNT
]>;

Expand description

Sized, const-friendly storage for uncompressed (PCM) audio clip data.

For unsized clip references, see PcmClip .

Sample rate is part of the type, so clips with different sample rates are not assignment-compatible.

See the audio_player module documentation for usage examples.

## Aliased Type §

pub struct PcmClipBuf<const SAMPLE_RATE_HZ:
u32
, const SAMPLE_COUNT:
usize
> {
/* private fields */
}

# Module button_ watch_ generated Copy item path

Path: button/button_watch_generated/index.html

device_envoy_rp

::

button

# Module button_ watch_ generated Copy item path

Source

Expand description

Module containing ButtonWatchGenerated , the sample struct type generated by the button_watch! macro.

This file shows what the macro expansion looks like for documentation purposes.

## Structs §

Button Watch Generated Sample struct type generated by the button_watch! macro, showing all methods.

# Struct Button Watch Generated Copy item path

Path: button/button_watch_generated/struct.ButtonWatchGenerated.html

device_envoy_rp

::

button :: button_watch_generated

# Struct Button Watch Generated Copy item path

Source

pub struct ButtonWatchGenerated {
/* private fields */
}

Expand description

Sample struct type generated by the button_watch! macro, showing all methods.

This page serves as the definitive reference for what a generated button watch type provides. For first-time readers, start with the examples in the button_watch! macro documentation, then return here for a complete list of available methods.

## Implementations §

Source

§

### impl ButtonWatchGenerated

Source

#### pub fn new ( pin: impl Into <Peri<'static, PIN_13>>, pressed_to: PressedTo , spawner: Spawner, ) -> Result <&'static mut Self>

Creates a new button monitor and spawns its background task.

#### § Parameters

pin : GPIO pin for the button

pressed_to : How the button is wired ( PressedTo::Ground or PressedTo::Voltage )

spawner : Task spawner for background operations

#### § Errors

Returns an error if the background task cannot be spawned.

#### § Example

See the button_watch! macro for usage.

## Trait Implementations §

Source

§

### impl Button for ButtonWatchGenerated

Source

§

#### async fn wait_for_press_duration (&mut self) -> PressDuration

Waits for the next press and returns whether it was short or long (debounced).

Read more

§

#### fn is_pressed (&self) -> bool

Returns whether the button is currently pressed.

§

#### async fn wait_for_press (&mut self)

Waits for the next press (button goes down, debounced). Does not wait for release.

Read more

Source

§

### impl Deref for ButtonWatchGenerated

Source

§

#### type Target = ButtonWatchRp

The resulting type after dereferencing.

Source

§

#### fn deref (&self) -> &Self:: Target

Dereferences the value.

## Auto Trait Implementations §

§

### impl Freeze for ButtonWatchGenerated

§

### impl ! RefUnwindSafe for ButtonWatchGenerated

§

### impl Send for ButtonWatchGenerated

§

### impl Sync for ButtonWatchGenerated

§

### impl Unpin for ButtonWatchGenerated

§

### impl ! UnwindSafe for ButtonWatchGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<P, T> Receiver for P where P: Deref <Target = T> + ? Sized , T: ? Sized ,

Source

§

#### type Target = T

🔬 This is a nightly-only experimental API. ( arbitrary_self_types )

The target type on which the method may be called.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Enum Press Duration Copy item path

Path: button/enum.PressDuration.html

device_envoy_rp

::

button

# Enum Press Duration Copy item path

pub enum PressDuration {
    Short,
    Long,
}

Expand description

Duration of a button press (short or long).

## Variants §

§

### Short

Button was held for less than [ LONG_PRESS_DURATION ] (500ms).

§

### Long

Button was held for at least [ LONG_PRESS_DURATION ] (500ms).

## Trait Implementations §

§

### impl Clone for PressDuration

§

#### fn clone (&self) -> PressDuration

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for PressDuration

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Format for PressDuration

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl Hash for PressDuration

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Ord for PressDuration

§

#### fn cmp (&self, other: & PressDuration ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for PressDuration

§

#### fn eq (&self, other: & PressDuration ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for PressDuration

§

#### fn partial_cmp (&self, other: & PressDuration ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Copy for PressDuration

§

### impl Eq for PressDuration

§

### impl StructuralPartialEq for PressDuration

## Auto Trait Implementations §

§

### impl Freeze for PressDuration

§

### impl RefUnwindSafe for PressDuration

§

### impl Send for PressDuration

§

### impl Sync for PressDuration

§

### impl Unpin for PressDuration

§

### impl UnwindSafe for PressDuration

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Enum Pressed To Copy item path

Path: button/enum.PressedTo.html

device_envoy_rp

::

button

# Enum Pressed To Copy item path

pub enum PressedTo {
    Voltage,
    Ground,
}

Expand description

Describes if the button connects to voltage or ground when pressed.

## Variants §

§

### Voltage

Button connects pin to voltage (3.3V) when pressed. Uses internal pull-down resistor. Pin reads HIGH when pressed.

§

### Ground

Button connects pin to ground (GND) when pressed. Uses internal pull-up resistor. Pin reads LOW when pressed.

## Implementations §

§

### impl PressedTo

#### pub const fn pressed_is_high (self) -> bool

Returns true when a high input level means “pressed”.

#### pub const fn is_pressed (self, level_is_high: bool ) -> bool

Evaluates whether the button is pressed for a sampled logic level.

## Trait Implementations §

§

### impl Clone for PressedTo

§

#### fn clone (&self) -> PressedTo

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for PressedTo

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Format for PressedTo

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl Hash for PressedTo

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Ord for PressedTo

§

#### fn cmp (&self, other: & PressedTo ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for PressedTo

§

#### fn eq (&self, other: & PressedTo ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for PressedTo

§

#### fn partial_cmp (&self, other: & PressedTo ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Copy for PressedTo

§

### impl Eq for PressedTo

§

### impl StructuralPartialEq for PressedTo

## Auto Trait Implementations §

§

### impl Freeze for PressedTo

§

### impl RefUnwindSafe for PressedTo

§

### impl Send for PressedTo

§

### impl Sync for PressedTo

§

### impl Unpin for PressedTo

§

### impl UnwindSafe for PressedTo

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module button Copy item path

Path: button/index.html

device_envoy_rp

# Module button Copy item path

Source

Expand description

A device abstraction for buttons with debouncing and press duration detection.

This module provides two ways to monitor button presses:

ButtonRp — Simple button monitoring. Each call to wait_for_press() , etc. starts fresh button monitoring.

button_watch! — Monitors a button in a background task so that it works even in a fast loop/select.

## Modules §

button_ watch_ generated Module containing ButtonWatchGenerated , the sample struct type generated by the button_watch! macro.

## Macros §

button_ watch Creates a button monitoring device abstraction with a background task.

## Structs §

Button Rp A device abstraction for a button with debouncing and press duration detection.

## Enums §

Press Duration Duration of a button press (short or long). Pressed To Describes if the button connects to voltage or ground when pressed.

## Traits §

Button Platform-agnostic button contract.

# macro.button_watch!.html

Path: button/macro.button_watch!.html

Redirecting to macro.button_watch.html ...

# Macro button_ watch Copy item path

Path: button/macro.button_watch.html

device_envoy_rp

::

button

# Macro button_ watch Copy item path

Source

macro_rules! button_watch {
    ($($tt:tt)*) => { ... };
}

Expand description

Creates a button monitoring device abstraction with a background task.

This macro creates a button monitor that runs in a dedicated background task, providing continuous monitoring without interruption.

See ButtonWatchGenerated for a sample of what the macro generates.

Syntax:

button_watch! {
    [<attributes>]
    [<visibility>] <Name> {
        pin: <pin_ident>,
    }
}

## § Constructors

new() — Create from a pin

## § Use Cases

Use button_watch! instead of ButtonRp when you need continuous monitoring that works even in fast loops or select() operations. ButtonRp starts fresh monitoring on each call to wait_for_press() , which can miss events in busy loops.

## § Parameters

name : The struct name for the button watch device

pin : The GPIO pin connected to the button

Optional:

vis : Visibility modifier (default: private)

## § Example

use
device_envoy_rp::button_watch;
use
device_envoy_rp::button::PressDuration;
use
device_envoy_rp::button::PressedTo;
use
device_envoy_rp::button::Button
as _
;
use
embassy_executor::Spawner;
button_watch!
{
    ButtonWatch13 {
        pin: PIN_13,
    }
}
async fn
example(
    p: embassy_rp::Peripherals,
    spawner: Spawner,
) -> device_envoy_rp::Result<()> {
// Create the button monitor (spawns background task automatically)
let
mut
button_watch13 = ButtonWatch13::new(p.PIN_13, PressedTo::Ground, spawner)
        .
await
?
;
loop
{
// Wait for button press - never misses events even if this loop is slow
match
button_watch13.wait_for_press_duration().
await
{
            PressDuration::Short => {
// Handle short press
}
            PressDuration::Long => {
// Handle long press
}
        }
    }
Ok
(())
}

# Struct Button Rp Copy item path

Path: button/struct.ButtonRp.html

device_envoy_rp

::

button

# Struct Button Rp Copy item path

Source

pub struct ButtonRp<'a> {
/* private fields */
}

Expand description

A device abstraction for a button with debouncing and press duration detection.

## § Hardware Requirements

The button can be wired in two ways:

PressedTo::Voltage : Button connects pin to 3.3V when pressed (uses pull-down)

PressedTo::Ground : Button connects pin to GND when pressed (uses pull-up)

Important : Pico 2 (RP2350) has a known silicon bug (erratum E9) with pull-down resistors that can leave the pin reading HIGH after release. Wire buttons to GND and use PressedTo::Ground on Pico 2.

## § Usage

Use Button::wait_for_press when you only need a debounced press event. It returns on the down edge and does not wait for release.

Use Button::wait_for_press_duration when you need to distinguish short vs. long presses. It returns as soon as it can decide, so long presses are reported before the button is released.

## § Example

use
device_envoy_rp::button::{Button
as _
, ButtonRp, PressDuration, PressedTo};
async fn
example(p: embassy_rp::Peripherals) {
let
mut
button = ButtonRp::new(p.PIN_13, PressedTo::Ground);
// Wait for a press without measuring duration.
button.wait_for_press().
await
;
// Measure press durations in a loop
loop
{
match
button.wait_for_press_duration().
await
{
            PressDuration::Short => {
// Handle short press
}
            PressDuration::Long => {
// Handle long press (fires before button is released)
}
        }
    }
}

## Implementations §

Source

§

### impl<'a> ButtonRp <'a>

Source

#### pub fn new <P: Pin>(pin: Peri<'a, P>, pressed_to: PressedTo ) -> Self

Creates a new ButtonRp instance from a pin.

The pin is configured based on the connection type:

PressedTo::Voltage : Uses internal pull-down (button to 3.3V)

PressedTo::Ground : Uses internal pull-up (button to GND)

## Trait Implementations §

Source

§

### impl Button for ButtonRp <'_>

§

#### fn is_pressed (&self) -> bool

Returns whether the button is currently pressed.

§

#### async fn wait_for_press (&mut self)

Waits for the next press (button goes down, debounced). Does not wait for release.

Read more

§

#### async fn wait_for_press_duration (&mut self) -> PressDuration

Waits for the next press and returns whether it was short or long (debounced).

Read more

## Auto Trait Implementations §

§

### impl<'a> Freeze for ButtonRp <'a>

§

### impl<'a> RefUnwindSafe for ButtonRp <'a>

§

### impl<'a> Send for ButtonRp <'a>

§

### impl<'a> Sync for ButtonRp <'a>

§

### impl<'a> Unpin for ButtonRp <'a>

§

### impl<'a> ! UnwindSafe for ButtonRp <'a>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Button Copy item path

Path: button/trait.Button.html

device_envoy_rp

::

button

# Trait Button Copy item path

pub trait Button: __ButtonMonitor {
    // Provided methods
    fn
is_pressed
(&self) ->
bool
{ ... }
async fn
wait_for_press
(&mut self) { ... }
async fn
wait_for_press_duration
(&mut self) ->
PressDuration
{ ... }
}

Expand description

Platform-agnostic button contract.

Platform crates inherit the default debouncing and press-duration behavior from shared core logic by implementing [ __ButtonMonitor ].

## § Hardware Requirements

The button can be wired in two ways:

PressedTo::Voltage : Button connects pin to voltage when pressed (active-high)

PressedTo::Ground : Button connects pin to ground when pressed (active-low)

## § Usage

Use Button::wait_for_press when you only need a debounced press event. It returns on the down edge and does not wait for release.

Use Button::wait_for_press_duration when you need to distinguish short vs. long presses. It returns as soon as it can decide, so long presses are reported before the button is released.

## § Example

use
device_envoy_core::button::{Button, PressDuration};
async fn
log_button_presses(button:
&mut
impl
Button) -> ! {
// Wait for a press without measuring duration.
button.wait_for_press().
await
;
// Measure press durations in a loop.
loop
{
match
button.wait_for_press_duration().
await
{
            PressDuration::Short => {
// Handle short press.
}
            PressDuration::Long => {
// Handle long press (fires before button is released).
}
        }
    }
}

## Provided Methods §

#### fn is_pressed (&self) -> bool

Returns whether the button is currently pressed.

#### async fn wait_for_press (&mut self)

Waits for the next press (button goes down, debounced). Does not wait for release.

See the Button trait documentation for usage examples.

#### async fn wait_for_press_duration (&mut self) -> PressDuration

Waits for the next press and returns whether it was short or long (debounced).

Returns as soon as it can decide, so long presses are reported before release.

See the Button trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Button for ButtonWatchGenerated

Source

§

### impl Button for ButtonRp <'_>

# Constant ONE_DAY Copy item path

Path: clock_sync/constant.ONE_DAY.html

device_envoy_rp

::

clock_sync

# Constant ONE_DAY Copy item path

pub const ONE_DAY: Duration;

Expand description

Duration representing one day (24 hours).

# Constant ONE_ MINUTE Copy item path

Path: clock_sync/constant.ONE_MINUTE.html

device_envoy_rp

::

clock_sync

# Constant ONE_ MINUTE Copy item path

pub const ONE_MINUTE: Duration;

Expand description

Duration representing one minute (60 seconds).

# Constant ONE_ SECOND Copy item path

Path: clock_sync/constant.ONE_SECOND.html

device_envoy_rp

::

clock_sync

# Constant ONE_ SECOND Copy item path

pub const ONE_SECOND: Duration;

Expand description

Duration representing one second.

# Enum Core Error Copy item path

Path: clock_sync/enum.CoreError.html

device_envoy_rp

::

clock_sync

# Enum Core Error Copy item path

#[non_exhaustive]
pub enum CoreError {
    TaskSpawn(SpawnError),
}

Expand description

Unified error type for device-envoy-core .

## Variants (Non-exhaustive) §

This enum is marked as non-exhaustive

Non-exhaustive enums could have additional variants added in future. Therefore, when matching against variants of non-exhaustive enums, an extra wildcard arm must be added to account for any future variants.

§

### TaskSpawn(SpawnError)

Spawning an Embassy task failed.

## Trait Implementations §

§

### impl Debug for Error

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

Source

§

### impl From < Error > for Error

Source

§

#### fn from (error: Error ) -> Self

Converts to this type from the input type.

§

### impl From <SpawnError> for Error

§

#### fn from (err: SpawnError) -> Error

Converts to this type from the input type.

## Auto Trait Implementations §

§

### impl Freeze for Error

§

### impl RefUnwindSafe for Error

§

### impl Send for Error

§

### impl Sync for Error

§

### impl Unpin for Error

§

### impl UnwindSafe for Error

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Function h12_m_s Copy item path

Path: clock_sync/fn.h12_m_s.html

device_envoy_rp

::

clock_sync

# Function h12_m_s Copy item path

pub fn h12_m_s(dt: &OffsetDateTime) -> (
u8
,
u8
,
u8
)

Expand description

Extract hour (12-hour format), minute, and second from an OffsetDateTime .

# Module clock_ sync Copy item path

Path: clock_sync/index.html

device_envoy_rp

# Module clock_ sync Copy item path

Source

Expand description

A device abstraction that combines NTP time synchronization with a local clock.

See ClockSyncRp for constructors and ClockSync for clock operations.

Constructor methods on ClockSyncRp come from device-envoy-core and return CoreResult with CoreError .

You can create up to two concurrent ClockSyncRp instances per program; a third is expected to fail at runtime because the clock_sync task pool uses pool_size = 2 .

## § Example: WiFi + ClockSync logging

use
device_envoy_rp::{
    Error,
Result
,
    button::PressedTo,
    button_watch,
    clock_sync::{ClockSync
as _
, ClockSyncRp, ClockSyncStaticRp, ONE_SECOND, h12_m_s},
    flash_block::FlashBlockRp,
    wifi_auto::fields::{TimezoneField, TimezoneFieldStatic},
    wifi_auto::{WifiAuto
as _
, WifiAutoEvent, WifiAutoRp},
};
use
defmt::info;
button_watch!
{
    ButtonWatch13 {
        pin: PIN_13,
    }
}
async fn
run(
    spawner: embassy_executor::Spawner,
    p: embassy_rp::Peripherals,
) ->
Result
<(), device_envoy_rp::Error> {
let
[wifi_credentials_flash_block, timezone_flash_block] = FlashBlockRp::new_array::<
2
>(p.FLASH)
?
;
static
TIMEZONE_STATIC: TimezoneFieldStatic = TimezoneField::new_static();
let
timezone_field = TimezoneField::new(
&
TIMEZONE_STATIC, timezone_flash_block);
let
button_watch13 = ButtonWatch13::new(p.PIN_13, PressedTo::Ground, spawner).
await
?
;
let
wifi_auto = WifiAutoRp::new(
        p.PIN_23,
        p.PIN_24,
        p.PIN_25,
        p.PIN_29,
        p.PIO0,
        p.DMA_CH0,
        wifi_credentials_flash_block,
"ClockSync"
,
        [timezone_field],
        spawner,
    )
?
;
let
stack = wifi_auto
        .connect(
&mut *
button_watch13, |event|
async move
{
match
event {
                WifiAutoEvent::CaptivePortalReady => {
info!
(
"WifiAutoRp: setup mode ready"
);
                }
                WifiAutoEvent::Connecting { .. } => {
info!
(
"WifiAutoRp: connecting"
);
                }
                WifiAutoEvent::ConnectionFailed => {
info!
(
"WifiAutoRp: connection failed"
);
                }
            }
Ok
(())
        })
        .
await
?
;
let
offset_minutes = timezone_field
        .offset_minutes()
?
.ok_or(Error::MissingCustomWifiAutoField)
?
;
static
CLOCK_SYNC_STATIC: ClockSyncStaticRp = ClockSyncRp::new_static();
let
clock_sync = ClockSyncRp::new(
&
CLOCK_SYNC_STATIC,
        stack,
        offset_minutes,
Some
(ONE_SECOND),
        spawner,
    )
?
;
loop
{
let
tick = clock_sync.wait_for_tick().
await
;
let
(hours, minutes, seconds) = h12_m_s(
&
tick.local_time);
info!
(
"Time {:02}:{:02}:{:02}, since sync {}s"
,
            hours,
            minutes,
            seconds,
            tick.since_last_sync.as_secs()
        );
    }
}

## Structs §

Clock Sync Rp A device abstraction that combines NTP time synchronization with a local clock. Clock Sync Static Rp Resources needed to construct ClockSyncRp . Clock Sync Tick Tick event emitted by ClockSync . Unix Seconds Units-safe wrapper for Unix timestamps (seconds since 1970-01-01 00:00:00 UTC).

## Enums §

Core Error Unified error type for device-envoy-core .

## Constants §

ONE_DAY Duration representing one day (24 hours). ONE_ MINUTE Duration representing one minute (60 seconds). ONE_ SECOND Duration representing one second.

## Traits §

Clock Sync Platform-agnostic ClockSync operation contract.

## Functions §

h12_m_s Extract hour (12-hour format), minute, and second from an OffsetDateTime .

## Type Aliases §

Core Result A specialized Result where the error is this crate’s Error type.

# Struct Clock Sync Rp Copy item path

Path: clock_sync/struct.ClockSyncRp.html

device_envoy_rp

::

clock_sync

# Struct Clock Sync Rp Copy item path

pub struct ClockSyncRp {
/* private fields */
}

Expand description

A device abstraction that combines NTP time synchronization with a local clock.

## Implementations §

§

### impl ClockSyncRuntime

#### pub const fn new_static () -> ClockSyncStatic

Create clock-sync static resources.

#### pub fn new ( clock_sync_static: &'static ClockSyncStatic , stack: &'static Stack<'static>, offset_minutes: i32 , tick_interval: Option <Duration>, spawner: Spawner, ) -> Result < ClockSyncRuntime , Error >

Create a clock-sync runtime using an existing network stack.

See the platform crate clock_sync module documentation for a full usage example. The tick_interval parameter uses embassy_time::Duration .

## Trait Implementations §

§

### impl ClockSync for ClockSyncRuntime

§

#### async fn wait_for_tick (&self) -> ClockSyncTick

Wait for and return the next tick after sync.

Read more

§

#### fn now_local (&self) -> OffsetDateTime

Get the current local time without waiting for a tick.

§

#### fn set_offset_minutes (&self, minutes: i32 )

Update the UTC offset used for local time.

§

#### fn offset_minutes (&self) -> i32

Get the current UTC offset in minutes.

§

#### fn set_tick_interval (&self, interval: Option <Duration>)

Set the tick interval. Use

None

to disable periodic ticks.

Read more

§

#### fn set_speed (&self, speed_multiplier: f32 )

Update the speed multiplier (1.0 = real time).

§

#### fn set_utc_time (&self, unix_seconds: UnixSeconds )

Manually set the current UTC time and mark the clock as synced.

## Auto Trait Implementations §

§

### impl Freeze for ClockSyncRuntime

§

### impl ! RefUnwindSafe for ClockSyncRuntime

§

### impl Send for ClockSyncRuntime

§

### impl Sync for ClockSyncRuntime

§

### impl Unpin for ClockSyncRuntime

§

### impl ! UnwindSafe for ClockSyncRuntime

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Clock Sync Static Rp Copy item path

Path: clock_sync/struct.ClockSyncStaticRp.html

device_envoy_rp

::

clock_sync

# Struct Clock Sync Static Rp Copy item path

pub struct ClockSyncStaticRp {
/* private fields */
}

Expand description

Resources needed to construct ClockSyncRp .

## Auto Trait Implementations §

§

### impl ! Freeze for ClockSyncStatic

§

### impl ! RefUnwindSafe for ClockSyncStatic

§

### impl Send for ClockSyncStatic

§

### impl Sync for ClockSyncStatic

§

### impl Unpin for ClockSyncStatic

§

### impl UnwindSafe for ClockSyncStatic

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Clock Sync Tick Copy item path

Path: clock_sync/struct.ClockSyncTick.html

device_envoy_rp

::

clock_sync

# Struct Clock Sync Tick Copy item path

pub struct ClockSyncTick {
    pub local_time: OffsetDateTime,
    pub since_last_sync: Duration,
}

Expand description

Tick event emitted by ClockSync .

See the platform crate’s clock_sync module for a usage example.

## Fields §

§ local_time: OffsetDateTime

The current local time (adjusted by timezone offset if set).

§ since_last_sync: Duration

Duration since the last successful NTP synchronization.

## Auto Trait Implementations §

§

### impl Freeze for ClockSyncTick

§

### impl RefUnwindSafe for ClockSyncTick

§

### impl Send for ClockSyncTick

§

### impl Sync for ClockSyncTick

§

### impl Unpin for ClockSyncTick

§

### impl UnwindSafe for ClockSyncTick

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Unix Seconds Copy item path

Path: clock_sync/struct.UnixSeconds.html

device_envoy_rp

::

clock_sync

# Struct Unix Seconds Copy item path

#[repr(transparent)]
pub struct UnixSeconds(pub
i64
);

Expand description

Units-safe wrapper for Unix timestamps (seconds since 1970-01-01 00:00:00 UTC).

## Tuple Fields §

§ 0: i64

## Implementations §

§

### impl UnixSeconds

#### pub const fn as_i64 (self) -> i64

Get the underlying i64 value.

#### pub const fn from_ntp_seconds (ntp: u32 ) -> Option < UnixSeconds >

Convert NTP seconds (since 1900-01-01) to Unix seconds (since 1970-01-01).

#### pub fn to_offset_datetime (self, offset: UtcOffset) -> Option <OffsetDateTime>

Convert to [ OffsetDateTime ] with the given timezone offset.

## Trait Implementations §

§

### impl Clone for UnixSeconds

§

#### fn clone (&self) -> UnixSeconds

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for UnixSeconds

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Format for UnixSeconds

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl Ord for UnixSeconds

§

#### fn cmp (&self, other: & UnixSeconds ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for UnixSeconds

§

#### fn eq (&self, other: & UnixSeconds ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for UnixSeconds

§

#### fn partial_cmp (&self, other: & UnixSeconds ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Copy for UnixSeconds

§

### impl Eq for UnixSeconds

§

### impl StructuralPartialEq for UnixSeconds

## Auto Trait Implementations §

§

### impl Freeze for UnixSeconds

§

### impl RefUnwindSafe for UnixSeconds

§

### impl Send for UnixSeconds

§

### impl Sync for UnixSeconds

§

### impl Unpin for UnixSeconds

§

### impl UnwindSafe for UnixSeconds

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Clock Sync Copy item path

Path: clock_sync/trait.ClockSync.html

device_envoy_rp

::

clock_sync

# Trait Clock Sync Copy item path

pub trait ClockSync {
    // Required methods
    async fn
wait_for_tick
(&self) ->
ClockSyncTick
;
fn
now_local
(&self) -> OffsetDateTime;
fn
set_offset_minutes
(&self, minutes:
i32
);
fn
offset_minutes
(&self) ->
i32
;
fn
set_tick_interval
(&self, interval:
Option
<Duration>);
fn
set_speed
(&self, speed_multiplier:
f32
);
fn
set_utc_time
(&self, unix_seconds:
UnixSeconds
);
}

Expand description

Platform-agnostic ClockSync operation contract.

Platform crates can use this trait for generic clock control helpers while keeping constructors on the concrete type.

## § Example

use
device_envoy_core::clock_sync::{ClockSync, h12_m_s};
async fn
log_one_tick(clock_sync:
&
impl
ClockSync) {
let
clock_sync_tick = clock_sync.wait_for_tick().
await
;
let
(hours, minutes, seconds) = h12_m_s(
&
clock_sync_tick.local_time);
let _
= (hours, minutes, seconds);
}

## Required Methods §

#### async fn wait_for_tick (&self) -> ClockSyncTick

Wait for and return the next tick after sync.

See the ClockSync trait documentation for usage examples.

#### fn now_local (&self) -> OffsetDateTime

Get the current local time without waiting for a tick.

#### fn set_offset_minutes (&self, minutes: i32 )

Update the UTC offset used for local time.

#### fn offset_minutes (&self) -> i32

Get the current UTC offset in minutes.

#### fn set_tick_interval (&self, interval: Option <Duration>)

Set the tick interval. Use None to disable periodic ticks.

This uses embassy_time::Duration for interval timing.

#### fn set_speed (&self, speed_multiplier: f32 )

Update the speed multiplier (1.0 = real time).

#### fn set_utc_time (&self, unix_seconds: UnixSeconds )

Manually set the current UTC time and mark the clock as synced.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

§

### impl ClockSync for ClockSyncRuntime

# Type Alias Core Result Copy item path

Path: clock_sync/type.CoreResult.html

device_envoy_rp

::

clock_sync

# Type Alias Core Result Copy item path

pub type CoreResult<T, E =
Error
> =
Result
<T, E>;

Expand description

A specialized Result where the error is this crate’s Error type.

## Aliased Type §

pub enum CoreResult<T, E =
Error
> {
    Ok(T),
    Err(E),
}

## Variants §

§

1.0.0

### Ok(T)

Contains the success value

§

1.0.0

### Err(E)

Contains the error value

# Module development_ guide Copy item path

Path: docs/development_guide/index.html

device_envoy_rp

::

docs

# Module development_ guide Copy item path

Source

Expand description

## § Development Guide

This guide is for editing the device-envoy workspace itself.

If you only want to use device-envoy-rp or device-envoy-esp in your own project, start from a template:

device-envoy-rp-blinky

device-envoy-esp-blinky

### § Toolchains and Targets

You may need all four embedded targets, depending on what you build:

RP Pico 1 (ARM): thumbv6m-none-eabi

RP Pico 2 (ARM): thumbv8m.main-none-eabihf

ESP32-C6 (RISC-V): riscv32imac-unknown-none-elf

ESP32-S3 (Xtensa): xtensa-esp32s3-none-elf

Install Rust targets:

rustup target add thumbv6m-none-eabi
rustup target add thumbv8m.main-none-eabihf
rustup target add riscv32imc-unknown-none-elf
rustup target add riscv32imac-unknown-none-elf

ESP32-S3 requires the ESP Rust toolchain/runtime flow used by this repo:

Install/use the +esp toolchain as needed for Xtensa builds.

Load your ESP environment before S3 commands (for this repo that is usually source "$HOME/export-esp.sh" ).

Use just run/check/build commands in crates/device-envoy-esp/justfile .

### § Run Examples

Use just from the crate you are in ( crates/device-envoy-rp or crates/device-envoy-esp ):

just run <name> [target]

just check <name> [target]

just build <name> [target]

Defaults:

ESP crate default target: c6

RP crate default target: 1

Common target suffixes:

ESP: c2 , c3 , c6 , h2 , esp32 , s2 , s3

RP: 1 , 2 , w , 2w

Examples:

cd crates/device-envoy-esp
just run blinky
just run blinky c2
just run blinky s3
just build f1 s3

cd ../device-envoy-rp
just run blinky
just run blinky 2
just check f1 2w

### § Run Full Workspace Checks

From the workspace root:

just check-all

Equivalent command:

cargo run --manifest-path xtask/Cargo.toml -- check-all

### § Generate Docs

From the workspace root:

just update-docs-rp
just update-docs-esp

Optional: build and open docs in a browser:

just show-docs-rp
just show-docs-esp

Note: show-docs-rp and show-docs-esp are currently WSL/Windows-oriented scripts.

### § Standard Pin Assignments

These are the default pins used by examples in this repository.

RP defaults:

PIN_0 : LED strip (8-pixel simple example)

PIN_1 : Single LED (blinky patterns)

PIN_3 : LED panel (12x4, 48 pixels)

PIN_4 : Extended LED panel (12x8, 96 pixels)

PIN_5 : Long LED strip (160 pixels, marquee effects)

PIN_6 : Large LED panel (16x16, 256 pixels)

PIN_8 : I2S audio data ( DIN )

PIN_9 : I2S bit clock ( BCLK )

PIN_10 : I2S word select ( LRC / LRCLK )

PIN_11 , PIN_12 : Servo signals

PIN_13 : Button (active-low)

ESP defaults are chip- and board-dependent.

For current source-of-truth pin/capability mappings, see:

crates/device-envoy-esp/xtask/src/boards.rs ( BOARD_PROFILES )

crates/device-envoy-esp/xtask/src/main.rs (capability-based example/test gating)

Generated board examples under crates/device-envoy-esp/examples/<chip>/<board>/

# Module docs Copy item path

Path: docs/index.html

device_envoy_rp

# Module docs Copy item path

Source

Expand description

Documentation-only pages for this crate.

## Modules §

development_ guide Development Guide

# Enum Error Copy item path

Path: enum.Error.html

device_envoy_rp

# Enum Error Copy item path

Source

pub enum Error {
Show 14 variants
TaskSpawn(SpawnError),
    BitsToIndexesNotEnoughSpace,
    BitsToIndexesFull,
    CannotSetOutputState,
    IndexOutOfBounds,
    Mfrc522Init(PCDErrorCode),
    Mfrc522Version(PCDErrorCode),
    FormatError,
    LcdText(LcdTextError),
    MissingCustomWifiAutoField,
    Ntp(&'static
str
),
    Flash(Error),
    StorageCorrupted,
    Core(
Error
),
}

Expand description

Define a unified error type for this crate.

## Variants §

§

### TaskSpawn(SpawnError)

§

### BitsToIndexesNotEnoughSpace

§

### BitsToIndexesFull

§

### CannotSetOutputState

§

### IndexOutOfBounds

§

### Mfrc522Init(PCDErrorCode)

§

### Mfrc522Version(PCDErrorCode)

§

### FormatError

§

### LcdText(LcdTextError)

§

### MissingCustomWifiAutoField

§

### Ntp(&'static str )

§

### Flash(Error)

§

### StorageCorrupted

§

### Core( Error )

## Trait Implementations §

Source

§

### impl Debug for Error

Source

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result

Formats the value using the given formatter.

Read more

Source

§

### impl Display for Error

Source

§

#### fn fmt (&self, __derive_more_f: &mut Formatter <'_>) -> Result

Formats the value using the given formatter.

Read more

Source

§

### impl Error for Error

1.30.0 · Source

§

#### fn source (&self) -> Option <&(dyn Error + 'static)>

Returns the lower-level source of this error, if any.

Read more

1.0.0 · Source

§

#### fn description (&self) -> & str

👎 Deprecated since 1.42.0: use the Display impl or to_string()

Read more

1.0.0 · Source

§

#### fn cause (&self) -> Option <&dyn Error >

👎 Deprecated since 1.33.0: replaced by Error::source, which can support downcasting

Source

§

#### fn provide <'a>(&'a self, request: &mut Request <'a>)

🔬 This is a nightly-only experimental API. ( error_generic_member_access )

Provides type-based access to context intended for error reports.

Read more

Source

§

### impl From < () > for Error

Source

§

#### fn from (_: () ) -> Self

Converts to this type from the input type.

Source

§

### impl From < Error > for Error

Source

§

#### fn from (error: Error ) -> Self

Converts to this type from the input type.

Source

§

### impl From < Infallible > for Error

Available on non-crate feature host only.

Source

§

#### fn from (value: Infallible ) -> Self

Converts to this type from the input type.

Source

§

### impl From <LcdTextError> for Error

Source

§

#### fn from (error: LcdTextError) -> Self

Converts to this type from the input type.

Source

§

### impl From <SpawnError> for Error

Source

§

#### fn from (err: SpawnError) -> Self

Converts to this type from the input type.

Source

§

### impl From <WifiAutoError> for Error

Source

§

#### fn from (error: WifiAutoError) -> Self

Converts to this type from the input type.

## Auto Trait Implementations §

§

### impl Freeze for Error

§

### impl RefUnwindSafe for Error

§

### impl Send for Error

§

### impl Sync for Error

§

### impl Unpin for Error

§

### impl UnwindSafe for Error

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# enum.Error.html

Path: error/enum.Error.html

Redirecting to ../../device_envoy_rp/enum.Error.html ...

# type.Result.html

Path: error/type.Result.html

Redirecting to ../../device_envoy_rp/type.Result.html ...

# Module flash_ block Copy item path

Path: flash_block/index.html

device_envoy_rp

# Module flash_ block Copy item path

Source

Expand description

A device abstraction for type-safe persistent storage in flash memory.

This module provides a generic flash block storage system that allows storing any serde -compatible type in Raspberry Pi Pico’s internal flash memory.

See FlashBlockRp for details and usage examples.

## Structs §

Flash Block Rp A device abstraction for type-safe persistent storage in flash memory.

## Traits §

Flash Block Operations on blocks of flash memory.

# Struct Flash Block Rp Copy item path

Path: flash_block/struct.FlashBlockRp.html

device_envoy_rp

::

flash_block

# Struct Flash Block Rp Copy item path

Source

pub struct FlashBlockRp {
/* private fields */
}

Expand description

A device abstraction for type-safe persistent storage in flash memory.

FlashBlockRp provides a generic flash-block storage system for Raspberry Pi Pico, allowing you to store any serde -compatible type in the device’s internal flash.

Use FlashBlockRp::new_array to allocate one or more blocks. Block operations like load , save , and clear are provided by FlashBlock , so bring the trait into scope:

use device_envoy_rp::flash_block::FlashBlock as _;

## § Features

Type safety : Hash-based type checking prevents reading data written under a different Rust type name. The hash is derived from the full type path (for example, app1::BootCounter ). Trying to read a different type returns Ok(None) . Structural changes (adding or removing fields) do not change the hash, but may cause deserialization to fail and return an error.

Postcard serialization : A compact, no_std -friendly binary format.

## § Block allocation

Conceptually, flash is treated as an array of fixed-size erase blocks counted from the end of memory backward. Your code can split that array using destructuring assignment and hand individual blocks to subsystems that need persistent storage.

⚠️ Warning : Pico 1 and Pico 2 store firmware, vector tables, and user data in the same flash device. Allocating too many blocks can overwrite your firmware.

## § Example

use
device_envoy_rp::flash_block::{FlashBlockRp, FlashBlock
as _
};
#[derive(serde::Serialize, serde::Deserialize, Debug, Clone, Copy)]
struct
BootCounter(u8);
impl
BootCounter {
const fn
new(value: u8) ->
Self
{
Self
(value)
    }
fn
increment(
self
) ->
Self
{
Self
((
self
.
0
+
1
) %
10
)
    }
}
async fn
example() -> device_envoy_rp::Result<Infallible> {
let
p = embassy_rp::init(Default::default());
let
[
mut
boot_counter_flash_block] = FlashBlockRp::new_array::<
1
>(p.FLASH)
?
;
let
boot_counter = boot_counter_flash_block
        .load()
?
.unwrap_or(BootCounter::new(
0
))
        .increment();

    boot_counter_flash_block.save(
&
boot_counter)
?
;
info!
(
"Boot counter: {}"
, boot_counter.
0
);
    pending().
await
}

## Implementations §

Source

§

### impl FlashBlockRp

Source

#### pub fn new_array <const N: usize >( peripheral: Peri<'static, FLASH>, ) -> Result <[ FlashBlockRp ; N ]>

Reserve N contiguous flash blocks and return them as an array.

## Trait Implementations §

Source

§

### impl Clone for FlashBlockRp

Source

§

#### fn clone (&self) -> FlashBlockRp

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

Source

§

### impl FlashBlock for FlashBlockRp

Source

§

#### type Error = Error

Error returned by block operations.

Source

§

#### fn load <T>(&mut self) -> Result < Option <T>> where T: Serialize + for<'de> Deserialize <'de>,

Load a typed value from this block.

Read more

Source

§

#### fn save <T>(&mut self, value: &T ) -> Result < () > where T: Serialize + for<'de> Deserialize <'de>,

Save a typed value to this block.

Read more

Source

§

#### fn clear (&mut self) -> Result < () >

Clear this block.

Read more

Source

§

### impl Copy for FlashBlockRp

## Auto Trait Implementations §

§

### impl Freeze for FlashBlockRp

§

### impl ! RefUnwindSafe for FlashBlockRp

§

### impl Send for FlashBlockRp

§

### impl Sync for FlashBlockRp

§

### impl Unpin for FlashBlockRp

§

### impl ! UnwindSafe for FlashBlockRp

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Flash Block Copy item path

Path: flash_block/trait.FlashBlock.html

device_envoy_rp

::

flash_block

# Trait Flash Block Copy item path

pub trait FlashBlock {
    type
Error
;

    // Required methods
    fn
load
<T>(&mut self) ->
Result
<
Option
<T>, Self::
Error
>
where T:
Serialize
+ for<'de>
Deserialize
<'de>
;
fn
save
<T>(&mut self, value:
&T
) ->
Result
<
()
, Self::
Error
>
where T:
Serialize
+ for<'de>
Deserialize
<'de>
;
fn
clear
(&mut self) ->
Result
<
()
, Self::
Error
>;
}

Expand description

Operations on blocks of flash memory.

Platform crates implement this trait on their concrete flash block handle types.

Constructors and hardware wiring remain platform-specific; this trait defines the shared operation surface used by higher-level abstractions.

## § Features

Type safety: hash-based type checking prevents reading data written under a different Rust type name. Trying to read a different type returns Ok(None) .

Postcard serialization: compact, no_std -friendly binary format.

This example increments a persisted boot counter and clears a separate scratch block in the same helper.

## § Example

use
core::convert::Infallible;
use
device_envoy_core::flash_block::FlashBlock;
#[derive(serde::Serialize, serde::Deserialize, Debug, Clone, Copy)]
struct
BootCounter(u8);
impl
BootCounter {
const fn
new(value: u8) ->
Self
{
Self
(value)
    }
fn
increment(
self
) ->
Self
{
Self
((
self
.
0
+
1
) %
10
)
    }
}
fn
update_boot_counter_and_clear_scratch(
    boot_counter_flash_block:
&mut
impl
FlashBlock<Error = Infallible>,
    scratch_flash_block:
&mut
impl
FlashBlock<Error = Infallible>,
) ->
Result
<BootCounter, Infallible> {
// Load the typed value, defaulting to 0 when the block is empty.
let
boot_counter = boot_counter_flash_block
        .load()
?
.unwrap_or(BootCounter::new(
0
))
        .increment();
// Save the updated value back to flash.
boot_counter_flash_block.save(
&
boot_counter)
?
;
// Clear the extra scratch block.
scratch_flash_block.clear()
?
;
Ok
(boot_counter)
}

## Required Associated Types §

#### type Error

Error returned by block operations.

## Required Methods §

#### fn load <T>(&mut self) -> Result < Option <T>, Self:: Error > where T: Serialize + for<'de> Deserialize <'de>,

Load a typed value from this block.

Returns Ok(None) when the block is empty or contains a different type.

See the FlashBlock trait documentation for usage examples.

#### fn save <T>(&mut self, value: &T ) -> Result < () , Self:: Error > where T: Serialize + for<'de> Deserialize <'de>,

Save a typed value to this block.

See the FlashBlock trait documentation for usage examples.

#### fn clear (&mut self) -> Result < () , Self:: Error >

Clear this block.

See the FlashBlock trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl FlashBlock for FlashBlockRp

Source

§

#### type Error = Error

# Enum IrEvent Copy item path

Path: ir/enum.IrEvent.html

device_envoy_rp

::

ir

# Enum IrEvent Copy item path

pub enum IrEvent {
    Press {
        addr:
u16
,
        cmd:
u8
,
    },
}

Expand description

Events received from the infrared receiver.

See the platform-specific crate for usage examples.

## Variants §

§

### Press

Button press with 16-bit address and 8-bit command. Supports both standard NEC (8-bit address) and extended NEC (16-bit address).

#### Fields

§ addr: u16

16-bit device address (or 8-bit address in low byte for standard NEC).

§ cmd: u8

8-bit command code.

## Trait Implementations §

§

### impl Clone for IrEvent

§

#### fn clone (&self) -> IrEvent

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for IrEvent

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl PartialEq for IrEvent

§

#### fn eq (&self, other: & IrEvent ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for IrEvent

§

### impl StructuralPartialEq for IrEvent

## Auto Trait Implementations §

§

### impl Freeze for IrEvent

§

### impl RefUnwindSafe for IrEvent

§

### impl Send for IrEvent

§

### impl Sync for IrEvent

§

### impl Unpin for IrEvent

§

### impl UnwindSafe for IrEvent

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Enum Kepler Keys Copy item path

Path: ir/enum.KeplerKeys.html

device_envoy_rp

::

ir

# Enum Kepler Keys Copy item path

pub enum KeplerKeys {
    Power,
    Mode,
    Mute,
    PlayPause,
    Prev,
    Next,
    Eq,
    Minus,
    Plus,
    Num(
u8
),
    Repeat,
    USd,
}

Expand description

Button types for the SunFounder Kepler Kit remote control.

## Variants §

§

### Power

Power button.

§

### Mode

Mode button.

§

### Mute

Mute button.

§

### PlayPause

Play/Pause button.

§

### Prev

Previous track button.

§

### Next

Next track button.

§

### Eq

Equalizer button.

§

### Minus

Minus/Decrease volume button.

§

### Plus

Plus/Increase volume button.

§

### Num( u8 )

Numbered button (0-9).

§

### Repeat

Repeat button.

§

### USd

USB/SD card mode button.

## Trait Implementations §

§

### impl Clone for KeplerKeys

§

#### fn clone (&self) -> KeplerKeys

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for KeplerKeys

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Format for KeplerKeys

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl PartialEq for KeplerKeys

§

#### fn eq (&self, other: & KeplerKeys ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for KeplerKeys

§

### impl Eq for KeplerKeys

§

### impl StructuralPartialEq for KeplerKeys

## Auto Trait Implementations §

§

### impl Freeze for KeplerKeys

§

### impl RefUnwindSafe for KeplerKeys

§

### impl Send for KeplerKeys

§

### impl Sync for KeplerKeys

§

### impl Unpin for KeplerKeys

§

### impl UnwindSafe for KeplerKeys

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module ir Copy item path

Path: ir/index.html

device_envoy_rp

# Module ir Copy item path

Source

Expand description

A device abstraction for infrared receivers using the NEC protocol.

This page provides the primary documentation and examples for receiving NEC infrared input on RP devices. It covers raw address/command events, mapped application keys, and Kepler remote keys. Traits define the shared API; macros generate concrete device types. Choose ir! for raw NEC events, ir_mapping! when mapping to your own enum, and ir_kepler! for the SunFounder Kepler remote.

After reading the examples below, see also:

Ir: Raw events — ir! , Ir , IrGenerated

IrMapping: Mapped events — ir_mapping! , IrMapping , IrMappingGenerated

IrKepler: Kepler mapped events — ir_kepler! , IrKepler , IrKeplerGenerated

## § Example: Read Raw NEC Events

In this example, the generated Ir15 type emits raw NEC press events with address and command bytes.

use
device_envoy_rp::{
Result
, ir, ir::Ir
as _
, ir::IrEvent};
ir!
{
    Ir15 { pio: PIO0, pin: PIN_15 }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
ir15 = Ir15::new(p.PIO0, p.PIN_15, spawner)
?
;
loop
{
let
IrEvent::Press { addr, cmd } = ir15.wait_for_press().
await
;
info!
(
"IR press: addr=0x{:04X}, cmd=0x{:02X}"
, addr, cmd);
    }
}

## § Example: Map NEC Events to App Keys

In this example, the generated IrMapping15 type maps raw NEC address/command pairs into an application-defined enum.

use
device_envoy_rp::{
Result
, ir::IrMapping
as _
, ir_mapping};
#[derive(Clone, Copy, Debug, Eq, PartialEq)]
enum
RemoteKeys {
    Power,
    Plus,
    Minus,
}
ir_mapping!
{
    IrMapping15 {
        pio: PIO0,
        pin: PIN_15,
        button: RemoteKeys,
        capacity:
3
,
    }
}
const
REMOTE_KEYS_MAP: [(u16, u8, RemoteKeys);
3
] = [
    (
0x0000
,
0x45
, RemoteKeys::Power),
    (
0x0000
,
0x09
, RemoteKeys::Plus),
    (
0x0000
,
0x15
, RemoteKeys::Minus),
];
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
ir_mapping15 = IrMapping15::new(p.PIO0, p.PIN_15,
&
REMOTE_KEYS_MAP, spawner)
?
;
loop
{
let
remote_key = ir_mapping15.wait_for_press().
await
;
match
remote_key {
            RemoteKeys::Power => {}
            RemoteKeys::Plus => {}
            RemoteKeys::Minus => {}
        }
    }
}

## § Example: Read Kepler Remote Keys

In this example, the generated IrKepler15 type returns typed keys from the SunFounder Kepler remote key mapping.

use
device_envoy_rp::{
Result
, ir::IrKepler
as _
, ir::KeplerKeys, ir_kepler};
ir_kepler!
{
    IrKepler15 { pio: PIO0, pin: PIN_15 }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
ir_kepler15 = IrKepler15::new(p.PIO0, p.PIN_15, spawner)
?
;
loop
{
let
kepler_key = ir_kepler15.wait_for_press().
await
;
match
kepler_key {
            KeplerKeys::Power =>
info!
(
"Power"
),
            KeplerKeys::PlayPause =>
info!
(
"PlayPause"
),
_
=>
info!
(
"Other: {:?}"
, kepler_key),
        }
    }
}

## Modules §

ir_ generated Module containing sample struct types generated by IR macros: IrGenerated , IrMappingGenerated , and IrKeplerGenerated .

## Macros §

ir Macro to generate an IR receiver struct type (includes syntax details). ir_ kepler Macro to generate a Kepler IR struct type (includes syntax details). ir_ keplers Macro to generate multiple Kepler IR struct types that share one PIO resource (includes syntax details). ir_ mapping Macro to generate an IR mapping struct type (includes syntax details). ir_ mappings Macro to generate multiple IR mapping struct types that share one PIO resource (includes syntax details). irs Macro to generate multiple IR receiver struct types that share one PIO resource (includes syntax details).

## Enums §

IrEvent Events received from the infrared receiver. Kepler Keys Button types for the SunFounder Kepler Kit remote control.

## Traits §

Ir Platform-agnostic infrared receiver device contract. IrKepler Platform-agnostic SunFounder Kepler IR device contract. IrMapping Platform-agnostic IR button mapper device contract.

# Enum Remote Keys Generated Copy item path

Path: ir/ir_generated/enum.RemoteKeysGenerated.html

device_envoy_rp

::

ir :: ir_generated

# Enum Remote Keys Generated Copy item path

Source

pub enum RemoteKeysGenerated {
    Power,
    Play,
    Stop,
}

Expand description

Sample key enum used by IrMappingGenerated .

## Variants §

§

### Power

Power key.

§

### Play

Play key.

§

### Stop

Stop key.

## Trait Implementations §

Source

§

### impl Clone for RemoteKeysGenerated

Source

§

#### fn clone (&self) -> RemoteKeysGenerated

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

Source

§

### impl Debug for RemoteKeysGenerated

Source

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result

Formats the value using the given formatter.

Read more

Source

§

### impl IrMapping < RemoteKeysGenerated > for IrMappingGenerated

Source

§

#### async fn wait_for_press (&self) -> RemoteKeysGenerated

Wait for the next recognized mapped button press.

Read more

Source

§

### impl PartialEq for RemoteKeysGenerated

Source

§

#### fn eq (&self, other: & RemoteKeysGenerated ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

Source

§

### impl Copy for RemoteKeysGenerated

Source

§

### impl Eq for RemoteKeysGenerated

Source

§

### impl StructuralPartialEq for RemoteKeysGenerated

## Auto Trait Implementations §

§

### impl Freeze for RemoteKeysGenerated

§

### impl RefUnwindSafe for RemoteKeysGenerated

§

### impl Send for RemoteKeysGenerated

§

### impl Sync for RemoteKeysGenerated

§

### impl Unpin for RemoteKeysGenerated

§

### impl UnwindSafe for RemoteKeysGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module ir_ generated Copy item path

Path: ir/ir_generated/index.html

device_envoy_rp

::

ir

# Module ir_ generated Copy item path

Source

Expand description

Module containing sample struct types generated by IR macros: IrGenerated , IrMappingGenerated , and IrKeplerGenerated .

Auto-generated.

## Structs §

IrGenerated Sample struct type generated by the ir! macro. IrKepler Generated Sample struct type generated by the ir_kepler! macro. IrMapping Generated Sample struct type generated by the ir_mapping! macro.

## Enums §

Remote Keys Generated Sample key enum used by IrMappingGenerated .

# Struct IrGenerated Copy item path

Path: ir/ir_generated/struct.IrGenerated.html

device_envoy_rp

::

ir :: ir_generated

# Struct IrGenerated Copy item path

Source

pub struct IrGenerated;

Expand description

Sample struct type generated by the ir! macro.

This page exists to show constructor and trait methods in one place. For narrative examples, see the ir module.

Auto-generated.

## Implementations §

Source

§

### impl IrGenerated

Source

#### pub fn new ( pio: Peri<'static, PIO0>, pin: Peri<'static, PIN_15>, spawner: Spawner, ) -> Result <&'static Self>

Create a new IR receiver instance of the struct type defined by ir! .

See the ir module docs for usage examples.

## Trait Implementations §

Source

§

### impl Ir for IrGenerated

Source

§

#### async fn wait_for_press (&self) -> IrEvent

Wait for the next IR event.

Read more

## Auto Trait Implementations §

§

### impl Freeze for IrGenerated

§

### impl RefUnwindSafe for IrGenerated

§

### impl Send for IrGenerated

§

### impl Sync for IrGenerated

§

### impl Unpin for IrGenerated

§

### impl UnwindSafe for IrGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct IrKepler Generated Copy item path

Path: ir/ir_generated/struct.IrKeplerGenerated.html

device_envoy_rp

::

ir :: ir_generated

# Struct IrKepler Generated Copy item path

Source

pub struct IrKeplerGenerated;

Expand description

Sample struct type generated by the ir_kepler! macro.

This page exists to show constructor and trait methods in one place. For narrative examples, see the ir module.

Auto-generated.

## Implementations §

Source

§

### impl IrKeplerGenerated

Source

#### pub fn new ( pio: Peri<'static, PIO0>, pin: Peri<'static, PIN_15>, spawner: Spawner, ) -> Result <&'static Self>

Create a new Kepler IR instance of the struct type defined by ir_kepler! .

See the ir module docs for usage examples.

## Trait Implementations §

Source

§

### impl IrKepler for IrKeplerGenerated

Source

§

#### async fn wait_for_press (&self) -> KeplerKeys

Wait for the next recognized Kepler button press.

Read more

## Auto Trait Implementations §

§

### impl Freeze for IrKeplerGenerated

§

### impl RefUnwindSafe for IrKeplerGenerated

§

### impl Send for IrKeplerGenerated

§

### impl Sync for IrKeplerGenerated

§

### impl Unpin for IrKeplerGenerated

§

### impl UnwindSafe for IrKeplerGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct IrMapping Generated Copy item path

Path: ir/ir_generated/struct.IrMappingGenerated.html

device_envoy_rp

::

ir :: ir_generated

# Struct IrMapping Generated Copy item path

Source

pub struct IrMappingGenerated;

Expand description

Sample struct type generated by the ir_mapping! macro.

This page exists to show constructor and trait methods in one place. For narrative examples, see the ir module.

Auto-generated.

## Implementations §

Source

§

### impl IrMappingGenerated

Source

#### pub fn new ( pio: Peri<'static, PIO0>, pin: Peri<'static, PIN_15>, button_map: &[( u16 , u8 , RemoteKeysGenerated )], spawner: Spawner, ) -> Result <&'static Self>

Create a new IR mapping instance of the struct type defined by ir_mapping! .

See the ir module docs for usage examples.

## Trait Implementations §

Source

§

### impl IrMapping < RemoteKeysGenerated > for IrMappingGenerated

Source

§

#### async fn wait_for_press (&self) -> RemoteKeysGenerated

Wait for the next recognized mapped button press.

Read more

## Auto Trait Implementations §

§

### impl Freeze for IrMappingGenerated

§

### impl RefUnwindSafe for IrMappingGenerated

§

### impl Send for IrMappingGenerated

§

### impl Sync for IrMappingGenerated

§

### impl Unpin for IrMappingGenerated

§

### impl UnwindSafe for IrMappingGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# enum.KeplerKeys.html

Path: ir/kepler/enum.KeplerKeys.html

Redirecting to ../../../device_envoy_rp/ir/enum.KeplerKeys.html ...

# macro.ir!.html

Path: ir/macro.ir!.html

Redirecting to macro.ir.html ...

# Macro ir Copy item path

Path: ir/macro.ir.html

device_envoy_rp

::

ir

# Macro ir Copy item path

Source

macro_rules! ir {
    (
        $(#[$attrs:meta])*
        $vis:vis $name:ident : { $($deprecated_fields:tt)* }
    ) => { ... };
    (
        $(#[$attrs:meta])*
        $vis:vis $name:ident { pio: $pio:ident, pin: $pin:ident $(,)? }
    ) => { ... };
}

Expand description

Macro to generate an IR receiver struct type (includes syntax details).

See the ir module documentation for usage examples.

Syntax:

ir! {
    [attrs...]
    [vis?] <Name> {
        pio: <pio_ident>,
        pin: <pin_ident>,
    }
}

Required fields:

pio — PIO resource (for example PIO0 or PIO1 )

pin — GPIO input pin connected to the IR receiver

## § Related Macros

irs! — Share one PIO resource with multiple IR receivers

ir_mapping! — Generate a mapped-button IR receiver type

# macro.ir_kepler!.html

Path: ir/macro.ir_kepler!.html

Redirecting to macro.ir_kepler.html ...

# Macro ir_ kepler Copy item path

Path: ir/macro.ir_kepler.html

device_envoy_rp

::

ir

# Macro ir_ kepler Copy item path

Source

macro_rules! ir_kepler {
    (
        $(#[$attrs:meta])*
        $vis:vis $name:ident : { $($deprecated_fields:tt)* }
    ) => { ... };
    (
        $(#[$attrs:meta])*
        $vis:vis $name:ident { pio: $pio:ident, pin: $pin:ident $(,)? }
    ) => { ... };
}

Expand description

Macro to generate a Kepler IR struct type (includes syntax details).

See the ir module documentation for usage examples.

Syntax:

ir_kepler! {
    [attrs...]
    [vis?] <Name> {
        pio: <pio_ident>,
        pin: <pin_ident>,
    }
}

Required fields:

pio — PIO resource (for example PIO0 or PIO1 )

pin — GPIO input pin connected to the IR receiver

## § Related Macros

ir_keplers! — Share one PIO resource with multiple Kepler IR receivers

ir! — Generate a raw IR receiver type

# macro.ir_keplers!.html

Path: ir/macro.ir_keplers!.html

Redirecting to macro.ir_keplers.html ...

# Macro ir_ keplers Copy item path

Path: ir/macro.ir_keplers.html

device_envoy_rp

::

ir

# Macro ir_ keplers Copy item path

Source

macro_rules! ir_keplers {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate multiple Kepler IR struct types that share one PIO resource (includes syntax details).

See the ir module documentation for usage examples.

Syntax:

ir_keplers! {
    pio: <pio_ident>,
    <GroupName> {
        <Name0>: { pin: <pin0_ident> },
        <Name1>: { pin: <pin1_ident> }, // optional
        <Name2>: { pin: <pin2_ident> }, // optional
        <Name3>: { pin: <pin3_ident> }, // optional
    }
}

Required fields:

pio — Shared PIO resource (for example PIO0 or PIO1 )

pin — One pin entry per generated receiver

Supports one to four generated receivers per invocation.

## § Related Macros

ir_kepler! — Generate a single Kepler IR receiver type

irs! — Generate raw IR receivers sharing one PIO resource

# macro.ir_mapping!.html

Path: ir/macro.ir_mapping!.html

Redirecting to macro.ir_mapping.html ...

# Macro ir_ mapping Copy item path

Path: ir/macro.ir_mapping.html

device_envoy_rp

::

ir

# Macro ir_ mapping Copy item path

Source

macro_rules! ir_mapping {
    (
        $(#[$attrs:meta])*
        $vis:vis $name:ident : { $($deprecated_fields:tt)* }
    ) => { ... };
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate an IR mapping struct type (includes syntax details).

See the ir module documentation for usage examples.

Syntax:

ir_mapping! {
    [attrs...]
    [vis?] <Name> {
        pio: <pio_ident>,
        pin: <pin_ident>,
        button: <button_type>,
        capacity: <usize_expr>,
    }
}

Required fields:

pio — PIO resource (for example PIO0 or PIO1 )

pin — GPIO input pin connected to the IR receiver

button — Output button/key type for mapping

capacity — Maximum mapping entries ( heapless::LinearMap capacity); must be at least the number of mapping entries you provide

## § Related Macros

ir_mappings! — Share one PIO resource with multiple mapping receivers

ir! — Generate a raw IR receiver type

# macro.ir_mappings!.html

Path: ir/macro.ir_mappings!.html

Redirecting to macro.ir_mappings.html ...

# Macro ir_ mappings Copy item path

Path: ir/macro.ir_mappings.html

device_envoy_rp

::

ir

# Macro ir_ mappings Copy item path

Source

macro_rules! ir_mappings {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate multiple IR mapping struct types that share one PIO resource (includes syntax details).

See the ir module documentation for usage examples.

Syntax:

ir_mappings! {
    pio: <pio_ident>,
    button: <button_type>,
    capacity: <usize_expr>,
    <GroupName> {
        <Name0>: { pin: <pin0_ident> },
        <Name1>: { pin: <pin1_ident> }, // optional
        <Name2>: { pin: <pin2_ident> }, // optional
        <Name3>: { pin: <pin3_ident> }, // optional
    }
}

Required fields:

pio — Shared PIO resource (for example PIO0 or PIO1 )

button — Output button/key type for all generated mappings

capacity — Maximum mapping entries ( heapless::LinearMap capacity); must be at least the number of mapping entries you provide

pin — One pin entry per generated mapping receiver

Supports one to four generated mapping receivers per invocation.

## § Related Macros

ir_mapping! — Generate a single IR mapping receiver type

irs! — Generate raw IR receivers sharing one PIO resource

# macro.irs!.html

Path: ir/macro.irs!.html

Redirecting to macro.irs.html ...

# Macro irs Copy item path

Path: ir/macro.irs.html

device_envoy_rp

::

ir

# Macro irs Copy item path

Source

macro_rules! irs {
    (
        pio: $pio:ident,
        $group_name:ident {
            $first_name:ident : { pin: $first_pin:ident $(,)? }
            $(, $rest_name:ident : { pin: $rest_pin:ident $(,)? })* $(,)?
        }
    ) => { ... };
}

Expand description

Macro to generate multiple IR receiver struct types that share one PIO resource (includes syntax details).

See the ir module documentation for usage examples.

Syntax:

irs! {
    pio: <pio_ident>,
    <GroupName> {
        <Name0>: { pin: <pin0_ident> },
        <Name1>: { pin: <pin1_ident> }, // optional
        <Name2>: { pin: <pin2_ident> }, // optional
        <Name3>: { pin: <pin3_ident> }, // optional
    }
}

Required fields:

pio — Shared PIO resource (for example PIO0 or PIO1 )

pin — One pin entry per generated receiver

Supports one to four generated receivers per invocation.

## § Related Macros

ir! — Generate a single IR receiver type

ir_mappings! — Generate mapped-button receivers sharing one PIO

# Trait Ir Copy item path

Path: ir/trait.Ir.html

device_envoy_rp

::

ir

# Trait Ir Copy item path

pub trait Ir {
    // Required method
    async fn
wait_for_press
(&self) ->
IrEvent
;
}

Expand description

Platform-agnostic infrared receiver device contract.

Platform crates implement this for their concrete Ir types so shared logic can wait for NEC button press events without depending on platform-specific modules.

This trait is the core wait_for_press surface used by platform IR receiver wrappers.

## § Example

use
device_envoy_core::ir::{Ir, IrEvent};
async fn
handle_ir_presses(ir:
&
impl
Ir) -> ! {
loop
{
let
ir_event = ir.wait_for_press().
await
;
match
ir_event {
            IrEvent::Press { addr, cmd } => {
// Handle decoded NEC press event.
let _
= (addr, cmd);
            }
        }
    }
}

## Required Methods §

#### async fn wait_for_press (&self) -> IrEvent

Wait for the next IR event.

See the Ir trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Ir for IrGenerated

# Trait IrKepler Copy item path

Path: ir/trait.IrKepler.html

device_envoy_rp

::

ir

# Trait IrKepler Copy item path

pub trait IrKepler {
    // Required method
    async fn
wait_for_press
(&self) ->
KeplerKeys
;
}

Expand description

Platform-agnostic SunFounder Kepler IR device contract.

Platform crates implement this for their concrete IrKepler types so shared logic can wait for button presses without depending on platform-specific modules.

This trait provides a simple interface with built-in Kepler button mappings.

## § Example

use
device_envoy_core::ir::{IrKepler, KeplerKeys};
async fn
handle_kepler_button_presses(ir_kepler:
&
impl
IrKepler) -> ! {
loop
{
let
kepler_button = ir_kepler.wait_for_press().
await
;
// Use mapped Kepler button in app logic.
match
kepler_button {
            KeplerKeys::Power => {
// Handle power.
}
            KeplerKeys::PlayPause => {
// Handle play/pause.
}
_
=> {
// Handle all other Kepler buttons.
}
        }
    }
}

## Required Methods §

#### async fn wait_for_press (&self) -> KeplerKeys

Wait for the next recognized Kepler button press.

See the IrKepler trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementations on Foreign Types §

§

### impl<T> IrKepler for &T where T: IrKepler + ? Sized ,

§

#### async fn wait_for_press (&self) -> KeplerKeys

## Implementors §

Source

§

### impl IrKepler for IrKeplerGenerated

# Trait IrMapping Copy item path

Path: ir/trait.IrMapping.html

device_envoy_rp

::

ir

# Trait IrMapping Copy item path

pub trait IrMapping<Button> {
    // Required method
    async fn
wait_for_press
(&self) -> Button;
}

Expand description

Platform-agnostic IR button mapper device contract.

Platform crates implement this for their concrete IrMapping types so shared logic can wait for mapped button presses without depending on platform-specific modules.

This trait is intended for app-level button enums mapped from (addr, cmd) pairs in platform-specific constructors.

## § Example

use
device_envoy_core::ir::IrMapping;
#[derive(Clone, Copy, Debug, Eq, PartialEq)]
enum
RemoteKeys {
    Power,
    Play,
    Stop,
}
async fn
handle_mapped_button_presses(ir_mapping:
&
impl
IrMapping<RemoteKeys>) -> ! {
loop
{
let
remote_key = ir_mapping.wait_for_press().
await
;
// Use mapped key in app logic.
match
remote_key {
            RemoteKeys::Power => {
// Handle power.
}
            RemoteKeys::Play => {
// Handle play.
}
            RemoteKeys::Stop => {
// Handle stop.
}
        }
    }
}

## Required Methods §

#### async fn wait_for_press (&self) -> Button

Wait for the next recognized mapped button press.

See the IrMapping trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl IrMapping < RemoteKeysGenerated > for IrMappingGenerated

# Module lcd_ text Copy item path

Path: lcd_text/index.html

device_envoy_rp

# Module lcd_ text Copy item path

Source

Expand description

A device abstraction for HD44780-compatible character LCDs (e.g., 16x2, 20x2, 20x4).

This page provides the primary documentation and examples for LCD text devices.

After reading the examples below, see also:

lcd_text! — Macro to generate a single LCD text type (includes syntax details).

i2cs! — Macro to generate multiple LCD text types sharing one I2C resource (includes syntax details).

LcdTextGenerated — Sample generated LCD text type showing the constructor path.

I2csGenerated — Sample generated I2C group type for multiple LCD text devices.

LcdText — Core LCD text trait implemented by generated types.

## § Text Behavior

write_text(...) behavior:

\n starts a new LCD row.

characters past WIDTH are ignored

rows past HEIGHT are ignored

Non-ASCII Unicode characters are replaced with ? .

Missing characters are padded with spaces.

## § Example: Write Text on One LCD

In this example, the generated type is LcdTextSimple .

use
device_envoy_rp::{
Result
, lcd_text, lcd_text::LcdText
as _
};
lcd_text!
{
    i2c: I2C0,
    sda_pin: PIN_4,
    scl_pin: PIN_5,
    LcdTextSimple {
        width:
16
,
        height:
2
,
        address:
0x27
}
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
lcd_text_simple = LcdTextSimple::new(p.I2C0, p.PIN_4, p.PIN_5, spawner)
?
;

    lcd_text_simple.write_text(
"Hello from\ndevice-envoy!"
);

    pending().
await
}

## § Example: Two LCDs Sharing One I2C Peripheral

In this example, the generated group type is I2cs0 .

use
device_envoy_rp::{
Result
, i2cs, lcd_text::LcdText
as _
};
i2cs!
{
    i2c: I2C0,
    sda_pin: PIN_4,
    scl_pin: PIN_5,
    I2cs0 {
        LcdText16x2 { width:
16
, height:
2
, address:
0x27
},
        LcdText20x4 { width:
20
, height:
4
, address:
0x3F
},
    }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
(lcd_text16x2, lcd_text20x4) = I2cs0::new(p.I2C0, p.PIN_4, p.PIN_5, spawner)
?
;

    lcd_text16x2.write_text(
"16x2\nready"
);
    lcd_text20x4.write_text(
"20x4\nshared i2c\naddress 0x3F"
);

    pending().
await
}

## Modules §

lcd_ text_ generated Module containing generated LCD text sample types.

## Macros §

i2cs Macro to generate multiple LCD text device types that share one I2C resource (includes syntax details). lcd_ text Macro to generate a single LCD text device type with a direct constructor.

## Traits §

LcdText Platform-agnostic LCD text device contract.

# Module lcd_ text_ generated Copy item path

Path: lcd_text/lcd_text_generated/index.html

device_envoy_rp

::

lcd_text

# Module lcd_ text_ generated Copy item path

Source

Expand description

Module containing generated LCD text sample types.

Auto-generated.

## Structs §

I2cs Generated Sample I2C LCD group type generated by i2cs! . LcdText Generated Sample struct type generated by the i2cs! macro. LcdText Generated20x4 Sample LCD text struct type generated by i2cs! .

# Struct I2cs Generated Copy item path

Path: lcd_text/lcd_text_generated/struct.I2csGenerated.html

device_envoy_rp

::

lcd_text :: lcd_text_generated

# Struct I2cs Generated Copy item path

Source

pub struct I2csGenerated;

Expand description

Sample I2C LCD group type generated by i2cs! .

## Implementations §

Source

§

### impl I2csGenerated

Source

#### pub fn new ( i2c_peripheral: Peri<'static, I2C0>, sda: Peri<'static, PIN_4>, scl: Peri<'static, PIN_5>, spawner: Spawner, ) -> Result <(&'static LcdTextGenerated , &'static LcdTextGenerated20x4 )>

Construct all generated LCD text devices in this group. See the lcd_text module documentation for usage examples.

## Auto Trait Implementations §

§

### impl Freeze for I2csGenerated

§

### impl RefUnwindSafe for I2csGenerated

§

### impl Send for I2csGenerated

§

### impl Sync for I2csGenerated

§

### impl Unpin for I2csGenerated

§

### impl UnwindSafe for I2csGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct LcdText Generated Copy item path

Path: lcd_text/lcd_text_generated/struct.LcdTextGenerated.html

device_envoy_rp

::

lcd_text :: lcd_text_generated

# Struct LcdText Generated Copy item path

Source

pub struct LcdTextGenerated;

Expand description

Sample struct type generated by the i2cs! macro.

This page exists to show constructor and methods in one place. For narrative examples, see the lcd_text module.

Auto-generated.

## Implementations §

Source

§

### impl LcdTextGenerated

Source

#### pub const WIDTH : usize = 16

Display width in characters.

Source

#### pub const HEIGHT : usize = 2

Display height in characters.

Source

#### pub const ADDRESS : u8 = 0x27

LCD I2C address.

Source

#### pub fn new ( i2c_peripheral: Peri<'static, I2C0>, sda: Peri<'static, PIN_4>, scl: Peri<'static, PIN_5>, spawner: Spawner, ) -> Result <&'static Self>

Create this generated LCD text instance. See the lcd_text module documentation for usage examples.

## Trait Implementations §

Source

§

### impl LcdText <16, 2> for LcdTextGenerated

Source

§

#### const ADDRESS : u8 = 0x27

LCD I2C address.

Source

§

#### fn write_text (&self, text: impl AsRef < str >)

Write text to the display. See the

LcdText trait documentation

for usage examples.

§

#### const WIDTH : usize = W

Display width in characters.

§

#### const HEIGHT : usize = H

Display height in characters.

## Auto Trait Implementations §

§

### impl Freeze for LcdTextGenerated

§

### impl RefUnwindSafe for LcdTextGenerated

§

### impl Send for LcdTextGenerated

§

### impl Sync for LcdTextGenerated

§

### impl Unpin for LcdTextGenerated

§

### impl UnwindSafe for LcdTextGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct LcdText Generated20x4 Copy item path

Path: lcd_text/lcd_text_generated/struct.LcdTextGenerated20x4.html

device_envoy_rp

::

lcd_text :: lcd_text_generated

# Struct LcdText Generated20x4 Copy item path

Source

pub struct LcdTextGenerated20x4;

Expand description

Sample LCD text struct type generated by i2cs! .

## Implementations §

Source

§

### impl LcdTextGenerated20x4

Source

#### pub const WIDTH : usize = 20

Display width in characters.

Source

#### pub const HEIGHT : usize = 4

Display height in characters.

Source

#### pub const ADDRESS : u8 = 0x3F

LCD I2C address.

Source

#### pub fn new ( i2c_peripheral: Peri<'static, I2C0>, sda: Peri<'static, PIN_4>, scl: Peri<'static, PIN_5>, spawner: Spawner, ) -> Result <&'static Self>

Create this generated LCD text instance. See the lcd_text module documentation for usage examples.

## Trait Implementations §

Source

§

### impl LcdText <20, 4> for LcdTextGenerated20x4

Source

§

#### const ADDRESS : u8 = 0x3F

LCD I2C address.

Source

§

#### fn write_text (&self, text: impl AsRef < str >)

Write text to the display. See the

LcdText trait documentation

for usage examples.

§

#### const WIDTH : usize = W

Display width in characters.

§

#### const HEIGHT : usize = H

Display height in characters.

## Auto Trait Implementations §

§

### impl Freeze for LcdTextGenerated20x4

§

### impl RefUnwindSafe for LcdTextGenerated20x4

§

### impl Send for LcdTextGenerated20x4

§

### impl Sync for LcdTextGenerated20x4

§

### impl Unpin for LcdTextGenerated20x4

§

### impl UnwindSafe for LcdTextGenerated20x4

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# macro.i2cs!.html

Path: lcd_text/macro.i2cs!.html

Redirecting to macro.i2cs.html ...

# Macro i2cs Copy item path

Path: lcd_text/macro.i2cs.html

device_envoy_rp

::

lcd_text

# Macro i2cs Copy item path

Source

macro_rules! i2cs {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate multiple LCD text device types that share one I2C resource (includes syntax details).

This page provides the primary documentation and examples for grouped LCD text devices that share one I2C peripheral.

Syntax:

i2cs! {
    i2c: <i2c_ident>,
    sda_pin: <sda_pin_ident>,
    scl_pin: <scl_pin_ident>,
    [<visibility>] <GroupName> {
        [<visibility>] <LcdName> {
            width: <usize_expr>,
            height: <usize_expr>,
            address: <u8_expr>
        },
        // ...more LCD entries...
    }
}

For a single LCD type, see lcd_text! .

See the lcd_text module documentation for usage examples.

# macro.lcd_text!.html

Path: lcd_text/macro.lcd_text!.html

Redirecting to macro.lcd_text.html ...

# Macro lcd_ text Copy item path

Path: lcd_text/macro.lcd_text.html

device_envoy_rp

::

lcd_text

# Macro lcd_ text Copy item path

Source

macro_rules! lcd_text {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate a single LCD text device type with a direct constructor.

For multiple LCD types sharing one I2C peripheral, see i2cs! .

Syntax:

lcd_text! {
    i2c: <i2c_ident>,
    sda_pin: <sda_pin_ident>,
    scl_pin: <scl_pin_ident>,
    [<visibility>] <LcdName> {
        width: <usize_expr>,
        height: <usize_expr>,
        address: <u8_expr>
    }
}

See the lcd_text module documentation for usage examples.

# Trait LcdText Copy item path

Path: lcd_text/trait.LcdText.html

device_envoy_rp

::

lcd_text

# Trait LcdText Copy item path

pub trait LcdText<const W:
usize
, const H:
usize
> {
    const
ADDRESS
:
u8
;
    const
WIDTH
:
usize
= W;
    const
HEIGHT
:
usize
= H;

    // Required method
    fn
write_text
(&self, text: impl
AsRef
<
str
>);
}

Expand description

Platform-agnostic LCD text device contract.

Platform crates implement this trait for their generated LCD text types so shared logic can write text without knowing the hardware backend.

Design intent:

This trait is intended for static dispatch on embedded targets.

Dimensions are const generics so geometry remains compile-time.

write_text accepts any string-like input via AsRef<str> .

## § Example: Write Text

This example writes text through a generic trait-bound helper.

use
device_envoy_core::lcd_text::LcdText;
fn
write_message<
const
W: usize,
const
H: usize>(lcd_text:
&
impl
LcdText<W, H>) {
    lcd_text.write_text(
"Hello from\ndevice-envoy!"
);
}

## Required Associated Constants §

#### const ADDRESS : u8

LCD I2C address.

## Provided Associated Constants §

#### const WIDTH : usize = W

Display width in characters.

#### const HEIGHT : usize = H

Display height in characters.

## Required Methods §

#### fn write_text (&self, text: impl AsRef < str >)

Write text to the display. See the LcdText trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl LcdText <16, 2> for LcdTextGenerated

Source

§

#### const ADDRESS : u8 = 0x27

Source

§

### impl LcdText <20, 4> for LcdTextGenerated20x4

Source

§

#### const ADDRESS : u8 = 0x3F

# Enum LedLevel Copy item path

Path: led/enum.LedLevel.html

device_envoy_rp

::

led

# Enum LedLevel Copy item path

pub enum LedLevel {
    On,
    Off,
}

Expand description

Logical LED level.

## Variants §

§

### On

LED on.

§

### Off

LED off.

## Trait Implementations §

§

### impl Clone for LedLevel

§

#### fn clone (&self) -> LedLevel

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for LedLevel

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for LedLevel

§

#### fn default () -> LedLevel

Returns the “default value” for a type.

Read more

§

### impl Hash for LedLevel

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Ord for LedLevel

§

#### fn cmp (&self, other: & LedLevel ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for LedLevel

§

#### fn eq (&self, other: & LedLevel ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for LedLevel

§

#### fn partial_cmp (&self, other: & LedLevel ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Copy for LedLevel

§

### impl Eq for LedLevel

§

### impl StructuralPartialEq for LedLevel

## Auto Trait Implementations §

§

### impl Freeze for LedLevel

§

### impl RefUnwindSafe for LedLevel

§

### impl Send for LedLevel

§

### impl Sync for LedLevel

§

### impl Unpin for LedLevel

§

### impl UnwindSafe for LedLevel

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Enum OnLevel Copy item path

Path: led/enum.OnLevel.html

device_envoy_rp

::

led

# Enum OnLevel Copy item path

pub enum OnLevel {
    High,
    Low,
}

Expand description

Which GPIO pin level turns the LED on.

## Variants §

§

### High

LED lights when the pin is driven high.

§

### Low

LED lights when the pin is driven low.

## Trait Implementations §

§

### impl Clone for OnLevel

§

#### fn clone (&self) -> OnLevel

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for OnLevel

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for OnLevel

§

#### fn default () -> OnLevel

Returns the “default value” for a type.

Read more

§

### impl Hash for OnLevel

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Ord for OnLevel

§

#### fn cmp (&self, other: & OnLevel ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for OnLevel

§

#### fn eq (&self, other: & OnLevel ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for OnLevel

§

#### fn partial_cmp (&self, other: & OnLevel ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Copy for OnLevel

§

### impl Eq for OnLevel

§

### impl StructuralPartialEq for OnLevel

## Auto Trait Implementations §

§

### impl Freeze for OnLevel

§

### impl RefUnwindSafe for OnLevel

§

### impl Send for OnLevel

§

### impl Sync for OnLevel

§

### impl Unpin for OnLevel

§

### impl UnwindSafe for OnLevel

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module led Copy item path

Path: led/index.html

device_envoy_rp

# Module led Copy item path

Source

Expand description

A device abstraction for a single digital LED with animation support.

Use the led! macro to generate one or more concrete LED device types.

See LedGenerated for a sample generated type.

## § Example

use
device_envoy_rp::{
Result
,
    led,
    led::{Led
as _
, LedLevel, OnLevel},
};
use
embassy_time::Duration;
led!
(
pub
LedOne { pin: PIN_1 });
led!
(
pub
LedTwo {
    pin: PIN_2,
    max_steps:
2
,
});
async fn
example(p: embassy_rp::Peripherals, spawner: embassy_executor::Spawner) ->
Result
<()> {
let
led_one = LedOne::new(p.PIN_1, OnLevel::High, spawner)
?
;
let
led_two = LedTwo::new(p.PIN_2, OnLevel::High, spawner)
?
;

    led_one.set_level(LedLevel::On);
    led_two.set_level(LedLevel::Off);
    embassy_time::Timer::after(Duration::from_millis(
250
)).
await
;

    led_one.animate([
        (LedLevel::On, Duration::from_millis(
200
)),
        (LedLevel::Off, Duration::from_millis(
200
)),
    ]);
    led_two.animate([
        (LedLevel::Off, Duration::from_millis(
150
)),
        (LedLevel::On, Duration::from_millis(
150
)),
    ]);

    pending().
await
}

## Modules §

led_ generated Module containing a sample struct type generated by the led! macro: LedGenerated .

## Macros §

led Macro to generate a single LED struct type (includes syntax details).

## Enums §

LedLevel Logical LED level. OnLevel Which GPIO pin level turns the LED on.

## Traits §

Led Platform-agnostic single LED device contract.

# Module led_ generated Copy item path

Path: led/led_generated/index.html

device_envoy_rp

::

led

# Module led_ generated Copy item path

Source

Expand description

Module containing a sample struct type generated by the led! macro: LedGenerated .

Auto-generated.

## Structs §

LedGenerated Sample struct type generated by the led! macro.

# Struct LedGenerated Copy item path

Path: led/led_generated/struct.LedGenerated.html

device_envoy_rp

::

led :: led_generated

# Struct LedGenerated Copy item path

Source

pub struct LedGenerated;

Expand description

Sample struct type generated by the led! macro.

This page exists to show constructor and trait methods in one place. For narrative examples, see the led module.

Auto-generated.

## Implementations §

Source

§

### impl LedGenerated

Source

#### pub const MAX_STEPS : usize = 32

Maximum number of animation steps for this generated LED type.

Source

#### pub fn new ( pin: Peri<'static, PIN_1>, on_level: OnLevel , spawner: Spawner, ) -> Result <Self>

Create a new LED instance of the struct type defined by led! .

See the led module docs for usage examples.

## Trait Implementations §

Source

§

### impl Led for LedGenerated

Source

§

#### fn set_level (&self, led_level: LedLevel )

Set the LED level immediately.

Read more

Source

§

#### fn animate <I>(&self, frames: I) where I: IntoIterator , I:: Item : Borrow <( LedLevel , Duration)>,

Play a looped animation sequence.

Read more

§

#### fn on (&self)

Turn the LED on.

Read more

§

#### fn off (&self)

Turn the LED off.

Read more

## Auto Trait Implementations §

§

### impl Freeze for LedGenerated

§

### impl RefUnwindSafe for LedGenerated

§

### impl Send for LedGenerated

§

### impl Sync for LedGenerated

§

### impl Unpin for LedGenerated

§

### impl UnwindSafe for LedGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# macro.led!.html

Path: led/macro.led!.html

Redirecting to macro.led.html ...

# Macro led Copy item path

Path: led/macro.led.html

device_envoy_rp

::

led

# Macro led Copy item path

Source

macro_rules! led {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate a single LED struct type (includes syntax details).

See the led module documentation for usage examples.

Syntax:

led! {
    [<visibility>] <Name> {
        pin: <pin_ident>,
        max_steps: <usize_expr>, // optional
    }
}

Required fields:

pin — GPIO pin resource type for this generated LED.

Optional fields:

max_steps — Maximum number of animation frames (default: 32).

max_steps = 0 disables animation storage; set_level() is still supported.

# Trait Led Copy item path

Path: led/trait.Led.html

device_envoy_rp

::

led

# Trait Led Copy item path

pub trait Led {
    // Required methods
    fn
set_level
(&self, led_level:
LedLevel
);
fn
animate
<I>(&self, frames: I)
where I:
IntoIterator
,
             <I as
IntoIterator
>::
Item
:
Borrow
<(
LedLevel
, Duration)>
;

    // Provided methods
    fn
on
(&self) { ... }
fn
off
(&self) { ... }
}

Expand description

Platform-agnostic single LED device contract.

This trait defines a minimal operation surface for a single digital LED:

Set the LED level immediately with Led::set_level .

Run a looped animation with Led::animate .

## § Example

use
device_envoy_core::led::{Led, LedLevel};
use
embassy_time::Duration;
async fn
run_led_example(led:
&
impl
Led) {
// Turn the LED on
led.set_level(LedLevel::On);
    embassy_time::Timer::after(Duration::from_secs(
1
)).
await
;
// Turn the LED off
led.set_level(LedLevel::Off);
    embassy_time::Timer::after(Duration::from_millis(
500
)).
await
;
// Play a blinking animation (looping: 200ms on, 200ms off)
led.animate([
        (LedLevel::On, Duration::from_millis(
200
)),
        (LedLevel::Off, Duration::from_millis(
200
)),
    ]);
}

## Required Methods §

#### fn set_level (&self, led_level: LedLevel )

Set the LED level immediately.

See the Led trait documentation for usage examples.

#### fn animate <I>(&self, frames: I) where I: IntoIterator , <I as IntoIterator >:: Item : Borrow <( LedLevel , Duration)>,

Play a looped animation sequence.

The duration type is embassy_time::Duration .

See the Led trait documentation for usage examples.

## Provided Methods §

#### fn on (&self)

Turn the LED on.

Equivalent to set_level(LedLevel::On) .

#### fn off (&self)

Turn the LED off.

Equivalent to set_level(LedLevel::Off) .

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Led for LedGenerated

# Enum Led2d Font Copy item path

Path: led2d/enum.Led2dFont.html

device_envoy_rp

::

led2d

# Enum Led2d Font Copy item path

pub enum Led2dFont {
Show 45 variants
Font3x4Trim,
    Font4x6,
    Font3x5Trim,
    Font5x7,
    Font4x6Trim,
    Font5x8,
    Font4x7Trim,
    Font6x9,
    Font5x8Trim,
    Font6x10,
    Font5x9Trim,
    Font6x12,
    Font5x11Trim,
    Font6x13,
    Font5x12Trim,
    Font6x13Bold,
    Font5x12TrimBold,
    Font6x13Italic,
    Font5x12TrimItalic,
    Font7x13,
    Font6x12Trim,
    Font7x13Bold,
    Font6x12TrimBold,
    Font7x13Italic,
    Font6x12TrimItalic,
    Font7x14,
    Font6x13Trim,
    Font7x14Bold,
    Font6x13TrimBold,
    Font8x13,
    Font7x12Trim,
    Font8x13Bold,
    Font7x12TrimBold,
    Font8x13Italic,
    Font7x12TrimItalic,
    Font9x15,
    Font8x14Trim,
    Font9x15Bold,
    Font8x14TrimBold,
    Font9x18,
    Font8x17Trim,
    Font9x18Bold,
    Font8x17TrimBold,
    Font10x20,
    Font9x19Trim,
}

Expand description

Fonts available for use with LED panel displays.

Fonts with Trim suffix remove blank spacing to pack text more tightly on small displays.

## Variants §

§

### Font3x4Trim

3x4 monospace font, trimmed (compact layout).

§

### Font4x6

4x6 monospace font.

§

### Font3x5Trim

3x5 monospace font, trimmed (compact layout).

§

### Font5x7

5x7 monospace font.

§

### Font4x6Trim

4x6 monospace font, trimmed (compact layout).

§

### Font5x8

5x8 monospace font.

§

### Font4x7Trim

4x7 monospace font, trimmed (compact layout).

§

### Font6x9

6x9 monospace font.

§

### Font5x8Trim

5x8 monospace font, trimmed (compact layout).

§

### Font6x10

6x10 monospace font.

§

### Font5x9Trim

5x9 monospace font, trimmed (compact layout).

§

### Font6x12

6x12 monospace font.

§

### Font5x11Trim

5x11 monospace font, trimmed (compact layout).

§

### Font6x13

6x13 monospace font.

§

### Font5x12Trim

5x12 monospace font, trimmed (compact layout).

§

### Font6x13Bold

6x13 bold monospace font.

§

### Font5x12TrimBold

5x12 bold monospace font, trimmed (compact layout).

§

### Font6x13Italic

6x13 italic monospace font.

§

### Font5x12TrimItalic

5x12 italic monospace font, trimmed (compact layout).

§

### Font7x13

7x13 monospace font.

§

### Font6x12Trim

6x12 monospace font, trimmed (compact layout).

§

### Font7x13Bold

7x13 bold monospace font.

§

### Font6x12TrimBold

6x12 bold monospace font, trimmed (compact layout).

§

### Font7x13Italic

7x13 italic monospace font.

§

### Font6x12TrimItalic

6x12 italic monospace font, trimmed (compact layout).

§

### Font7x14

7x14 monospace font.

§

### Font6x13Trim

6x13 monospace font, trimmed (compact layout).

§

### Font7x14Bold

7x14 bold monospace font.

§

### Font6x13TrimBold

6x13 bold monospace font, trimmed (compact layout).

§

### Font8x13

8x13 monospace font.

§

### Font7x12Trim

7x12 monospace font, trimmed (compact layout).

§

### Font8x13Bold

8x13 bold monospace font.

§

### Font7x12TrimBold

7x12 bold monospace font, trimmed (compact layout).

§

### Font8x13Italic

8x13 italic monospace font.

§

### Font7x12TrimItalic

7x12 italic monospace font, trimmed (compact layout).

§

### Font9x15

9x15 monospace font.

§

### Font8x14Trim

8x14 monospace font, trimmed (compact layout).

§

### Font9x15Bold

9x15 bold monospace font.

§

### Font8x14TrimBold

8x14 bold monospace font, trimmed (compact layout).

§

### Font9x18

9x18 monospace font.

§

### Font8x17Trim

8x17 monospace font, trimmed (compact layout).

§

### Font9x18Bold

9x18 bold monospace font.

§

### Font8x17TrimBold

8x17 bold monospace font, trimmed (compact layout).

§

### Font10x20

10x20 monospace font.

§

### Font9x19Trim

9x19 monospace font, trimmed (compact layout).

## Implementations §

§

### impl Led2dFont

#### pub fn to_font (self) -> MonoFont<'static>

Return the MonoFont for this variant.

#### pub const fn spacing_reduction (self) -> ( i32 , i32 )

Return spacing reduction for trimmed variants as (width_reduction, height_reduction) .

## Trait Implementations §

§

### impl Clone for Led2dFont

§

#### fn clone (&self) -> Led2dFont

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Led2dFont

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Copy for Led2dFont

## Auto Trait Implementations §

§

### impl Freeze for Led2dFont

§

### impl RefUnwindSafe for Led2dFont

§

### impl Send for Led2dFont

§

### impl Sync for Led2dFont

§

### impl Unpin for Led2dFont

§

### impl UnwindSafe for Led2dFont

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Function bit_ matrix3x4_ font Copy item path

Path: led2d/fn.bit_matrix3x4_font.html

device_envoy_rp

::

led2d

# Function bit_ matrix3x4_ font Copy item path

pub fn bit_matrix3x4_font() -> MonoFont<'static>

Expand description

Monospace 3x4 font matching the internal BIT_MATRIX3X4 bitmap data.

# Function render_ text_ to_ frame Copy item path

Path: led2d/fn.render_text_to_frame.html

device_envoy_rp

::

led2d

# Function render_ text_ to_ frame Copy item path

pub fn render_text_to_frame<const W:
usize
, const H:
usize
>(
    frame: &mut
Frame2d
<W, H>,
    font: &MonoFont<'static>,
    text: &
str
,
    colors: &[Rgb<
u8
>],
    spacing_reduction: (
i32
,
i32
),
)

Expand description

Render text into a frame using the provided font.

Text flows left-to-right within the frame width; a \n character advances to the next row. Characters that exceed the frame width are skipped (no wrapping). Colors cycle over the colors slice (one color per character); an empty slice defaults to white.

spacing_reduction is a (width_reduction, height_reduction) pair in pixels used by the trimmed Led2dFont variants to pack characters more tightly.

# Module led2d Copy item path

Path: led2d/index.html

device_envoy_rp

# Module led2d Copy item path

Source

Expand description

A device abstraction for rectangular NeoPixel-style (WS2812) LED panel displays. For 1-dimensional LED strips, see the led_strip module.

This page provides the primary documentation and examples for programming LED panels. The device abstraction supports text, graphics, and animation.

After reading the examples below, see also:

led2d! — Macro to generate an LED-panel struct type (includes syntax details).

Led2d — Core trait that defines the LED panel API surface.

Led2dGenerated — Sample generated panel type showing the constructor path.

LedLayout — Compile-time description of panel geometry and wiring, including dimensions (with examples)

Frame2d — 2D pixel array used for general graphics (includes examples)

led_strips! — Alternative macro to share a PIO resource with other panels or LED strips (includes examples)

## § Example: Write Text

In this example, we render text on a 12×4 panel. Here, the generated struct type is named Led12x4 .

use
device_envoy_rp::{
Result
, led2d, led2d::layout::LedLayout, led2d::Led2dFont, led_strip::colors, led2d::Led2d
as _
};
// Tells us how the LED strip is wired up in the panel
// in this case, a common snake-like pattern.
const
LED_LAYOUT_12X4: LedLayout<
48
,
12
,
4
> = LedLayout::serpentine_column_major();
// Generate a type named `Led12x4`.
led2d!
{
    Led12x4 {
        pin: PIN_3,
// GPIO pin for LED data signal
led_layout: LED_LAYOUT_12X4,
// LED layout mapping (defines dimensions)
font: Led2dFont::Font3x4Trim,
// Font variant
}
}
async fn
example(spawner: Spawner) ->
Result
<Infallible> {
let
p = init(Default::default());
// Create a device abstraction for the LED panel.
    // Behind the scenes, this creates a channel & background task to manage the display.
let
led12x4 = Led12x4::new(p.PIN_3, p.PIO0, p.DMA_CH0, spawner)
?
;
// Write text to the display with per-character colors.
let
colors = [colors::CYAN, colors::RED, colors::YELLOW];
// Each character takes the next color; when we run out, we start over.
led12x4.write_text(
"Rust"
,
&
colors);

    pending().
await
// run forever
}

## § Example: Animated Text on a Rotated Panel

This example animates text on a rotated 12×8 panel built from two stacked 12×4 panels.

use
device_envoy_rp::{
Result
, led2d, led2d::layout::LedLayout, led2d::Frame2d, led2d::Led2dFont, led_strip::{Current, Gamma, colors}, led2d::Led2d
as _
};
use
embassy_time::Duration;
// Our panel is two 12x4 panels stacked vertically and then rotated clockwise.
const
LED_LAYOUT_12X4: LedLayout<
48
,
12
,
4
> = LedLayout::serpentine_column_major();
const
LED_LAYOUT_12X8: LedLayout<
96
,
12
,
8
> = LED_LAYOUT_12X4.combine_v(LED_LAYOUT_12X4);
const
LED_LAYOUT_12X8_ROTATED: LedLayout<
96
,
8
,
12
> = LED_LAYOUT_12X8.rotate_cw();
// Generate a type named `Led12x8Animated`.
led2d!
{
pub
(
self
) Led12x8Animated {
// Can provide a visibility modifier
pin: PIN_4,
// GPIO pin for LED data signal
led_layout: LED_LAYOUT_12X8_ROTATED,
// Two 12×4 panels stacked and rotated
font: Led2dFont::Font4x6Trim,
// Use a 4x6 pixel font without the usual 1 pixel padding
pio: PIO1,
// PIO resource, default is PIO0
dma: DMA_CH1,
// DMA resource, default is DMA_CH0
max_current: Current::Milliamps(
300
),
// Power budget, default is 250 mA.
gamma: Gamma::Linear,
// Color correction curve, default is Gamma::Srgb
max_frames:
2
,
// maximum animation frames, default is 16
}
}
async fn
example(spawner: Spawner) ->
Result
<Infallible> {
let
p = init(Default::default());
// Create a device abstraction for the rotated LED panel.
let
led_12x8_animated = Led12x8Animated::new(p.PIN_4, p.PIO1, p.DMA_CH1, spawner)
?
;
// Write "Go" into an in-memory frame buffer.
let
mut
frame_0 = Frame2d::new();
// Empty text colors array defaults to white.
led_12x8_animated.write_text_to_frame(
"Go"
,
&
[],
&mut
frame_0);
// Write "Go" into a second frame buffer with custom colors and on the 2nd line.
let
mut
frame_1 = Frame2d::new();
// "\n" starts a new line. Text does not wrap but rather clips.
led_12x8_animated.write_text_to_frame(
"\nGo"
,
&
[colors::HOT_PINK, colors::LIME],
&mut
frame_1,
    );
// Animate between the two frames indefinitely.
let
frame_duration = Duration::from_secs(
1
);
    led_12x8_animated
        .animate([(frame_0, frame_duration), (frame_1, frame_duration)]);

    pending().
await
// run forever
}

## Modules §

layout Layout mapping types for 2D LED panels. See the led2d module documentation for details. led2d_ generated Module containing Led2dGenerated , the sample struct type generated by the led2d! macro.

## Macros §

led2d Macro to generate an LED-panel struct type (includes syntax details). See Led2d for the shared API.

## Structs §

Frame2d 2D pixel array used for general graphics on LED panels. LedLayout Compile-time description of panel geometry and wiring, including dimensions (with examples). Point 2D point. Size 2D size.

## Enums §

Led2d Font Fonts available for use with LED panel displays.

## Traits §

Led2d Platform-agnostic LED panel device contract.

## Functions §

bit_ matrix3x4_ font Monospace 3x4 font matching the internal BIT_MATRIX3X4 bitmap data. render_ text_ to_ frame Render text into a frame using the provided font.

# Module layout Copy item path

Path: led2d/layout/index.html

device_envoy_rp

::

led2d

# Module layout Copy item path

Source

Expand description

Layout mapping types for 2D LED panels. See the led2d module documentation for details.

## Structs §

LedLayout Compile-time description of panel geometry and wiring, including dimensions (with examples).

# Struct LedLayout Copy item path

Path: led2d/layout/struct.LedLayout.html

device_envoy_rp

::

led2d :: layout

# Struct LedLayout Copy item path

pub struct LedLayout<const N:
usize
, const W:
usize
, const H:
usize
> {
/* private fields */
}

Expand description

Compile-time description of panel geometry and wiring, including dimensions (with examples).

LedLayout defines how a rectangular (x, y) panel of LEDs maps to the linear wiring order of LEDs on a NeoPixel-style (WS2812) panel.

For examples of LedLayout in use, see the led2d module, Frame2d , and the example below.

What LedLayout does:

Lets you describe panel wiring once

Enables drawing text, graphics, and animations in (x, y) space

Hides LED strip order from rendering code

Coordinates use a screen-style convention:

(0, 0) is the top-left corner

x increases to the right

y increases downward

Most users should start with one of the constructors below and then apply transforms ( rotate_cw , flip_h , combine_v , etc.) as needed.

### § Constructing layouts

Prefer the built-in constructors when possible:

serpentine_row_major

serpentine_column_major

linear_h / linear_v

For unusual wiring, you can construct a layout directly with LedLayout::new by listing (x, y) for each LED in the order the strip is wired.

The example below shows both construction methods. Also, the documentation for every constructor and method includes illustrations of use.

### § Transforming layouts

You can adapt a layout without rewriting it:

rotate: rotate_cw , rotate_ccw , rotate_180

flip: flip_h , flip_v

combine: combine_h , combine_v (join two layouts into a larger one)

### § Validation

Layouts are validated at compile time :

coordinates must be in-bounds

every (x, y) cell must appear exactly once

If you want the final mapping, use index_to_xy .

## § Example

Rotate a serpentine-wired 3×2 panel into a 2×3 layout and verify the result at compile time:

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
2
,
3
> = LedLayout::serpentine_column_major().rotate_cw();
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
1
,
0
), (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
2
), (
0
,
2
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));
// Compile-time assert

Serpentine 3×2 rotated to 2×3:

  Before:              After:
    LED0  LED3  LED4     LED1  LED0
    LED1  LED2  LED5     LED2  LED3
                         LED5  LED4

## Implementations §

§

### impl<const N: usize , const W: usize , const H: usize > LedLayout <N, W, H>

#### pub const fn index_to_xy (&self) -> &[( u16 , u16 ); N ]

Return the array mapping LED wiring order to (x, y) coordinates.

#### pub const fn width (&self) -> usize

The width of the layout.

#### pub const fn height (&self) -> usize

The height of the layout.

#### pub const fn len (&self) -> usize

Total LEDs in this layout (width × height).

#### pub const fn xy_to_index (&self) -> [ u16 ; N ]

Return the inverse mapping: (x, y) coordinates to LED wiring index.

The returned array is indexed by y * W + x and contains the LED wiring index for each pixel position. This is the inverse of index_to_xy .

#### pub const fn equals (&self, other: & LedLayout <N, W, H>) -> bool

Const equality helper for doctests/examples.

use
device_envoy_core::led2d::layout::LedLayout;
const
LINEAR: LedLayout<
4
,
4
,
1
> = LedLayout::linear_h();
const
ROTATED: LedLayout<
4
,
4
,
1
> = LedLayout::linear_v().rotate_cw();
const _
: () =
assert!
(LINEAR.equals(
&
LINEAR));
// assert equal
const _
: () =
assert!
(!LINEAR.equals(
&
ROTATED));
// assert not equal

LINEAR:  LED0  LED1  LED2  LED3
ROTATED: LED3  LED2  LED1  LED0

#### pub const fn new (map: [( u16 , u16 ); N ]) -> LedLayout <N, W, H>

Construct a LedLayout by explicitly specifying the wiring order.

Use this constructor when your panel wiring does not match one of the built-in patterns (linear, serpentine, etc.). You provide the (x, y) coordinate for each LED in strip order , and LedLayout derives the panel geometry from that mapping.

This constructor is const and is intended to be used in a const definition, so layout errors are caught at compile time , not at runtime.

use
device_envoy_core::led2d::layout::LedLayout;
// 3×2 panel (landscape, W×H)
const
MAP: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
0
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
1
,
1
), (
0
,
1
)]);
// Rotate to portrait (CW)
const
ROTATED: LedLayout<
6
,
2
,
3
> = MAP.rotate_cw();
// Expected: 2×3 panel (W×H)
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
1
,
0
), (
1
,
1
), (
1
,
2
), (
0
,
2
), (
0
,
1
), (
0
,
0
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

3×2 input (col,row by LED index):
  LED0  LED1  LED2
  LED5  LED4  LED3

After rotate to 2×3:
  LED1  LED0
  LED2  LED3
  LED5  LED4

#### pub const fn linear_h () -> LedLayout <N, W, H>

Linear row-major mapping for a single-row strip (cols increase left-to-right).

use
device_envoy_core::led2d::layout::LedLayout;
const
LINEAR: LedLayout<
6
,
6
,
1
> = LedLayout::linear_h();
const
EXPECTED: LedLayout<
6
,
6
,
1
> =
    LedLayout::new([(
0
,
0
), (
1
,
0
), (
2
,
0
), (
3
,
0
), (
4
,
0
), (
5
,
0
)]);
const _
: () =
assert!
(LINEAR.equals(
&
EXPECTED));

6×1 strip maps to single row:
  LED0  LED1  LED2  LED3  LED4  LED5

#### pub const fn linear_v () -> LedLayout <N, W, H>

Linear column-major mapping for a single-column strip (rows increase top-to-bottom).

use
device_envoy_core::led2d::layout::LedLayout;
const
LINEAR: LedLayout<
6
,
1
,
6
> = LedLayout::linear_v();
const
EXPECTED: LedLayout<
6
,
1
,
6
> =
    LedLayout::new([(
0
,
0
), (
0
,
1
), (
0
,
2
), (
0
,
3
), (
0
,
4
), (
0
,
5
)]);
const _
: () =
assert!
(LINEAR.equals(
&
EXPECTED));

1×6 strip maps to single column:
  LED0
  LED1
  LED2
  LED3
  LED4
  LED5

#### pub const fn serpentine_column_major () -> LedLayout <N, W, H>

Serpentine column-major mapping returned as a checked LedLayout .

use
device_envoy_core::led2d::layout::LedLayout;
const
MAP: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
0
), (
2
,
0
), (
2
,
1
)]);
const _
: () =
assert!
(MAP.equals(
&
EXPECTED));

Strip snakes down columns (3×2 example):
  LED0  LED3  LED4
  LED1  LED2  LED5

#### pub const fn serpentine_row_major () -> LedLayout <N, W, H>

Serpentine row-major mapping (alternating left-to-right and right-to-left across rows).

use
device_envoy_core::led2d::layout::LedLayout;
const
MAP: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_row_major();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
0
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
1
,
1
), (
0
,
1
)]);
const _
: () =
assert!
(MAP.equals(
&
EXPECTED));

Strip snakes across rows (3×2 example):
  LED0  LED1  LED2
  LED5  LED4  LED3

#### pub const fn rotate_cw (self) -> LedLayout <N, H, W>

Rotate 90° clockwise (dims swap).

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
2
,
3
> = LedLayout::serpentine_column_major().rotate_cw();
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
1
,
0
), (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
2
), (
0
,
2
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

Before (3×2 serpentine): After (2×3):
  LED0  LED3  LED4        LED1  LED0
  LED1  LED2  LED5        LED2  LED3
                          LED5  LED4

#### pub const fn flip_h (self) -> LedLayout <N, W, H>

Flip horizontally (mirror columns).

use
device_envoy_core::led2d::layout::LedLayout;
const
FLIPPED: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major().flip_h();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
2
,
0
), (
2
,
1
), (
1
,
1
), (
1
,
0
), (
0
,
0
), (
0
,
1
)]);
const _
: () =
assert!
(FLIPPED.equals(
&
EXPECTED));

Before (serpentine): After:
  LED0  LED3  LED4      LED4  LED3  LED0
  LED1  LED2  LED5      LED5  LED2  LED1

#### pub const fn rotate_180 (self) -> LedLayout <N, W, H>

Rotate 180° derived from rotate_cw.

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major().rotate_180();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
2
,
1
), (
2
,
0
), (
1
,
0
), (
1
,
1
), (
0
,
1
), (
0
,
0
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

Before (3×2 serpentine): After 180°:
  LED0  LED3  LED4        LED5  LED2  LED1
  LED1  LED2  LED5        LED4  LED3  LED0

#### pub const fn rotate_ccw (self) -> LedLayout <N, H, W>

Rotate 90° counter-clockwise derived from rotate_cw.

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
2
,
3
> = LedLayout::serpentine_column_major().rotate_ccw();
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
0
,
2
), (
1
,
2
), (
1
,
1
), (
0
,
1
), (
0
,
0
), (
1
,
0
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

Before (3×2 serpentine): After (2×3):
  LED0  LED3  LED4        LED4  LED5
  LED1  LED2  LED5        LED3  LED2
                          LED0  LED1

#### pub const fn flip_v (self) -> LedLayout <N, W, H>

Flip vertically derived from rotation + horizontal flip.

use
device_envoy_core::led2d::layout::LedLayout;
const
FLIPPED: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major().flip_v();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
1
), (
0
,
0
), (
1
,
0
), (
1
,
1
), (
2
,
1
), (
2
,
0
)]);
const _
: () =
assert!
(FLIPPED.equals(
&
EXPECTED));

Before (serpentine): After:
  LED0  LED3  LED4      LED1  LED2  LED5
  LED1  LED2  LED5      LED0  LED3  LED4

#### pub const fn combine_h <const N2: usize , const OUT_N: usize , const W2: usize , const OUT_W: usize >( self, right: LedLayout <N2, W2, H>, ) -> LedLayout <OUT_N, OUT_W, H>

Concatenate horizontally with another mapping sharing the same rows.

use
device_envoy_core::led2d::layout::LedLayout;
const
LED_LAYOUT: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major();
const
COMBINED: LedLayout<
12
,
6
,
2
> = LED_LAYOUT.combine_h::<
6
,
12
,
3
,
6
>(LED_LAYOUT);
const
EXPECTED: LedLayout<
12
,
6
,
2
> = LedLayout::new([
    (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
3
,
0
), (
3
,
1
), (
4
,
1
),
    (
4
,
0
), (
5
,
0
), (
5
,
1
),
]);
const _
: () =
assert!
(COMBINED.equals(
&
EXPECTED));

Left serpentine (3×2):    Right serpentine (3×2):
  0  3  4                   6  9 10
  1  2  5                   7  8 11

Combined (6×2):
  0  3  4  6  9 10
  1  2  5  7  8 11

#### pub const fn combine_v <const N2: usize , const OUT_N: usize , const H2: usize , const OUT_H: usize >( self, bottom: LedLayout <N2, W, H2>, ) -> LedLayout <OUT_N, W, OUT_H>

Concatenate vertically with another mapping sharing the same columns.

use
device_envoy_core::led2d::layout::LedLayout;
const
LED_LAYOUT: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major();
const
COMBINED: LedLayout<
12
,
3
,
4
> = LED_LAYOUT.combine_v::<
6
,
12
,
2
,
4
>(LED_LAYOUT);
const
EXPECTED: LedLayout<
12
,
3
,
4
> = LedLayout::new([
    (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
0
,
2
), (
0
,
3
), (
1
,
3
),
    (
1
,
2
), (
2
,
2
), (
2
,
3
),
]);
const _
: () =
assert!
(COMBINED.equals(
&
EXPECTED));

Top serpentine (3×2):    Bottom serpentine (3×2):
  0  3  4                   6  9 10
  1  2  5                   7  8 11

Combined (3×4):
  0  3  4
  1  2  5
  6  9 10
  7  8 11

## Trait Implementations §

§

### impl<const N: usize , const W: usize , const H: usize > Clone for LedLayout <N, W, H>

§

#### fn clone (&self) -> LedLayout <N, W, H>

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl<const N: usize , const W: usize , const H: usize > Debug for LedLayout <N, W, H>

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl<const N: usize , const W: usize , const H: usize > PartialEq for LedLayout <N, W, H>

§

#### fn eq (&self, other: & LedLayout <N, W, H>) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl<const N: usize , const W: usize , const H: usize > Copy for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Eq for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > StructuralPartialEq for LedLayout <N, W, H>

## Auto Trait Implementations §

§

### impl<const N: usize , const W: usize , const H: usize > Freeze for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > RefUnwindSafe for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Send for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Sync for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Unpin for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > UnwindSafe for LedLayout <N, W, H>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module led2d_ generated Copy item path

Path: led2d/led2d_generated/index.html

device_envoy_rp

::

led2d

# Module led2d_ generated Copy item path

Source

Expand description

Module containing Led2dGenerated , the sample struct type generated by the led2d! macro.

Auto-generated.

## Structs §

Led2d Generated Sample struct type generated by the led2d! macro.

# Struct Led2d Generated Copy item path

Path: led2d/led2d_generated/struct.Led2dGenerated.html

device_envoy_rp

::

led2d :: led2d_generated

# Struct Led2d Generated Copy item path

Source

pub struct Led2dGenerated;

Expand description

Sample struct type generated by the led2d! macro.

This page exists to show constructor, constants, and trait methods in one place. For narrative examples, see the led2d module.

Auto-generated.

## Implementations §

Source

§

### impl Led2dGenerated

Source

#### pub const MAX_FRAMES : usize = 16

Maximum number of animation frames.

Source

#### pub const MAX_BRIGHTNESS : u8 = 22

Maximum brightness level after current limiting.

Source

#### pub const FONT : Led2dFont = Led2dFont::Font3x4Trim

Default font used by text helpers.

Source

#### pub const WIDTH : usize = 12

Panel width in pixels.

Source

#### pub const HEIGHT : usize = 4

Panel height in pixels.

Source

#### pub const LEN : usize = 48

Total LED count ( WIDTH * HEIGHT ).

Source

#### pub const SIZE : Size = Frame2d<12, 4>::SIZE

Panel dimensions.

Source

#### pub const TOP_LEFT : Point = Frame2d<12, 4>::TOP_LEFT

Top-left corner coordinate.

Source

#### pub const TOP_RIGHT : Point = Frame2d<12, 4>::TOP_RIGHT

Top-right corner coordinate.

Source

#### pub const BOTTOM_LEFT : Point = Frame2d<12, 4>::BOTTOM_LEFT

Bottom-left corner coordinate.

Source

#### pub const BOTTOM_RIGHT : Point = Frame2d<12, 4>::BOTTOM_RIGHT

Bottom-right corner coordinate.

Source

#### pub fn new ( pin: Peri<'static, PIN_3>, pio: Peri<'static, PIO0>, dma: Peri<'static, DMA_CH0>, spawner: Spawner, ) -> Result <Self>

Create a new LED panel instance of the struct type defined by led2d! .

See the led2d module docs for usage examples.

## Trait Implementations §

Source

§

### impl Led2d <12, 4> for Led2dGenerated

Source

§

#### const WIDTH : usize = Self::WIDTH

The width of the panel.

Source

§

#### const HEIGHT : usize = Self::HEIGHT

The height of the panel.

Source

§

#### const LEN : usize = Self::LEN

Total LEDs in this panel (width × height).

Source

§

#### const SIZE : Size = Self::SIZE

Panel dimensions as a

Size

.

Read more

Source

§

#### const TOP_LEFT : Point = Self::TOP_LEFT

Top-left corner coordinate as a

Point

.

Read more

Source

§

#### const TOP_RIGHT : Point = Self::TOP_RIGHT

Top-right corner coordinate as a

Point

.

Read more

Source

§

#### const BOTTOM_LEFT : Point = Self::BOTTOM_LEFT

Bottom-left corner coordinate as a

Point

.

Read more

Source

§

#### const BOTTOM_RIGHT : Point = Self::BOTTOM_RIGHT

Bottom-right corner coordinate as a

Point

.

Read more

Source

§

#### const MAX_FRAMES : usize = Self::MAX_FRAMES

Maximum number of animation frames allowed.

Read more

Source

§

#### const MAX_BRIGHTNESS : u8 = Self::MAX_BRIGHTNESS

Maximum brightness level, automatically limited by the power budget.

Read more

Source

§

#### const FONT : Led2dFont = Self::FONT

The font used by default text helpers.

Read more

Source

§

#### fn write_frame (&self, frame2d: Frame2d <12, 4>)

Write a frame to the LED panel.

Read more

Source

§

#### fn animate <I>(&self, frames: I) where I: IntoIterator , I:: Item : Borrow <( Frame2d <12, 4>, Duration)>,

Animate frames on the LED panel.

Read more

§

#### fn write_text_to_frame ( &self, text: & str , colors: &[Rgb< u8 >], frame: &mut Frame2d <W, H>, )

Write text into a frame.

Read more

§

#### fn write_text (&self, text: & str , colors: &[Rgb< u8 >])

Write text to the LED panel.

Read more

## Auto Trait Implementations §

§

### impl Freeze for Led2dGenerated

§

### impl RefUnwindSafe for Led2dGenerated

§

### impl Send for Led2dGenerated

§

### impl Sync for Led2dGenerated

§

### impl Unpin for Led2dGenerated

§

### impl UnwindSafe for Led2dGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# macro.led2d!.html

Path: led2d/macro.led2d!.html

Redirecting to macro.led2d.html ...

# Macro led2d Copy item path

Path: led2d/macro.led2d.html

device_envoy_rp

::

led2d

# Macro led2d Copy item path

Source

macro_rules! led2d {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate an LED-panel struct type (includes syntax details). See Led2d for the shared API.

See the led2d module for usage examples.

Syntax:

led2d! {
    [<visibility>] <Name> {
        pin: <pin_ident>,
        led_layout: <LedLayout_expr>,
        font: <Led2dFont_expr>,
        pio: <pio_ident>,               // optional
        dma: <dma_ident>,               // optional
        max_current: <Current_expr>,    // optional
        gamma: <Gamma_expr>,            // optional
        max_frames: <usize_expr>,       // optional
    }
}

## § Fields

Required fields:

pin — GPIO pin for LED data

led_layout — LED strip physical layout (see LedLayout ); this defines the panel size

font — Built-in font variant (see Led2dFont ), e.g. Led2dFont::Font4x6Trim . Bring Led2dFont into scope or use a full path like device_envoy_rp::led2d::Led2dFont::Font4x6Trim .

The led_layout value must be a const so its dimensions can be derived at compile time.

Optional fields:

pio — PIO resource to use (default: PIO0 )

dma — DMA channel (default: DMA_CH0 )

max_current — Electrical current budget (default: 250 mA)

gamma — Color curve (default: Gamma::Srgb )

max_frames — Maximum number of animation frames for the generated strip (default: 16 frames)

max_frames = 0 disables animation and allocates no frame storage; write_frame() is still supported.

## § Current Limiting

The max_current field automatically scales brightness to stay within your electrical current budget.

Each WS2812 LED is assumed to draw 60 mA at full brightness. For example:

16 LEDs × 60 mA = 960 mA at full brightness

With max_current: Current::Milliamps(1000) , all LEDs fit at 100% brightness

With the default electrical current limit (250 mA), the generated MAX_BRIGHTNESS limits LEDs to ~26% brightness

The electrical current limit is compiled into a lookup table at device initialization, so it has no per-frame runtime cost.

Powering LEDs from a board 5 V pin: On Pico, this is pin 40 ( VBUS ). On ESP32-C6 D6 and ESP32-S3 boards, this is usually a header pin labeled 5V or VBUS . These rails are typically USB pass-through and have practical current limits, so avoid heavy loads through the board itself. Small LED panels (a few hundred mA) are usually fine with a solid USB supply; for larger loads (around 1 A+), use a separate 5 V supply and share ground with the board. If your board labels the pin 5Vin , treat it as a power-input rail (not a general-purpose 5 V output for peripherals).

## § Color Correction (Gamma)

The gamma field applies a color response curve to make colors look more natural:

Gamma::Linear — No correction (raw values)

Gamma::Srgb — Perceptual sRGB semantics (default; preserves named color constants)

Gamma::SmartLeds — smart_leds::gamma() compatibility (2.8)

The gamma curve is compiled into a lookup table at device initialization, so it has no per-frame runtime cost.

## § Related Macros

led_strips! — Alternative macro to share a PIO resource with other panels or LED strips (includes examples)

led_strip! — For 1-dimensional LED strips

# Struct Frame2d Copy item path

Path: led2d/struct.Frame2d.html

device_envoy_rp

::

led2d

# Struct Frame2d Copy item path

pub struct Frame2d<const W:
usize
, const H:
usize
>(pub [[Rgb<
u8
>;
W
];
H
]);

Expand description

2D pixel array used for general graphics on LED panels.

Coordinates are (x, y) with (0, 0) at the top-left. The x-axis increases to the right, and the y-axis increases downward.

Set pixels using tuple indexing: frame[(x, y)] = colors::RED; .

For shapes, lines, and text rendering, use the embedded-graphics crate.

### § Indexing and storage

Frame2d supports both:

(x, y) tuple indexing: frame[(x, y)]

Row-major array indexing: frame[y][x]

Tuple indexing matches display coordinates. Array indexing matches the underlying storage.

## § Example: Draw pixels both directly and with embedded-graphics

use
device_envoy_core::{led2d::Frame2d, led_strip::ToRgb888};
use
embedded_graphics::{
    prelude::
*
,
    primitives::{Circle, PrimitiveStyle, Rectangle},
};
use
smart_leds::colors;
type
Frame = Frame2d<
12
,
8
>;
/// Calculate the top-left corner position to center a shape within a bounding box.
const fn
centered_top_left(width: usize, height: usize, size: usize) -> Point {
assert!
(size <= width);
assert!
(size <= height);
    Point::new(((width - size) /
2
)
as
i32, ((height - size) /
2
)
as
i32)
}
// Create a frame to draw on. This is just an in-memory 2D pixel buffer.
let
mut
frame = Frame::new();
// Use the embedded-graphics crate to draw a red rectangle border around the edge of the frame.
// We use `to_rgb888()` to convert from smart-leds RGB8 to embedded-graphics Rgb888.
Rectangle::new(Frame::TOP_LEFT, Frame::SIZE)
    .into_styled(PrimitiveStyle::with_stroke(colors::RED.to_rgb888(),
1
))
    .draw(
&mut
frame)
?
;
// Direct pixel access: set the upper-left LED pixel (x = 0, y = 0).
// Frame2d stores LED colors directly, so we write an LED color here.
frame[(
0
,
0
)] = colors::CYAN;
// Use the embedded-graphics crate to draw a green circle centered in the frame.
const
DIAMETER: u32 =
6
;
const
CIRCLE_TOP_LEFT: Point = centered_top_left(Frame::WIDTH, Frame::HEIGHT, DIAMETER
as
usize);
Circle::new(CIRCLE_TOP_LEFT, DIAMETER)
    .into_styled(PrimitiveStyle::with_stroke(colors::LIME.to_rgb888(),
1
))
    .draw(
&mut
frame)
?
;

## Tuple Fields §

§ 0: [[Rgb< u8 >; W ]; H ]

## Implementations §

§

### impl<const W: usize , const H: usize > Frame2d <W, H>

#### pub const WIDTH : usize = W

The width of the frame.

#### pub const HEIGHT : usize = H

The height of the frame.

#### pub const LEN : usize

Total pixels in this frame (width × height).

#### pub const SIZE : Size

Frame dimensions as a Size .

For embedded-graphics drawing operations.

#### pub const TOP_LEFT : Point

Top-left corner coordinate as a Point .

For embedded-graphics drawing operations.

#### pub const TOP_RIGHT : Point

Top-right corner coordinate as a Point .

For embedded-graphics drawing operations.

#### pub const BOTTOM_LEFT : Point

Bottom-left corner coordinate as a Point .

For embedded-graphics drawing operations.

#### pub const BOTTOM_RIGHT : Point

Bottom-right corner coordinate as a Point .

For embedded-graphics drawing operations.

#### pub const fn new () -> Frame2d <W, H>

Create a new blank (all black) frame.

#### pub const fn filled (color: Rgb< u8 >) -> Frame2d <W, H>

Create a frame filled with a single color.

## Trait Implementations §

§

### impl<const W: usize , const H: usize > Clone for Frame2d <W, H>

§

#### fn clone (&self) -> Frame2d <W, H>

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl<const W: usize , const H: usize > Debug for Frame2d <W, H>

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl<const W: usize , const H: usize > Default for Frame2d <W, H>

§

#### fn default () -> Frame2d <W, H>

Returns the “default value” for a type.

Read more

§

### impl<const W: usize , const H: usize > Deref for Frame2d <W, H>

§

#### type Target = [[Rgb< u8 >; W ]; H ]

The resulting type after dereferencing.

§

#### fn deref (&self) -> &< Frame2d <W, H> as Deref >:: Target

Dereferences the value.

§

### impl<const W: usize , const H: usize > DerefMut for Frame2d <W, H>

§

#### fn deref_mut (&mut self) -> &mut < Frame2d <W, H> as Deref >:: Target

Mutably dereferences the value.

§

### impl<const W: usize , const H: usize > DrawTarget for Frame2d <W, H>

§

#### type Color = Rgb888

The pixel color type the targetted display supports.

§

#### type Error = Infallible

Error type to return when a drawing operation fails.

Read more

§

#### fn draw_iter <I>( &mut self, pixels: I, ) -> Result < () , < Frame2d <W, H> as DrawTarget>::Error> where I: IntoIterator <Item = Pixel<< Frame2d <W, H> as DrawTarget>::Color>>,

Draw individual pixels to the display without a defined order.

Read more

§

#### fn fill_contiguous <I>( &mut self, area: &Rectangle, colors: I, ) -> Result < () , Self::Error> where I: IntoIterator <Item = Self::Color>,

Fill a given area with an iterator providing a contiguous stream of pixel colors.

Read more

§

#### fn fill_solid ( &mut self, area: &Rectangle, color: Self::Color, ) -> Result < () , Self::Error>

Fill a given area with a solid color.

Read more

§

#### fn clear (&mut self, color: Self::Color) -> Result < () , Self::Error>

Fill the entire display with a solid color.

Read more

§

### impl<const W: usize , const H: usize > From <[[Rgb< u8 >; W ]; H ]> for Frame2d <W, H>

§

#### fn from (array: [[Rgb< u8 >; W ]; H ]) -> Frame2d <W, H>

Converts to this type from the input type.

§

### impl<const W: usize , const H: usize > From < Frame2d <W, H>> for [[Rgb< u8 >; W ]; H ]

§

#### fn from (frame: Frame2d <W, H>) -> [[Rgb< u8 >; W ]; H ]

Converts to this type from the input type.

§

### impl<const W: usize , const H: usize > Index <( usize , usize )> for Frame2d <W, H>

§

#### type Output = Rgb< u8 >

The returned type after indexing.

§

#### fn index ( &self, _: ( usize , usize ), ) -> &< Frame2d <W, H> as Index <( usize , usize )>>:: Output

Performs the indexing (

container[index]

) operation.

Read more

§

### impl<const W: usize , const H: usize > IndexMut <( usize , usize )> for Frame2d <W, H>

§

#### fn index_mut ( &mut self, _: ( usize , usize ), ) -> &mut < Frame2d <W, H> as Index <( usize , usize )>>:: Output

Performs the mutable indexing (

container[index]

) operation.

Read more

§

### impl<const W: usize , const H: usize > OriginDimensions for Frame2d <W, H>

§

#### fn size (&self) -> Size

Returns the size of the bounding box.

§

### impl<const W: usize , const H: usize > Copy for Frame2d <W, H>

## Auto Trait Implementations §

§

### impl<const W: usize , const H: usize > Freeze for Frame2d <W, H>

§

### impl<const W: usize , const H: usize > RefUnwindSafe for Frame2d <W, H>

§

### impl<const W: usize , const H: usize > Send for Frame2d <W, H>

§

### impl<const W: usize , const H: usize > Sync for Frame2d <W, H>

§

### impl<const W: usize , const H: usize > Unpin for Frame2d <W, H>

§

### impl<const W: usize , const H: usize > UnwindSafe for Frame2d <W, H>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

§

### impl<T> Dimensions for T where T: OriginDimensions,

§

#### fn bounding_box (&self) -> Rectangle

Returns the bounding box.

§

### impl<T> DrawTargetExt for T where T: DrawTarget,

§

#### fn translated (&mut self, offset: Point ) -> Translated<'_, T>

Creates a translated draw target based on this draw target.

Read more

§

#### fn cropped (&mut self, area: &Rectangle) -> Cropped<'_, T>

Creates a cropped draw target based on this draw target.

Read more

§

#### fn clipped (&mut self, area: &Rectangle) -> Clipped<'_, T>

Creates a clipped draw target based on this draw target.

Read more

§

#### fn color_converted <C>(&mut self) -> ColorConverted<'_, T, C> where C: PixelColor + Into <<T as DrawTarget>::Color>,

Creates a color conversion draw target.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<P, T> Receiver for P where P: Deref <Target = T> + ? Sized , T: ? Sized ,

Source

§

#### type Target = T

🔬 This is a nightly-only experimental API. ( arbitrary_self_types )

The target type on which the method may be called.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct LedLayout Copy item path

Path: led2d/struct.LedLayout.html

device_envoy_rp

::

led2d

# Struct LedLayout Copy item path

pub struct LedLayout<const N:
usize
, const W:
usize
, const H:
usize
> {
/* private fields */
}

Expand description

Compile-time description of panel geometry and wiring, including dimensions (with examples).

LedLayout defines how a rectangular (x, y) panel of LEDs maps to the linear wiring order of LEDs on a NeoPixel-style (WS2812) panel.

For examples of LedLayout in use, see the led2d module, Frame2d , and the example below.

What LedLayout does:

Lets you describe panel wiring once

Enables drawing text, graphics, and animations in (x, y) space

Hides LED strip order from rendering code

Coordinates use a screen-style convention:

(0, 0) is the top-left corner

x increases to the right

y increases downward

Most users should start with one of the constructors below and then apply transforms ( rotate_cw , flip_h , combine_v , etc.) as needed.

### § Constructing layouts

Prefer the built-in constructors when possible:

serpentine_row_major

serpentine_column_major

linear_h / linear_v

For unusual wiring, you can construct a layout directly with LedLayout::new by listing (x, y) for each LED in the order the strip is wired.

The example below shows both construction methods. Also, the documentation for every constructor and method includes illustrations of use.

### § Transforming layouts

You can adapt a layout without rewriting it:

rotate: rotate_cw , rotate_ccw , rotate_180

flip: flip_h , flip_v

combine: combine_h , combine_v (join two layouts into a larger one)

### § Validation

Layouts are validated at compile time :

coordinates must be in-bounds

every (x, y) cell must appear exactly once

If you want the final mapping, use index_to_xy .

## § Example

Rotate a serpentine-wired 3×2 panel into a 2×3 layout and verify the result at compile time:

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
2
,
3
> = LedLayout::serpentine_column_major().rotate_cw();
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
1
,
0
), (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
2
), (
0
,
2
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));
// Compile-time assert

Serpentine 3×2 rotated to 2×3:

  Before:              After:
    LED0  LED3  LED4     LED1  LED0
    LED1  LED2  LED5     LED2  LED3
                         LED5  LED4

## Implementations §

§

### impl<const N: usize , const W: usize , const H: usize > LedLayout <N, W, H>

#### pub const fn index_to_xy (&self) -> &[( u16 , u16 ); N ]

Return the array mapping LED wiring order to (x, y) coordinates.

#### pub const fn width (&self) -> usize

The width of the layout.

#### pub const fn height (&self) -> usize

The height of the layout.

#### pub const fn len (&self) -> usize

Total LEDs in this layout (width × height).

#### pub const fn xy_to_index (&self) -> [ u16 ; N ]

Return the inverse mapping: (x, y) coordinates to LED wiring index.

The returned array is indexed by y * W + x and contains the LED wiring index for each pixel position. This is the inverse of index_to_xy .

#### pub const fn equals (&self, other: & LedLayout <N, W, H>) -> bool

Const equality helper for doctests/examples.

use
device_envoy_core::led2d::layout::LedLayout;
const
LINEAR: LedLayout<
4
,
4
,
1
> = LedLayout::linear_h();
const
ROTATED: LedLayout<
4
,
4
,
1
> = LedLayout::linear_v().rotate_cw();
const _
: () =
assert!
(LINEAR.equals(
&
LINEAR));
// assert equal
const _
: () =
assert!
(!LINEAR.equals(
&
ROTATED));
// assert not equal

LINEAR:  LED0  LED1  LED2  LED3
ROTATED: LED3  LED2  LED1  LED0

#### pub const fn new (map: [( u16 , u16 ); N ]) -> LedLayout <N, W, H>

Construct a LedLayout by explicitly specifying the wiring order.

Use this constructor when your panel wiring does not match one of the built-in patterns (linear, serpentine, etc.). You provide the (x, y) coordinate for each LED in strip order , and LedLayout derives the panel geometry from that mapping.

This constructor is const and is intended to be used in a const definition, so layout errors are caught at compile time , not at runtime.

use
device_envoy_core::led2d::layout::LedLayout;
// 3×2 panel (landscape, W×H)
const
MAP: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
0
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
1
,
1
), (
0
,
1
)]);
// Rotate to portrait (CW)
const
ROTATED: LedLayout<
6
,
2
,
3
> = MAP.rotate_cw();
// Expected: 2×3 panel (W×H)
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
1
,
0
), (
1
,
1
), (
1
,
2
), (
0
,
2
), (
0
,
1
), (
0
,
0
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

3×2 input (col,row by LED index):
  LED0  LED1  LED2
  LED5  LED4  LED3

After rotate to 2×3:
  LED1  LED0
  LED2  LED3
  LED5  LED4

#### pub const fn linear_h () -> LedLayout <N, W, H>

Linear row-major mapping for a single-row strip (cols increase left-to-right).

use
device_envoy_core::led2d::layout::LedLayout;
const
LINEAR: LedLayout<
6
,
6
,
1
> = LedLayout::linear_h();
const
EXPECTED: LedLayout<
6
,
6
,
1
> =
    LedLayout::new([(
0
,
0
), (
1
,
0
), (
2
,
0
), (
3
,
0
), (
4
,
0
), (
5
,
0
)]);
const _
: () =
assert!
(LINEAR.equals(
&
EXPECTED));

6×1 strip maps to single row:
  LED0  LED1  LED2  LED3  LED4  LED5

#### pub const fn linear_v () -> LedLayout <N, W, H>

Linear column-major mapping for a single-column strip (rows increase top-to-bottom).

use
device_envoy_core::led2d::layout::LedLayout;
const
LINEAR: LedLayout<
6
,
1
,
6
> = LedLayout::linear_v();
const
EXPECTED: LedLayout<
6
,
1
,
6
> =
    LedLayout::new([(
0
,
0
), (
0
,
1
), (
0
,
2
), (
0
,
3
), (
0
,
4
), (
0
,
5
)]);
const _
: () =
assert!
(LINEAR.equals(
&
EXPECTED));

1×6 strip maps to single column:
  LED0
  LED1
  LED2
  LED3
  LED4
  LED5

#### pub const fn serpentine_column_major () -> LedLayout <N, W, H>

Serpentine column-major mapping returned as a checked LedLayout .

use
device_envoy_core::led2d::layout::LedLayout;
const
MAP: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
0
), (
2
,
0
), (
2
,
1
)]);
const _
: () =
assert!
(MAP.equals(
&
EXPECTED));

Strip snakes down columns (3×2 example):
  LED0  LED3  LED4
  LED1  LED2  LED5

#### pub const fn serpentine_row_major () -> LedLayout <N, W, H>

Serpentine row-major mapping (alternating left-to-right and right-to-left across rows).

use
device_envoy_core::led2d::layout::LedLayout;
const
MAP: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_row_major();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
0
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
1
,
1
), (
0
,
1
)]);
const _
: () =
assert!
(MAP.equals(
&
EXPECTED));

Strip snakes across rows (3×2 example):
  LED0  LED1  LED2
  LED5  LED4  LED3

#### pub const fn rotate_cw (self) -> LedLayout <N, H, W>

Rotate 90° clockwise (dims swap).

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
2
,
3
> = LedLayout::serpentine_column_major().rotate_cw();
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
1
,
0
), (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
2
), (
0
,
2
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

Before (3×2 serpentine): After (2×3):
  LED0  LED3  LED4        LED1  LED0
  LED1  LED2  LED5        LED2  LED3
                          LED5  LED4

#### pub const fn flip_h (self) -> LedLayout <N, W, H>

Flip horizontally (mirror columns).

use
device_envoy_core::led2d::layout::LedLayout;
const
FLIPPED: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major().flip_h();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
2
,
0
), (
2
,
1
), (
1
,
1
), (
1
,
0
), (
0
,
0
), (
0
,
1
)]);
const _
: () =
assert!
(FLIPPED.equals(
&
EXPECTED));

Before (serpentine): After:
  LED0  LED3  LED4      LED4  LED3  LED0
  LED1  LED2  LED5      LED5  LED2  LED1

#### pub const fn rotate_180 (self) -> LedLayout <N, W, H>

Rotate 180° derived from rotate_cw.

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major().rotate_180();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
2
,
1
), (
2
,
0
), (
1
,
0
), (
1
,
1
), (
0
,
1
), (
0
,
0
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

Before (3×2 serpentine): After 180°:
  LED0  LED3  LED4        LED5  LED2  LED1
  LED1  LED2  LED5        LED4  LED3  LED0

#### pub const fn rotate_ccw (self) -> LedLayout <N, H, W>

Rotate 90° counter-clockwise derived from rotate_cw.

use
device_envoy_core::led2d::layout::LedLayout;
const
ROTATED: LedLayout<
6
,
2
,
3
> = LedLayout::serpentine_column_major().rotate_ccw();
const
EXPECTED: LedLayout<
6
,
2
,
3
> =
    LedLayout::new([(
0
,
2
), (
1
,
2
), (
1
,
1
), (
0
,
1
), (
0
,
0
), (
1
,
0
)]);
const _
: () =
assert!
(ROTATED.equals(
&
EXPECTED));

Before (3×2 serpentine): After (2×3):
  LED0  LED3  LED4        LED4  LED5
  LED1  LED2  LED5        LED3  LED2
                          LED0  LED1

#### pub const fn flip_v (self) -> LedLayout <N, W, H>

Flip vertically derived from rotation + horizontal flip.

use
device_envoy_core::led2d::layout::LedLayout;
const
FLIPPED: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major().flip_v();
const
EXPECTED: LedLayout<
6
,
3
,
2
> =
    LedLayout::new([(
0
,
1
), (
0
,
0
), (
1
,
0
), (
1
,
1
), (
2
,
1
), (
2
,
0
)]);
const _
: () =
assert!
(FLIPPED.equals(
&
EXPECTED));

Before (serpentine): After:
  LED0  LED3  LED4      LED1  LED2  LED5
  LED1  LED2  LED5      LED0  LED3  LED4

#### pub const fn combine_h <const N2: usize , const OUT_N: usize , const W2: usize , const OUT_W: usize >( self, right: LedLayout <N2, W2, H>, ) -> LedLayout <OUT_N, OUT_W, H>

Concatenate horizontally with another mapping sharing the same rows.

use
device_envoy_core::led2d::layout::LedLayout;
const
LED_LAYOUT: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major();
const
COMBINED: LedLayout<
12
,
6
,
2
> = LED_LAYOUT.combine_h::<
6
,
12
,
3
,
6
>(LED_LAYOUT);
const
EXPECTED: LedLayout<
12
,
6
,
2
> = LedLayout::new([
    (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
3
,
0
), (
3
,
1
), (
4
,
1
),
    (
4
,
0
), (
5
,
0
), (
5
,
1
),
]);
const _
: () =
assert!
(COMBINED.equals(
&
EXPECTED));

Left serpentine (3×2):    Right serpentine (3×2):
  0  3  4                   6  9 10
  1  2  5                   7  8 11

Combined (6×2):
  0  3  4  6  9 10
  1  2  5  7  8 11

#### pub const fn combine_v <const N2: usize , const OUT_N: usize , const H2: usize , const OUT_H: usize >( self, bottom: LedLayout <N2, W, H2>, ) -> LedLayout <OUT_N, W, OUT_H>

Concatenate vertically with another mapping sharing the same columns.

use
device_envoy_core::led2d::layout::LedLayout;
const
LED_LAYOUT: LedLayout<
6
,
3
,
2
> = LedLayout::serpentine_column_major();
const
COMBINED: LedLayout<
12
,
3
,
4
> = LED_LAYOUT.combine_v::<
6
,
12
,
2
,
4
>(LED_LAYOUT);
const
EXPECTED: LedLayout<
12
,
3
,
4
> = LedLayout::new([
    (
0
,
0
), (
0
,
1
), (
1
,
1
), (
1
,
0
), (
2
,
0
), (
2
,
1
), (
0
,
2
), (
0
,
3
), (
1
,
3
),
    (
1
,
2
), (
2
,
2
), (
2
,
3
),
]);
const _
: () =
assert!
(COMBINED.equals(
&
EXPECTED));

Top serpentine (3×2):    Bottom serpentine (3×2):
  0  3  4                   6  9 10
  1  2  5                   7  8 11

Combined (3×4):
  0  3  4
  1  2  5
  6  9 10
  7  8 11

## Trait Implementations §

§

### impl<const N: usize , const W: usize , const H: usize > Clone for LedLayout <N, W, H>

§

#### fn clone (&self) -> LedLayout <N, W, H>

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl<const N: usize , const W: usize , const H: usize > Debug for LedLayout <N, W, H>

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl<const N: usize , const W: usize , const H: usize > PartialEq for LedLayout <N, W, H>

§

#### fn eq (&self, other: & LedLayout <N, W, H>) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl<const N: usize , const W: usize , const H: usize > Copy for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Eq for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > StructuralPartialEq for LedLayout <N, W, H>

## Auto Trait Implementations §

§

### impl<const N: usize , const W: usize , const H: usize > Freeze for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > RefUnwindSafe for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Send for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Sync for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > Unpin for LedLayout <N, W, H>

§

### impl<const N: usize , const W: usize , const H: usize > UnwindSafe for LedLayout <N, W, H>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Point Copy item path

Path: led2d/struct.Point.html

device_envoy_rp

::

led2d

# Struct Point Copy item path

pub struct Point {
    pub x:
i32
,
    pub y:
i32
,
}

Expand description

2D point.

A point can be used to define the position of a graphics object. For example, a Rectangle may be defined that has its top left corner at (-1, -2) . To specify the size of an object Size should be used instead.

Nalgebra support can be enabled with the nalgebra_support feature. This implements From<Vector2<N>> and From<&Vector2<N>> where N is Scalar + Into<i32> . This allows use of Nalgebra’s Vector2 with embedded-graphics where i8 , i16 , i32 , u16 or u8 is used for value storage.

## § Examples

### § Create a Point from two integers

use
embedded_graphics::geometry::Point;
// Create a coord using the `new` constructor method
let
p = Point::new(
10
,
20
);

### § Create a Point from a Nalgebra Vector2

Be sure to enable the nalgebra_support feature to get Nalgebra integration.

use
embedded_graphics::geometry::Point;
use
nalgebra::Vector2;
let
n_coord = Vector2::new(
10i32
,
20
);
assert_eq!
(Point::from(n_coord), Point::new(
10
,
20
));

### § Convert a Vector2<u8> into a Point

Be sure to enable the nalgebra_support feature to get Nalgebra integration.

Smaller unsigned types that can be converted to i32 are also supported in conversions.

use
embedded_graphics::geometry::Point;
use
nalgebra::Vector2;
let
n_coord = Vector2::new(
10u8
,
20
);
assert_eq!
(Point::from(n_coord), Point::new(
10
,
20
));

## Fields §

§ x: i32

The x coordinate.

§ y: i32

The y coordinate.

## Implementations §

§

### impl Point

#### pub const fn new (x: i32 , y: i32 ) -> Point

Creates a point from X and Y coordinates.

#### pub const fn new_equal (value: i32 ) -> Point

Creates a point with X and Y values set to an equal value.

#### § Examples

use
embedded_graphics::geometry::Point;
let
point = Point::new_equal(
11
);
assert_eq!
(point, Point { x:
11
, y:
11
});

#### pub const fn zero () -> Point

Creates a point with X and Y equal to zero.

#### pub const fn x_axis (self) -> Point

Returns a point with equal x value and y set to 0 .

#### § Examples

#### § Move a Point along the X axis.

use
embedded_graphics::geometry::Point;
let
translate = Point::new(
20
,
30
);
let
point = Point::new(
10
,
15
);
let
moved_x = point + translate.x_axis();
assert_eq!
(moved_x, Point::new(
30
,
15
));

#### pub const fn y_axis (self) -> Point

Returns a point with equal y value and x set to 0 .

#### § Examples

#### § Move a Point along the Y axis.

use
embedded_graphics::geometry::Point;
let
translate = Point::new(
20
,
30
);
let
point = Point::new(
10
,
15
);
let
moved_y = point + translate.y_axis();
assert_eq!
(moved_y, Point::new(
10
,
45
));

#### pub const fn abs (self) -> Point

Remove the sign from a coordinate

#### § Examples

let
point = Point::new(-
5
, -
10
);
assert_eq!
(point.abs(), Point::new(
5
,
10
));

#### pub fn component_min (self, other: Point ) -> Point

Returns the componentwise minimum of two Point s

#### § Examples

use
embedded_graphics::geometry::Point;
let
min = Point::new(
20
,
30
).component_min(Point::new(
15
,
50
));
assert_eq!
(min, Point::new(
15
,
30
));

#### pub fn component_max (self, other: Point ) -> Point

Returns the componentwise maximum of two Point s

#### § Examples

use
embedded_graphics::geometry::Point;
let
min = Point::new(
20
,
30
).component_max(Point::new(
15
,
50
));
assert_eq!
(min, Point::new(
20
,
50
));

#### pub const fn component_mul (self, other: Point ) -> Point

Returns the componentwise multiplication of two Point s.

use
embedded_graphics::geometry::Point;
let
result = Point::new(
20
,
30
).component_mul(Point::new(-
2
,
3
));
assert_eq!
(result, Point::new(-
40
,
90
));

#### pub const fn component_div (self, other: Point ) -> Point

Returns the componentwise division of two Points s.

#### § Panics

Panics if one of the components of other equals zero.

use
embedded_graphics::geometry::Point;
let
result = Point::new(
20
,
30
).component_div(Point::new(
10
, -
3
));
assert_eq!
(result, Point::new(
2
, -
10
));

## Trait Implementations §

§

### impl Add < Size > for Point

§

#### fn add (self, other: Size ) -> Point

Offsets a point by adding a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

#### type Output = Point

The resulting type after applying the

+

operator.

§

### impl Add for Point

§

#### type Output = Point

The resulting type after applying the

+

operator.

§

#### fn add (self, other: Point ) -> Point

Performs the

+

operation.

Read more

§

### impl AddAssign < Size > for Point

§

#### fn add_assign (&mut self, other: Size )

Offsets a point by adding a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

### impl AddAssign for Point

§

#### fn add_assign (&mut self, other: Point )

Performs the

+=

operation.

Read more

§

### impl Clone for Point

§

#### fn clone (&self) -> Point

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Point

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for Point

§

#### fn default () -> Point

Returns the “default value” for a type.

Read more

§

### impl Display for Point

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Div < i32 > for Point

§

#### type Output = Point

The resulting type after applying the

/

operator.

§

#### fn div (self, rhs: i32 ) -> Point

Performs the

/

operation.

Read more

§

### impl DivAssign < i32 > for Point

§

#### fn div_assign (&mut self, rhs: i32 )

Performs the

/=

operation.

Read more

§

### impl From <&[ i32 ; 2 ]> for Point

§

#### fn from (other: &[ i32 ; 2 ]) -> Point

Converts to this type from the input type.

§

### impl From <[ i32 ; 2 ]> for Point

§

#### fn from (other: [ i32 ; 2 ]) -> Point

Converts to this type from the input type.

§

### impl From <( i32 , i32 )> for Point

§

#### fn from (other: ( i32 , i32 )) -> Point

Converts to this type from the input type.

§

### impl From < Point > for [ i32 ; 2 ]

§

#### fn from (other: Point ) -> [ i32 ; 2 ]

Converts to this type from the input type.

§

### impl Hash for Point

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Index < usize > for Point

§

#### type Output = i32

The returned type after indexing.

§

#### fn index (&self, idx: usize ) -> & i32

Performs the indexing (

container[index]

) operation.

Read more

§

### impl Mul < i32 > for Point

§

#### type Output = Point

The resulting type after applying the

*

operator.

§

#### fn mul (self, rhs: i32 ) -> Point

Performs the

*

operation.

Read more

§

### impl MulAssign < i32 > for Point

§

#### fn mul_assign (&mut self, rhs: i32 )

Performs the

*=

operation.

Read more

§

### impl Neg for Point

§

#### type Output = Point

The resulting type after applying the

-

operator.

§

#### fn neg (self) -> < Point as Neg >:: Output

Performs the unary

-

operation.

Read more

§

### impl Ord for Point

§

#### fn cmp (&self, other: & Point ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for Point

§

#### fn eq (&self, other: & Point ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for Point

§

#### fn partial_cmp (&self, other: & Point ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Sub < Size > for Point

§

#### fn sub (self, other: Size ) -> Point

Offsets a point by subtracting a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

#### type Output = Point

The resulting type after applying the

-

operator.

§

### impl Sub for Point

§

#### type Output = Point

The resulting type after applying the

-

operator.

§

#### fn sub (self, other: Point ) -> Point

Performs the

-

operation.

Read more

§

### impl SubAssign < Size > for Point

§

#### fn sub_assign (&mut self, other: Size )

Offsets a point by subtracting a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

### impl SubAssign for Point

§

#### fn sub_assign (&mut self, other: Point )

Performs the

-=

operation.

Read more

§

### impl TryFrom <&[ u32 ; 2 ]> for Point

§

#### type Error = TryFromIntError

The type returned in the event of a conversion error.

§

#### fn try_from ( point: &[ u32 ; 2 ], ) -> Result < Point , < Point as TryFrom <&[ u32 ; 2 ]>>:: Error >

Performs the conversion.

§

### impl TryFrom <[ u32 ; 2 ]> for Point

§

#### type Error = TryFromIntError

The type returned in the event of a conversion error.

§

#### fn try_from ( point: [ u32 ; 2 ], ) -> Result < Point , < Point as TryFrom <[ u32 ; 2 ]>>:: Error >

Performs the conversion.

§

### impl TryFrom <( u32 , u32 )> for Point

§

#### type Error = TryFromIntError

The type returned in the event of a conversion error.

§

#### fn try_from ( point: ( u32 , u32 ), ) -> Result < Point , < Point as TryFrom <( u32 , u32 )>>:: Error >

Performs the conversion.

§

### impl TryFrom < Point > for [ u32 ; 2 ]

§

#### type Error = TryFromIntError

The type returned in the event of a conversion error.

§

#### fn try_from ( point: Point , ) -> Result <[ u32 ; 2 ], <[ u32 ; 2 ] as TryFrom < Point >>:: Error >

Performs the conversion.

§

### impl Copy for Point

§

### impl Eq for Point

§

### impl StructuralPartialEq for Point

## Auto Trait Implementations §

§

### impl Freeze for Point

§

### impl RefUnwindSafe for Point

§

### impl Send for Point

§

### impl Sync for Point

§

### impl Unpin for Point

§

### impl UnwindSafe for Point

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Size Copy item path

Path: led2d/struct.Size.html

device_envoy_rp

::

led2d

# Struct Size Copy item path

pub struct Size {
    pub width:
u32
,
    pub height:
u32
,
}

Expand description

2D size.

Size is used to define the width and height of an object.

Nalgebra support can be enabled with the nalgebra_support feature. This implements From<Vector2<N>> and From<&Vector2<N>> where N is Scalar + Into<u32> . This allows use of Nalgebra’s Vector2 with embedded-graphics where u32 , u16 or u8 is used for value storage.

## § Examples

### § Create a Size from two integers

use
embedded_graphics::geometry::Size;
// Create a size using the `new` constructor method
let
s = Size::new(
10
,
20
);

### § Create a Size from a Nalgebra Vector2

Be sure to enable the nalgebra_support feature to get Nalgebra integration.

Any Vector2<N> can be used where N: Into<u32> + nalgebra::Scalar . This includes the primitive types u32 , u16 and u8 .

use
embedded_graphics::geometry::Size;
use
nalgebra::Vector2;
assert_eq!
(Size::from(Vector2::new(
10u32
,
20
)), Size::new(
10u32
,
20
));
assert_eq!
(Size::from(Vector2::new(
10u16
,
20
)), Size::new(
10u32
,
20
));
assert_eq!
(Size::from(Vector2::new(
10u8
,
20
)), Size::new(
10u32
,
20
));

.into() can also be used, but may require more type annotations:

use
embedded_graphics::geometry::Size;
use
nalgebra::Vector2;
let
c: Size = Vector2::new(
10u32
,
20
).into();
assert_eq!
(c, Size::new(
10u32
,
20
));

## Fields §

§ width: u32

The width.

§ height: u32

The height.

## Implementations §

§

### impl Size

#### pub const fn new (width: u32 , height: u32 ) -> Size

Creates a size from a width and a height.

#### pub const fn new_equal (value: u32 ) -> Size

Creates a size with width and height set to an equal value.

use
embedded_graphics::geometry::Size;
let
size = Size::new_equal(
11
);
assert_eq!
(
    size,
    Size {
        width:
11
,
        height:
11
}
);

#### pub const fn zero () -> Size

Creates a size with width and height equal to zero.

#### pub const fn x_axis (self) -> Size

Returns a size with equal width value and height set to 0 .

#### § Examples

#### § Move a Point along the X axis.

use
embedded_graphics::geometry::{Point, Size};
let
size = Size::new(
20
,
30
);
let
point = Point::new(
10
,
15
);
let
moved_x = point + size.x_axis();
assert_eq!
(moved_x, Point::new(
30
,
15
));

#### pub const fn y_axis (self) -> Size

Returns a size with equal height value and width set to 0 .

#### § Examples

#### § Move a Point along the Y axis.

use
embedded_graphics::geometry::{Point, Size};
let
size = Size::new(
20
,
30
);
let
point = Point::new(
10
,
15
);
let
moved_y = point + size.y_axis();
assert_eq!
(moved_y, Point::new(
10
,
45
));

#### pub const fn saturating_add (self, other: Size ) -> Size

Saturating addition.

Returns u32::max_value() for width and/or height instead of overflowing.

#### pub const fn saturating_sub (self, other: Size ) -> Size

Saturating subtraction.

Returns 0 for width and/or height instead of overflowing, if the value in other is larger then in self .

#### pub fn component_min (self, other: Size ) -> Size

Returns the componentwise minimum of two Size s.

use
embedded_graphics::geometry::Size;
let
min = Size::new(
20
,
30
).component_min(Size::new(
15
,
50
));
assert_eq!
(min, Size::new(
15
,
30
));

#### pub fn component_max (self, other: Size ) -> Size

Returns the componentwise maximum of two Size s.

use
embedded_graphics::geometry::Size;
let
min = Size::new(
20
,
30
).component_max(Size::new(
15
,
50
));
assert_eq!
(min, Size::new(
20
,
50
));

#### pub const fn component_mul (self, other: Size ) -> Size

Returns the componentwise multiplication of two Size s.

use
embedded_graphics::geometry::Size;
let
result = Size::new(
20
,
30
).component_mul(Size::new(
2
,
3
));
assert_eq!
(result, Size::new(
40
,
90
));

#### pub const fn component_div (self, other: Size ) -> Size

Returns the componentwise division of two Size s.

#### § Panics

Panics if one of the components of other equals zero.

use
embedded_graphics::geometry::Size;
let
result = Size::new(
20
,
30
).component_div(Size::new(
5
,
10
));
assert_eq!
(result, Size::new(
4
,
3
));

## Trait Implementations §

§

### impl Add < Size > for Point

§

#### fn add (self, other: Size ) -> Point

Offsets a point by adding a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

#### type Output = Point

The resulting type after applying the

+

operator.

§

### impl Add for Size

§

#### type Output = Size

The resulting type after applying the

+

operator.

§

#### fn add (self, other: Size ) -> Size

Performs the

+

operation.

Read more

§

### impl AddAssign < Size > for Point

§

#### fn add_assign (&mut self, other: Size )

Offsets a point by adding a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

### impl AddAssign for Size

§

#### fn add_assign (&mut self, other: Size )

Performs the

+=

operation.

Read more

§

### impl Clone for Size

§

#### fn clone (&self) -> Size

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Size

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for Size

§

#### fn default () -> Size

Returns the “default value” for a type.

Read more

§

### impl Display for Size

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Div < u32 > for Size

§

#### type Output = Size

The resulting type after applying the

/

operator.

§

#### fn div (self, rhs: u32 ) -> Size

Performs the

/

operation.

Read more

§

### impl DivAssign < u32 > for Size

§

#### fn div_assign (&mut self, rhs: u32 )

Performs the

/=

operation.

Read more

§

### impl From <&[ u32 ; 2 ]> for Size

§

#### fn from (other: &[ u32 ; 2 ]) -> Size

Converts to this type from the input type.

§

### impl From <[ u32 ; 2 ]> for Size

§

#### fn from (other: [ u32 ; 2 ]) -> Size

Converts to this type from the input type.

§

### impl From <( u32 , u32 )> for Size

§

#### fn from (other: ( u32 , u32 )) -> Size

Converts to this type from the input type.

§

### impl From < Size > for [ u32 ; 2 ]

§

#### fn from (other: Size ) -> [ u32 ; 2 ]

Converts to this type from the input type.

§

### impl Hash for Size

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Index < usize > for Size

§

#### type Output = u32

The returned type after indexing.

§

#### fn index (&self, idx: usize ) -> & u32

Performs the indexing (

container[index]

) operation.

Read more

§

### impl Mul < u32 > for Size

§

#### type Output = Size

The resulting type after applying the

*

operator.

§

#### fn mul (self, rhs: u32 ) -> Size

Performs the

*

operation.

Read more

§

### impl MulAssign < u32 > for Size

§

#### fn mul_assign (&mut self, rhs: u32 )

Performs the

*=

operation.

Read more

§

### impl Ord for Size

§

#### fn cmp (&self, other: & Size ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for Size

§

#### fn eq (&self, other: & Size ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for Size

§

#### fn partial_cmp (&self, other: & Size ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl Sub < Size > for Point

§

#### fn sub (self, other: Size ) -> Point

Offsets a point by subtracting a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

#### type Output = Point

The resulting type after applying the

-

operator.

§

### impl Sub for Size

§

#### type Output = Size

The resulting type after applying the

-

operator.

§

#### fn sub (self, other: Size ) -> Size

Performs the

-

operation.

Read more

§

### impl SubAssign < Size > for Point

§

#### fn sub_assign (&mut self, other: Size )

Offsets a point by subtracting a size.

#### § Panics

This function will panic if width or height are too large to be represented as an i32 and debug assertions are enabled.

§

### impl SubAssign for Size

§

#### fn sub_assign (&mut self, other: Size )

Performs the

-=

operation.

Read more

§

### impl Copy for Size

§

### impl Eq for Size

§

### impl StructuralPartialEq for Size

## Auto Trait Implementations §

§

### impl Freeze for Size

§

### impl RefUnwindSafe for Size

§

### impl Send for Size

§

### impl Sync for Size

§

### impl Unpin for Size

§

### impl UnwindSafe for Size

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Led2d Copy item path

Path: led2d/trait.Led2d.html

device_envoy_rp

::

led2d

# Trait Led2d Copy item path

pub trait Led2d<const W:
usize
, const H:
usize
> {
    const
MAX_FRAMES
:
usize
;
    const
MAX_BRIGHTNESS
:
u8
;
    const
FONT
:
Led2dFont
;
    const
WIDTH
:
usize
= W;
    const
HEIGHT
:
usize
= H;
    const
LEN
:
usize
= _;
    const
SIZE
:
Size
= Frame2d<W, H>::SIZE;
    const
TOP_LEFT
:
Point
= Frame2d<W, H>::TOP_LEFT;
    const
TOP_RIGHT
:
Point
= Frame2d<W, H>::TOP_RIGHT;
    const
BOTTOM_LEFT
:
Point
= Frame2d<W, H>::BOTTOM_LEFT;
    const
BOTTOM_RIGHT
:
Point
= Frame2d<W, H>::BOTTOM_RIGHT;

    // Required methods
    fn
write_frame
(&self, frame2d:
Frame2d
<W, H>);
fn
animate
<I>(&self, frames: I)
where I:
IntoIterator
,
             <I as
IntoIterator
>::
Item
:
Borrow
<(
Frame2d
<W, H>, Duration)>
;

    // Provided methods
    fn
write_text_to_frame
(
        &self,
        text: &
str
,
        colors: &[Rgb<
u8
>],
        frame: &mut
Frame2d
<W, H>,
    ) { ... }
fn
write_text
(&self, text: &
str
, colors: &[Rgb<
u8
>]) { ... }
}

Expand description

Platform-agnostic LED panel device contract.

Platform crates implement this for their concrete LED panel types so shared logic can drive LED panels without knowing the underlying hardware backend.

This page serves as the definitive reference for what a generated LED panel type provides. For first-time readers, start with the led2d module documentation in your platform crate ( device-envoy-rp or device-envoy-esp ), then return here for a complete list of available methods and associated constants.

Design intent:

Primitive operations are Led2d::write_frame and Led2d::animate .

Convenience text operations ( Led2d::write_text_to_frame and Led2d::write_text ) are default methods derived from primitives and associated constants.

This trait is intended for static dispatch on embedded targets.

The trait takes const W and const H so dimensions remain compile-time constants and can be used in frame types like Frame2d<W, H> .

## § Example: Write Text

In this example, we render text on a 12x4 panel.

use
device_envoy_core::led2d::Led2d;
use
smart_leds::RGB8;
fn
write_rust<
const
W: usize,
const
H: usize>(led2d:
&
impl
Led2d<W, H>) {
let
colors = [
        RGB8::new(
0
,
255
,
255
),
        RGB8::new(
255
,
0
,
0
),
        RGB8::new(
255
,
255
,
0
),
    ];
    led2d.write_text(
"Rust"
,
&
colors);
}

## § Example: Animated Text

This example animates text on an LED panel.

use
device_envoy_core::led2d::{Frame2d, Led2d};
use
smart_leds::colors;
fn
animate_go_go<
const
W: usize,
const
H: usize>(led2d:
&
impl
Led2d<W, H>) {
let
mut
frame_0 = Frame2d::new();
    led2d.write_text_to_frame(
"Go"
,
&
[],
&mut
frame_0);
let
mut
frame_1 = Frame2d::new();
    led2d.write_text_to_frame(
"\nGo"
,
&
[colors::HOT_PINK, colors::LIME],
&mut
frame_1);
let
frame_duration = embassy_time::Duration::from_secs(
1
);
    led2d.animate([(frame_0, frame_duration), (frame_1, frame_duration)]);
}

## Required Associated Constants §

#### const MAX_FRAMES : usize

Maximum number of animation frames allowed.

Usually configured by the platform macro (for example led2d! ).

#### const MAX_BRIGHTNESS : u8

Maximum brightness level, automatically limited by the power budget.

Many implementations assume each LED draws about 60 mA at full brightness and compute a safe cap from power budget and LED count.

#### const FONT : Led2dFont

The font used by default text helpers.

Used by Led2d::write_text_to_frame and Led2d::write_text .

## Provided Associated Constants §

#### const WIDTH : usize = W

The width of the panel.

#### const HEIGHT : usize = H

The height of the panel.

#### const LEN : usize = _

Total LEDs in this panel (width × height).

#### const SIZE : Size = Frame2d<W, H>::SIZE

Panel dimensions as a Size .

For embedded-graphics drawing operations.

#### const TOP_LEFT : Point = Frame2d<W, H>::TOP_LEFT

Top-left corner coordinate as a Point .

For embedded-graphics drawing operations.

#### const TOP_RIGHT : Point = Frame2d<W, H>::TOP_RIGHT

Top-right corner coordinate as a Point .

For embedded-graphics drawing operations.

#### const BOTTOM_LEFT : Point = Frame2d<W, H>::BOTTOM_LEFT

Bottom-left corner coordinate as a Point .

For embedded-graphics drawing operations.

#### const BOTTOM_RIGHT : Point = Frame2d<W, H>::BOTTOM_RIGHT

Bottom-right corner coordinate as a Point .

For embedded-graphics drawing operations.

## Required Methods §

#### fn write_frame (&self, frame2d: Frame2d <W, H>)

Write a frame to the LED panel.

See your platform crate’s led2d module docs for possible usage examples.

#### fn animate <I>(&self, frames: I) where I: IntoIterator , <I as IntoIterator >:: Item : Borrow <( Frame2d <W, H>, Duration)>,

Animate frames on the LED panel.

The duration type is embassy_time::Duration , and frames can be any iterator whose items borrow (Frame2d<W, H>, embassy_time::Duration) .

See the Led2d trait documentation for usage examples.

## Provided Methods §

#### fn write_text_to_frame ( &self, text: & str , colors: &[Rgb< u8 >], frame: &mut Frame2d <W, H>, )

Write text into a frame.

This is a default helper built on render_text_to_frame plus associated constants.

Behavior:

Text is drawn with Led2d::FONT .

colors cycles one color per character; an empty slice defaults to white.

A \n character starts a new line.

Characters beyond frame width are clipped.

See the Led2d trait documentation for usage examples.

#### fn write_text (&self, text: & str , colors: &[Rgb< u8 >])

Write text to the LED panel.

This default helper is equivalent to:

Create Frame2d::<W, H>::new() .

Call Led2d::write_text_to_frame .

Call Led2d::write_frame .

See the Led2d trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Led2d <12, 4> for Led2dGenerated

Source

§

#### const WIDTH : usize = Self::WIDTH

Source

§

#### const HEIGHT : usize = Self::HEIGHT

Source

§

#### const LEN : usize = Self::LEN

Source

§

#### const SIZE : Size = Self::SIZE

Source

§

#### const TOP_LEFT : Point = Self::TOP_LEFT

Source

§

#### const TOP_RIGHT : Point = Self::TOP_RIGHT

Source

§

#### const BOTTOM_LEFT : Point = Self::BOTTOM_LEFT

Source

§

#### const BOTTOM_RIGHT : Point = Self::BOTTOM_RIGHT

Source

§

#### const MAX_FRAMES : usize = Self::MAX_FRAMES

Source

§

#### const MAX_BRIGHTNESS : u8 = Self::MAX_BRIGHTNESS

Source

§

#### const FONT : Led2dFont = Self::FONT

# Constant ANIMATION_ MAX_ FRAMES Copy item path

Path: led4/constant.ANIMATION_MAX_FRAMES.html

device_envoy_rp

::

led4

# Constant ANIMATION_ MAX_ FRAMES Copy item path

pub const ANIMATION_MAX_FRAMES:
usize
= 16; // 16usize

Expand description

Maximum number of animation frames accepted by led4-style APIs.

# Enum Blink State Copy item path

Path: led4/enum.BlinkState.html

device_envoy_rp

::

led4

# Enum Blink State Copy item path

pub enum BlinkState {
    Solid,
    BlinkingAndOn,
    BlinkingButOff,
}

Expand description

Blinking behavior for 4-digit LED displays.

## Variants §

§

### Solid

Display is always on (solid, no blinking).

§

### BlinkingAndOn

Display blinks; currently shows on.

§

### BlinkingButOff

Display blinks; currently shows off.

## Trait Implementations §

§

### impl Clone for BlinkState

§

#### fn clone (&self) -> BlinkState

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for BlinkState

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for BlinkState

§

#### fn default () -> BlinkState

Returns the “default value” for a type.

Read more

§

### impl Format for BlinkState

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl Copy for BlinkState

## Auto Trait Implementations §

§

### impl Freeze for BlinkState

§

### impl RefUnwindSafe for BlinkState

§

### impl Send for BlinkState

§

### impl Sync for BlinkState

§

### impl Unpin for BlinkState

§

### impl UnwindSafe for BlinkState

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Function circular_ outline_ animation Copy item path

Path: led4/fn.circular_outline_animation.html

device_envoy_rp

::

led4

# Function circular_ outline_ animation Copy item path

Source

pub fn circular_outline_animation(clockwise:
bool
) ->
Animation

Expand description

Creates a circular outline animation that chases around display edges.

# Module led4 Copy item path

Path: led4/index.html

device_envoy_rp

# Module led4 Copy item path

Source

Expand description

A device abstraction for a 4-digit, 7-segment LED display for text with optional animation and blinking.

See Led4Rp for the primary text/blinking example and Led4 for trait methods.

Limations : You can create up to two concurrent Led4Rp instances per program; a third is expected to fail at runtime because the led4 task pool uses pool_size = 2 . Animation APIs support up to 16 steps per animation ( ANIMATION_MAX_FRAMES ).

This module provides device abstraction for controlling common-cathode 4-digit 7-segment LED displays. Supports displaying text and numbers with optional blinking.

## Structs §

Animation Frame Frame of animated text for Led4::animate_text . Led4Rp A device abstraction for a 4-digit, 7-segment LED display with blinking support. Led4 RpStatic Static for the Led4Rp device. Output Array Array of GPIO output pins for LED displays.

## Enums §

Blink State Blinking behavior for 4-digit LED displays.

## Constants §

ANIMATION_ MAX_ FRAMES Maximum number of animation frames accepted by led4-style APIs.

## Traits §

Led4 Platform-agnostic 4-digit display contract.

## Functions §

circular_ outline_ animation Creates a circular outline animation that chases around display edges.

## Type Aliases §

Animation Frame buffer type used by led4 text animations.

# struct.OutputArray.html

Path: led4/output_array/struct.OutputArray.html

Redirecting to ../../../device_envoy_rp/led4/struct.OutputArray.html ...

# Struct Animation Frame Copy item path

Path: led4/struct.AnimationFrame.html

device_envoy_rp

::

led4

# Struct Animation Frame Copy item path

pub struct AnimationFrame {
    pub text: [
char
;
4
],
    pub duration: Duration,
}

Expand description

Frame of animated text for Led4::animate_text .

## Fields §

§ text: [ char ; 4 ]

Text to display (4 characters for a 4-digit display).

§ duration: Duration

Duration to display this frame. This uses embassy_time::Duration .

## Implementations §

§

### impl AnimationFrame

#### pub const fn new (text: [ char ; 4 ], duration: Duration) -> AnimationFrame

Creates a new animation frame with text and duration. This method uses embassy_time::Duration for frame timing.

## Trait Implementations §

§

### impl Clone for AnimationFrame

§

#### fn clone (&self) -> AnimationFrame

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for AnimationFrame

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Copy for AnimationFrame

## Auto Trait Implementations §

§

### impl Freeze for AnimationFrame

§

### impl RefUnwindSafe for AnimationFrame

§

### impl Send for AnimationFrame

§

### impl Sync for AnimationFrame

§

### impl Unpin for AnimationFrame

§

### impl UnwindSafe for AnimationFrame

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Led4Rp Copy item path

Path: led4/struct.Led4Rp.html

device_envoy_rp

::

led4

# Struct Led4Rp Copy item path

Source

pub struct Led4Rp<'a>(
/* private fields */
);

Expand description

A device abstraction for a 4-digit, 7-segment LED display with blinking support.

## § Hardware Requirements

This abstraction is designed for common-cathode 7-segment displays where:

Cell pins control which digit is active (LOW = on, HIGH = off)

Segment pins control which segments light up (HIGH = on, LOW = off)

## § Example

use
embassy_time::{Duration, Timer};
use
device_envoy_rp::{Error, led4::{BlinkState, Led4
as _
, Led4Rp, Led4RpStatic, OutputArray, circular_outline_animation}};
async fn
example(p: embassy_rp::Peripherals, spawner: embassy_executor::Spawner) ->
Result
<(), Error> {
// Set up cell pins (control which digit is active)
let
cells = OutputArray::new([
        embassy_rp::gpio::Output::new(p.PIN_1, embassy_rp::gpio::Level::High),
        embassy_rp::gpio::Output::new(p.PIN_2, embassy_rp::gpio::Level::High),
        embassy_rp::gpio::Output::new(p.PIN_3, embassy_rp::gpio::Level::High),
        embassy_rp::gpio::Output::new(p.PIN_4, embassy_rp::gpio::Level::High),
    ]);
// Set up segment pins (control which segments light up)
let
segments = OutputArray::new([
        embassy_rp::gpio::Output::new(p.PIN_5, embassy_rp::gpio::Level::Low),
// Segment A
embassy_rp::gpio::Output::new(p.PIN_6, embassy_rp::gpio::Level::Low),
// Segment B
embassy_rp::gpio::Output::new(p.PIN_7, embassy_rp::gpio::Level::Low),
// Segment C
embassy_rp::gpio::Output::new(p.PIN_8, embassy_rp::gpio::Level::Low),
// Segment D
embassy_rp::gpio::Output::new(p.PIN_9, embassy_rp::gpio::Level::Low),
// Segment E
embassy_rp::gpio::Output::new(p.PIN_10, embassy_rp::gpio::Level::Low),
// Segment F
embassy_rp::gpio::Output::new(p.PIN_11, embassy_rp::gpio::Level::Low),
// Segment G
embassy_rp::gpio::Output::new(p.PIN_12, embassy_rp::gpio::Level::Low),
// Decimal point
]);
// Create the display
static
LED4_STATIC: Led4RpStatic = Led4Rp::new_static();
let
display = Led4Rp::new(
&
LED4_STATIC, cells, segments, spawner)
?
;
// Blink "1234" for three seconds.
display.write_text([
'1'
,
'2'
,
'3'
,
'4'
], BlinkState::BlinkingAndOn);
    Timer::after(Duration::from_secs(
3
)).
await
;
// Run the circular outline animation for three seconds.
display.animate_text(circular_outline_animation(
true
));
    Timer::after(Duration::from_secs(
3
)).
await
;
// Show "rUSt" solid forever.
display.write_text([
'r'
,
'U'
,
'S'
,
't'
], BlinkState::Solid);
    pending().
await
}

Beyond simple text, the driver can loop animations via Led4::animate_text . The struct owns the background task and signal wiring; create it once with Led4Rp::new and use the returned handle for all display updates.

## Implementations §

Source

§

### impl Led4Rp <'_>

Source

#### pub fn new ( led4_static: &'static Led4RpStatic , cell_pins: OutputArray <'static, CELL_COUNT>, segment_pins: OutputArray <'static, SEGMENT_COUNT>, spawner: Spawner, ) -> Result <Self>

Creates the display device and spawns its background task; see Led4Rp docs.

Source

#### pub const fn new_static () -> Led4RpStatic

Creates static channel resources for Led4Rp::new ; see Led4Rp docs.

## Trait Implementations §

Source

§

### impl Led4 for Led4Rp <'_>

Source

§

#### fn write_text (&self, text: [ char ; 4 ], blink_state: BlinkState )

Send text to the display with optional blinking.

Read more

Source

§

#### fn animate_text <I>(&self, animation: I) where I: IntoIterator , I:: Item : Borrow < AnimationFrame >,

Play a looped text animation from the provided frames.

Read more

## Auto Trait Implementations §

§

### impl<'a> Freeze for Led4Rp <'a>

§

### impl<'a> ! RefUnwindSafe for Led4Rp <'a>

§

### impl<'a> Send for Led4Rp <'a>

§

### impl<'a> Sync for Led4Rp <'a>

§

### impl<'a> Unpin for Led4Rp <'a>

§

### impl<'a> ! UnwindSafe for Led4Rp <'a>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Led4 RpStatic Copy item path

Path: led4/struct.Led4RpStatic.html

device_envoy_rp

::

led4

# Struct Led4 RpStatic Copy item path

Source

pub struct Led4RpStatic {
/* private fields */
}

Expand description

Static for the Led4Rp device.

## Auto Trait Implementations §

§

### impl ! Freeze for Led4RpStatic

§

### impl ! RefUnwindSafe for Led4RpStatic

§

### impl Send for Led4RpStatic

§

### impl Sync for Led4RpStatic

§

### impl Unpin for Led4RpStatic

§

### impl UnwindSafe for Led4RpStatic

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Output Array Copy item path

Path: led4/struct.OutputArray.html

device_envoy_rp

::

led4

# Struct Output Array Copy item path

Source

pub struct OutputArray<'a, const N:
usize
>(
/* private fields */
);

Expand description

Array of GPIO output pins for LED displays.

See the Led4Rp documentation for usage examples.

## Implementations §

Source

§

### impl<'a, const N: usize > OutputArray <'a, N>

Source

#### pub const fn new (outputs: [Output<'a>; N ]) -> Self

Creates a new output array from GPIO pins.

## Auto Trait Implementations §

§

### impl<'a, const N: usize > Freeze for OutputArray <'a, N>

§

### impl<'a, const N: usize > RefUnwindSafe for OutputArray <'a, N>

§

### impl<'a, const N: usize > Send for OutputArray <'a, N>

§

### impl<'a, const N: usize > Sync for OutputArray <'a, N>

§

### impl<'a, const N: usize > Unpin for OutputArray <'a, N>

§

### impl<'a, const N: usize > ! UnwindSafe for OutputArray <'a, N>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Led4 Copy item path

Path: led4/trait.Led4.html

device_envoy_rp

::

led4

# Trait Led4 Copy item path

pub trait Led4 {
    // Required methods
    fn
write_text
(&self, text: [
char
;
4
], blink_state:
BlinkState
);
fn
animate_text
<I>(&self, animation: I)
where I:
IntoIterator
,
             <I as
IntoIterator
>::
Item
:
Borrow
<
AnimationFrame
>
;
}

Expand description

Platform-agnostic 4-digit display contract.

Platform crates implement this trait for their concrete runtime handles. Constructors ( new , new_static ) remain inherent on platform types.

## § Example

use
device_envoy_core::led4::{AnimationFrame, BlinkState, Led4, circular_outline_animation};
use
embassy_time::{Duration, Timer};
async fn
show_status(led4:
&
impl
Led4) -> ! {
// Blink "1234" for three seconds.
led4.write_text([
'1'
,
'2'
,
'3'
,
'4'
], BlinkState::BlinkingAndOn);
    Timer::after(Duration::from_secs(
3
)).
await
;
// Run the circular outline animation for three seconds.
led4.animate_text(circular_outline_animation(
true
));
    Timer::after(Duration::from_secs(
3
)).
await
;
// Show "rUSt" solid forever.
led4.write_text([
'r'
,
'U'
,
'S'
,
't'
], BlinkState::Solid);
    core::future::pending().
await
}

## Required Methods §

#### fn write_text (&self, text: [ char ; 4 ], blink_state: BlinkState )

Send text to the display with optional blinking.

See the Led4 trait documentation for usage examples.

#### fn animate_text <I>(&self, animation: I) where I: IntoIterator , <I as IntoIterator >:: Item : Borrow < AnimationFrame >,

Play a looped text animation from the provided frames.

See the Led4 trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Led4 for Led4Rp <'_>

# Type Alias Animation Copy item path

Path: led4/type.Animation.html

device_envoy_rp

::

led4

# Type Alias Animation Copy item path

Source

pub type Animation = Animation;

Expand description

Frame buffer type used by led4 text animations.

## Aliased Type §

pub struct Animation {
/* private fields */
}

# Constant ALICE_ BLUE Copy item path

Path: led_strip/colors/constant.ALICE_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant ALICE_ BLUE Copy item path

pub const ALICE_BLUE: Rgb<
u8
>;

# Constant ANTINQUE_ WHITE Copy item path

Path: led_strip/colors/constant.ANTINQUE_WHITE.html

device_envoy_rp

::

led_strip :: colors

# Constant ANTINQUE_ WHITE Copy item path

pub const ANTINQUE_WHITE: Rgb<
u8
>;

# Constant AQUA Copy item path

Path: led_strip/colors/constant.AQUA.html

device_envoy_rp

::

led_strip :: colors

# Constant AQUA Copy item path

pub const AQUA: Rgb<
u8
>;

# Constant AQUAMARINE Copy item path

Path: led_strip/colors/constant.AQUAMARINE.html

device_envoy_rp

::

led_strip :: colors

# Constant AQUAMARINE Copy item path

pub const AQUAMARINE: Rgb<
u8
>;

# Constant AZURE Copy item path

Path: led_strip/colors/constant.AZURE.html

device_envoy_rp

::

led_strip :: colors

# Constant AZURE Copy item path

pub const AZURE: Rgb<
u8
>;

# Constant BEIGE Copy item path

Path: led_strip/colors/constant.BEIGE.html

device_envoy_rp

::

led_strip :: colors

# Constant BEIGE Copy item path

pub const BEIGE: Rgb<
u8
>;

# Constant BISQUE Copy item path

Path: led_strip/colors/constant.BISQUE.html

device_envoy_rp

::

led_strip :: colors

# Constant BISQUE Copy item path

pub const BISQUE: Rgb<
u8
>;

# Constant BLACK Copy item path

Path: led_strip/colors/constant.BLACK.html

device_envoy_rp

::

led_strip :: colors

# Constant BLACK Copy item path

pub const BLACK: Rgb<
u8
>;

# Constant BLANCHED_ ALMOND Copy item path

Path: led_strip/colors/constant.BLANCHED_ALMOND.html

device_envoy_rp

::

led_strip :: colors

# Constant BLANCHED_ ALMOND Copy item path

pub const BLANCHED_ALMOND: Rgb<
u8
>;

# Constant BLUE Copy item path

Path: led_strip/colors/constant.BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant BLUE Copy item path

pub const BLUE: Rgb<
u8
>;

# Constant BLUE_ VIOLET Copy item path

Path: led_strip/colors/constant.BLUE_VIOLET.html

device_envoy_rp

::

led_strip :: colors

# Constant BLUE_ VIOLET Copy item path

pub const BLUE_VIOLET: Rgb<
u8
>;

# Constant BROWN Copy item path

Path: led_strip/colors/constant.BROWN.html

device_envoy_rp

::

led_strip :: colors

# Constant BROWN Copy item path

pub const BROWN: Rgb<
u8
>;

# Constant BURLYWOOD Copy item path

Path: led_strip/colors/constant.BURLYWOOD.html

device_envoy_rp

::

led_strip :: colors

# Constant BURLYWOOD Copy item path

pub const BURLYWOOD: Rgb<
u8
>;

# Constant CADET_ BLUE Copy item path

Path: led_strip/colors/constant.CADET_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant CADET_ BLUE Copy item path

pub const CADET_BLUE: Rgb<
u8
>;

# Constant CHARTREUSE Copy item path

Path: led_strip/colors/constant.CHARTREUSE.html

device_envoy_rp

::

led_strip :: colors

# Constant CHARTREUSE Copy item path

pub const CHARTREUSE: Rgb<
u8
>;

# Constant CHOCOLATE Copy item path

Path: led_strip/colors/constant.CHOCOLATE.html

device_envoy_rp

::

led_strip :: colors

# Constant CHOCOLATE Copy item path

pub const CHOCOLATE: Rgb<
u8
>;

# Constant CORAL Copy item path

Path: led_strip/colors/constant.CORAL.html

device_envoy_rp

::

led_strip :: colors

# Constant CORAL Copy item path

pub const CORAL: Rgb<
u8
>;

# Constant CORNFLOWER_ BLUE Copy item path

Path: led_strip/colors/constant.CORNFLOWER_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant CORNFLOWER_ BLUE Copy item path

pub const CORNFLOWER_BLUE: Rgb<
u8
>;

# Constant CORNSILK Copy item path

Path: led_strip/colors/constant.CORNSILK.html

device_envoy_rp

::

led_strip :: colors

# Constant CORNSILK Copy item path

pub const CORNSILK: Rgb<
u8
>;

# Constant CRIMSON Copy item path

Path: led_strip/colors/constant.CRIMSON.html

device_envoy_rp

::

led_strip :: colors

# Constant CRIMSON Copy item path

pub const CRIMSON: Rgb<
u8
>;

# Constant CYAN Copy item path

Path: led_strip/colors/constant.CYAN.html

device_envoy_rp

::

led_strip :: colors

# Constant CYAN Copy item path

pub const CYAN: Rgb<
u8
>;

# Constant DARK_ BLUE Copy item path

Path: led_strip/colors/constant.DARK_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ BLUE Copy item path

pub const DARK_BLUE: Rgb<
u8
>;

# Constant DARK_ CYAN Copy item path

Path: led_strip/colors/constant.DARK_CYAN.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ CYAN Copy item path

pub const DARK_CYAN: Rgb<
u8
>;

# Constant DARK_ GOLDENROD Copy item path

Path: led_strip/colors/constant.DARK_GOLDENROD.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ GOLDENROD Copy item path

pub const DARK_GOLDENROD: Rgb<
u8
>;

# Constant DARK_ GRAY Copy item path

Path: led_strip/colors/constant.DARK_GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ GRAY Copy item path

pub const DARK_GRAY: Rgb<
u8
>;

# Constant DARK_ GREEN Copy item path

Path: led_strip/colors/constant.DARK_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ GREEN Copy item path

pub const DARK_GREEN: Rgb<
u8
>;

# Constant DARK_ KHAKI Copy item path

Path: led_strip/colors/constant.DARK_KHAKI.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ KHAKI Copy item path

pub const DARK_KHAKI: Rgb<
u8
>;

# Constant DARK_ MAGENTA Copy item path

Path: led_strip/colors/constant.DARK_MAGENTA.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ MAGENTA Copy item path

pub const DARK_MAGENTA: Rgb<
u8
>;

# Constant DARK_ OLIVE_ GREEN Copy item path

Path: led_strip/colors/constant.DARK_OLIVE_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ OLIVE_ GREEN Copy item path

pub const DARK_OLIVE_GREEN: Rgb<
u8
>;

# Constant DARK_ ORANGE Copy item path

Path: led_strip/colors/constant.DARK_ORANGE.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ ORANGE Copy item path

pub const DARK_ORANGE: Rgb<
u8
>;

# Constant DARK_ ORCHID Copy item path

Path: led_strip/colors/constant.DARK_ORCHID.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ ORCHID Copy item path

pub const DARK_ORCHID: Rgb<
u8
>;

# Constant DARK_ RED Copy item path

Path: led_strip/colors/constant.DARK_RED.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ RED Copy item path

pub const DARK_RED: Rgb<
u8
>;

# Constant DARK_ SALMON Copy item path

Path: led_strip/colors/constant.DARK_SALMON.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ SALMON Copy item path

pub const DARK_SALMON: Rgb<
u8
>;

# Constant DARK_ SEA_ GREEN Copy item path

Path: led_strip/colors/constant.DARK_SEA_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ SEA_ GREEN Copy item path

pub const DARK_SEA_GREEN: Rgb<
u8
>;

# Constant DARK_ SLATE_ BLUE Copy item path

Path: led_strip/colors/constant.DARK_SLATE_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ SLATE_ BLUE Copy item path

pub const DARK_SLATE_BLUE: Rgb<
u8
>;

# Constant DARK_ SLATE_ GRAY Copy item path

Path: led_strip/colors/constant.DARK_SLATE_GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ SLATE_ GRAY Copy item path

pub const DARK_SLATE_GRAY: Rgb<
u8
>;

# Constant DARK_ TURQUOISE Copy item path

Path: led_strip/colors/constant.DARK_TURQUOISE.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ TURQUOISE Copy item path

pub const DARK_TURQUOISE: Rgb<
u8
>;

# Constant DARK_ VIOLET Copy item path

Path: led_strip/colors/constant.DARK_VIOLET.html

device_envoy_rp

::

led_strip :: colors

# Constant DARK_ VIOLET Copy item path

pub const DARK_VIOLET: Rgb<
u8
>;

# Constant DEEP_ PINK Copy item path

Path: led_strip/colors/constant.DEEP_PINK.html

device_envoy_rp

::

led_strip :: colors

# Constant DEEP_ PINK Copy item path

pub const DEEP_PINK: Rgb<
u8
>;

# Constant DEEP_ SKY_ BLUE Copy item path

Path: led_strip/colors/constant.DEEP_SKY_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant DEEP_ SKY_ BLUE Copy item path

pub const DEEP_SKY_BLUE: Rgb<
u8
>;

# Constant DIM_ GRAY Copy item path

Path: led_strip/colors/constant.DIM_GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant DIM_ GRAY Copy item path

pub const DIM_GRAY: Rgb<
u8
>;

# Constant DODGER_ BLUE Copy item path

Path: led_strip/colors/constant.DODGER_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant DODGER_ BLUE Copy item path

pub const DODGER_BLUE: Rgb<
u8
>;

# Constant FIREBRICK Copy item path

Path: led_strip/colors/constant.FIREBRICK.html

device_envoy_rp

::

led_strip :: colors

# Constant FIREBRICK Copy item path

pub const FIREBRICK: Rgb<
u8
>;

# Constant FLORAL_ WHITE Copy item path

Path: led_strip/colors/constant.FLORAL_WHITE.html

device_envoy_rp

::

led_strip :: colors

# Constant FLORAL_ WHITE Copy item path

pub const FLORAL_WHITE: Rgb<
u8
>;

# Constant FOREST_ GREEN Copy item path

Path: led_strip/colors/constant.FOREST_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant FOREST_ GREEN Copy item path

pub const FOREST_GREEN: Rgb<
u8
>;

# Constant FUCHSIA Copy item path

Path: led_strip/colors/constant.FUCHSIA.html

device_envoy_rp

::

led_strip :: colors

# Constant FUCHSIA Copy item path

pub const FUCHSIA: Rgb<
u8
>;

# Constant GAINSBORO Copy item path

Path: led_strip/colors/constant.GAINSBORO.html

device_envoy_rp

::

led_strip :: colors

# Constant GAINSBORO Copy item path

pub const GAINSBORO: Rgb<
u8
>;

# Constant GHOST_ WHITE Copy item path

Path: led_strip/colors/constant.GHOST_WHITE.html

device_envoy_rp

::

led_strip :: colors

# Constant GHOST_ WHITE Copy item path

pub const GHOST_WHITE: Rgb<
u8
>;

# Constant GOLD Copy item path

Path: led_strip/colors/constant.GOLD.html

device_envoy_rp

::

led_strip :: colors

# Constant GOLD Copy item path

pub const GOLD: Rgb<
u8
>;

# Constant GOLDENROD Copy item path

Path: led_strip/colors/constant.GOLDENROD.html

device_envoy_rp

::

led_strip :: colors

# Constant GOLDENROD Copy item path

pub const GOLDENROD: Rgb<
u8
>;

# Constant GRAY Copy item path

Path: led_strip/colors/constant.GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant GRAY Copy item path

pub const GRAY: Rgb<
u8
>;

# Constant GREEN Copy item path

Path: led_strip/colors/constant.GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant GREEN Copy item path

pub const GREEN: Rgb<
u8
>;

# Constant GREEN_ YELLOW Copy item path

Path: led_strip/colors/constant.GREEN_YELLOW.html

device_envoy_rp

::

led_strip :: colors

# Constant GREEN_ YELLOW Copy item path

pub const GREEN_YELLOW: Rgb<
u8
>;

# Constant HONEYDEW Copy item path

Path: led_strip/colors/constant.HONEYDEW.html

device_envoy_rp

::

led_strip :: colors

# Constant HONEYDEW Copy item path

pub const HONEYDEW: Rgb<
u8
>;

# Constant HOT_ PINK Copy item path

Path: led_strip/colors/constant.HOT_PINK.html

device_envoy_rp

::

led_strip :: colors

# Constant HOT_ PINK Copy item path

pub const HOT_PINK: Rgb<
u8
>;

# Constant INDIAN_ RED Copy item path

Path: led_strip/colors/constant.INDIAN_RED.html

device_envoy_rp

::

led_strip :: colors

# Constant INDIAN_ RED Copy item path

pub const INDIAN_RED: Rgb<
u8
>;

# Constant INDIGO Copy item path

Path: led_strip/colors/constant.INDIGO.html

device_envoy_rp

::

led_strip :: colors

# Constant INDIGO Copy item path

pub const INDIGO: Rgb<
u8
>;

# Constant IVORY Copy item path

Path: led_strip/colors/constant.IVORY.html

device_envoy_rp

::

led_strip :: colors

# Constant IVORY Copy item path

pub const IVORY: Rgb<
u8
>;

# Constant KHAKI Copy item path

Path: led_strip/colors/constant.KHAKI.html

device_envoy_rp

::

led_strip :: colors

# Constant KHAKI Copy item path

pub const KHAKI: Rgb<
u8
>;

# Constant LAVENDER Copy item path

Path: led_strip/colors/constant.LAVENDER.html

device_envoy_rp

::

led_strip :: colors

# Constant LAVENDER Copy item path

pub const LAVENDER: Rgb<
u8
>;

# Constant LAVENDER_ BLUSH Copy item path

Path: led_strip/colors/constant.LAVENDER_BLUSH.html

device_envoy_rp

::

led_strip :: colors

# Constant LAVENDER_ BLUSH Copy item path

pub const LAVENDER_BLUSH: Rgb<
u8
>;

# Constant LAWN_ GREEN Copy item path

Path: led_strip/colors/constant.LAWN_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant LAWN_ GREEN Copy item path

pub const LAWN_GREEN: Rgb<
u8
>;

# Constant LEMON_ CHIFFON Copy item path

Path: led_strip/colors/constant.LEMON_CHIFFON.html

device_envoy_rp

::

led_strip :: colors

# Constant LEMON_ CHIFFON Copy item path

pub const LEMON_CHIFFON: Rgb<
u8
>;

# Constant LIGHT_ BLUE Copy item path

Path: led_strip/colors/constant.LIGHT_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ BLUE Copy item path

pub const LIGHT_BLUE: Rgb<
u8
>;

# Constant LIGHT_ CORAL Copy item path

Path: led_strip/colors/constant.LIGHT_CORAL.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ CORAL Copy item path

pub const LIGHT_CORAL: Rgb<
u8
>;

# Constant LIGHT_ CYAN Copy item path

Path: led_strip/colors/constant.LIGHT_CYAN.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ CYAN Copy item path

pub const LIGHT_CYAN: Rgb<
u8
>;

# Constant LIGHT_ GOLDENROD_ YELLOW Copy item path

Path: led_strip/colors/constant.LIGHT_GOLDENROD_YELLOW.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ GOLDENROD_ YELLOW Copy item path

pub const LIGHT_GOLDENROD_YELLOW: Rgb<
u8
>;

# Constant LIGHT_ GRAY Copy item path

Path: led_strip/colors/constant.LIGHT_GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ GRAY Copy item path

pub const LIGHT_GRAY: Rgb<
u8
>;

# Constant LIGHT_ GREEN Copy item path

Path: led_strip/colors/constant.LIGHT_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ GREEN Copy item path

pub const LIGHT_GREEN: Rgb<
u8
>;

# Constant LIGHT_ PINK Copy item path

Path: led_strip/colors/constant.LIGHT_PINK.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ PINK Copy item path

pub const LIGHT_PINK: Rgb<
u8
>;

# Constant LIGHT_ SALMON Copy item path

Path: led_strip/colors/constant.LIGHT_SALMON.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ SALMON Copy item path

pub const LIGHT_SALMON: Rgb<
u8
>;

# Constant LIGHT_ SEA_ GREEN Copy item path

Path: led_strip/colors/constant.LIGHT_SEA_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ SEA_ GREEN Copy item path

pub const LIGHT_SEA_GREEN: Rgb<
u8
>;

# Constant LIGHT_ SKY_ BLUE Copy item path

Path: led_strip/colors/constant.LIGHT_SKY_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ SKY_ BLUE Copy item path

pub const LIGHT_SKY_BLUE: Rgb<
u8
>;

# Constant LIGHT_ SLATE_ GRAY Copy item path

Path: led_strip/colors/constant.LIGHT_SLATE_GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ SLATE_ GRAY Copy item path

pub const LIGHT_SLATE_GRAY: Rgb<
u8
>;

# Constant LIGHT_ STEEL_ BLUE Copy item path

Path: led_strip/colors/constant.LIGHT_STEEL_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ STEEL_ BLUE Copy item path

pub const LIGHT_STEEL_BLUE: Rgb<
u8
>;

# Constant LIGHT_ YELLOW Copy item path

Path: led_strip/colors/constant.LIGHT_YELLOW.html

device_envoy_rp

::

led_strip :: colors

# Constant LIGHT_ YELLOW Copy item path

pub const LIGHT_YELLOW: Rgb<
u8
>;

# Constant LIME Copy item path

Path: led_strip/colors/constant.LIME.html

device_envoy_rp

::

led_strip :: colors

# Constant LIME Copy item path

pub const LIME: Rgb<
u8
>;

# Constant LIME_ GREEN Copy item path

Path: led_strip/colors/constant.LIME_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant LIME_ GREEN Copy item path

pub const LIME_GREEN: Rgb<
u8
>;

# Constant LINEN Copy item path

Path: led_strip/colors/constant.LINEN.html

device_envoy_rp

::

led_strip :: colors

# Constant LINEN Copy item path

pub const LINEN: Rgb<
u8
>;

# Constant MAGENTA Copy item path

Path: led_strip/colors/constant.MAGENTA.html

device_envoy_rp

::

led_strip :: colors

# Constant MAGENTA Copy item path

pub const MAGENTA: Rgb<
u8
>;

# Constant MAROON Copy item path

Path: led_strip/colors/constant.MAROON.html

device_envoy_rp

::

led_strip :: colors

# Constant MAROON Copy item path

pub const MAROON: Rgb<
u8
>;

# Constant MEDIUM_ AQUAMARINE Copy item path

Path: led_strip/colors/constant.MEDIUM_AQUAMARINE.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ AQUAMARINE Copy item path

pub const MEDIUM_AQUAMARINE: Rgb<
u8
>;

# Constant MEDIUM_ BLUE Copy item path

Path: led_strip/colors/constant.MEDIUM_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ BLUE Copy item path

pub const MEDIUM_BLUE: Rgb<
u8
>;

# Constant MEDIUM_ ORCHID Copy item path

Path: led_strip/colors/constant.MEDIUM_ORCHID.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ ORCHID Copy item path

pub const MEDIUM_ORCHID: Rgb<
u8
>;

# Constant MEDIUM_ PURPLE Copy item path

Path: led_strip/colors/constant.MEDIUM_PURPLE.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ PURPLE Copy item path

pub const MEDIUM_PURPLE: Rgb<
u8
>;

# Constant MEDIUM_ SEA_ GREEN Copy item path

Path: led_strip/colors/constant.MEDIUM_SEA_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ SEA_ GREEN Copy item path

pub const MEDIUM_SEA_GREEN: Rgb<
u8
>;

# Constant MEDIUM_ SLATE_ BLUE Copy item path

Path: led_strip/colors/constant.MEDIUM_SLATE_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ SLATE_ BLUE Copy item path

pub const MEDIUM_SLATE_BLUE: Rgb<
u8
>;

# Constant MEDIUM_ SPRING_ GREEN Copy item path

Path: led_strip/colors/constant.MEDIUM_SPRING_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ SPRING_ GREEN Copy item path

pub const MEDIUM_SPRING_GREEN: Rgb<
u8
>;

# Constant MEDIUM_ TURQUOISE Copy item path

Path: led_strip/colors/constant.MEDIUM_TURQUOISE.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ TURQUOISE Copy item path

pub const MEDIUM_TURQUOISE: Rgb<
u8
>;

# Constant MEDIUM_ VIOLET_ RED Copy item path

Path: led_strip/colors/constant.MEDIUM_VIOLET_RED.html

device_envoy_rp

::

led_strip :: colors

# Constant MEDIUM_ VIOLET_ RED Copy item path

pub const MEDIUM_VIOLET_RED: Rgb<
u8
>;

# Constant MIDNIGHT_ BLUE Copy item path

Path: led_strip/colors/constant.MIDNIGHT_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant MIDNIGHT_ BLUE Copy item path

pub const MIDNIGHT_BLUE: Rgb<
u8
>;

# Constant MINT_ CREAM Copy item path

Path: led_strip/colors/constant.MINT_CREAM.html

device_envoy_rp

::

led_strip :: colors

# Constant MINT_ CREAM Copy item path

pub const MINT_CREAM: Rgb<
u8
>;

# Constant MISTY_ ROSE Copy item path

Path: led_strip/colors/constant.MISTY_ROSE.html

device_envoy_rp

::

led_strip :: colors

# Constant MISTY_ ROSE Copy item path

pub const MISTY_ROSE: Rgb<
u8
>;

# Constant MOCCASIN Copy item path

Path: led_strip/colors/constant.MOCCASIN.html

device_envoy_rp

::

led_strip :: colors

# Constant MOCCASIN Copy item path

pub const MOCCASIN: Rgb<
u8
>;

# Constant NAVAJO_ WHITE Copy item path

Path: led_strip/colors/constant.NAVAJO_WHITE.html

device_envoy_rp

::

led_strip :: colors

# Constant NAVAJO_ WHITE Copy item path

pub const NAVAJO_WHITE: Rgb<
u8
>;

# Constant NAVY Copy item path

Path: led_strip/colors/constant.NAVY.html

device_envoy_rp

::

led_strip :: colors

# Constant NAVY Copy item path

pub const NAVY: Rgb<
u8
>;

# Constant OLD_ LACE Copy item path

Path: led_strip/colors/constant.OLD_LACE.html

device_envoy_rp

::

led_strip :: colors

# Constant OLD_ LACE Copy item path

pub const OLD_LACE: Rgb<
u8
>;

# Constant OLIVE Copy item path

Path: led_strip/colors/constant.OLIVE.html

device_envoy_rp

::

led_strip :: colors

# Constant OLIVE Copy item path

pub const OLIVE: Rgb<
u8
>;

# Constant OLIVE_ DRAB Copy item path

Path: led_strip/colors/constant.OLIVE_DRAB.html

device_envoy_rp

::

led_strip :: colors

# Constant OLIVE_ DRAB Copy item path

pub const OLIVE_DRAB: Rgb<
u8
>;

# Constant ORANGE Copy item path

Path: led_strip/colors/constant.ORANGE.html

device_envoy_rp

::

led_strip :: colors

# Constant ORANGE Copy item path

pub const ORANGE: Rgb<
u8
>;

# Constant ORANGE_ RED Copy item path

Path: led_strip/colors/constant.ORANGE_RED.html

device_envoy_rp

::

led_strip :: colors

# Constant ORANGE_ RED Copy item path

pub const ORANGE_RED: Rgb<
u8
>;

# Constant ORCHID Copy item path

Path: led_strip/colors/constant.ORCHID.html

device_envoy_rp

::

led_strip :: colors

# Constant ORCHID Copy item path

pub const ORCHID: Rgb<
u8
>;

# Constant PALE_ GOLDENROD Copy item path

Path: led_strip/colors/constant.PALE_GOLDENROD.html

device_envoy_rp

::

led_strip :: colors

# Constant PALE_ GOLDENROD Copy item path

pub const PALE_GOLDENROD: Rgb<
u8
>;

# Constant PALE_ GREEN Copy item path

Path: led_strip/colors/constant.PALE_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant PALE_ GREEN Copy item path

pub const PALE_GREEN: Rgb<
u8
>;

# Constant PALE_ TURQUOISE Copy item path

Path: led_strip/colors/constant.PALE_TURQUOISE.html

device_envoy_rp

::

led_strip :: colors

# Constant PALE_ TURQUOISE Copy item path

pub const PALE_TURQUOISE: Rgb<
u8
>;

# Constant PALE_ VIOLET_ RED Copy item path

Path: led_strip/colors/constant.PALE_VIOLET_RED.html

device_envoy_rp

::

led_strip :: colors

# Constant PALE_ VIOLET_ RED Copy item path

pub const PALE_VIOLET_RED: Rgb<
u8
>;

# Constant PAPAYA_ WHIP Copy item path

Path: led_strip/colors/constant.PAPAYA_WHIP.html

device_envoy_rp

::

led_strip :: colors

# Constant PAPAYA_ WHIP Copy item path

pub const PAPAYA_WHIP: Rgb<
u8
>;

# Constant PEACH_ PUFF Copy item path

Path: led_strip/colors/constant.PEACH_PUFF.html

device_envoy_rp

::

led_strip :: colors

# Constant PEACH_ PUFF Copy item path

pub const PEACH_PUFF: Rgb<
u8
>;

# Constant PERU Copy item path

Path: led_strip/colors/constant.PERU.html

device_envoy_rp

::

led_strip :: colors

# Constant PERU Copy item path

pub const PERU: Rgb<
u8
>;

# Constant PINK Copy item path

Path: led_strip/colors/constant.PINK.html

device_envoy_rp

::

led_strip :: colors

# Constant PINK Copy item path

pub const PINK: Rgb<
u8
>;

# Constant PLUM Copy item path

Path: led_strip/colors/constant.PLUM.html

device_envoy_rp

::

led_strip :: colors

# Constant PLUM Copy item path

pub const PLUM: Rgb<
u8
>;

# Constant POWDER_ BLUE Copy item path

Path: led_strip/colors/constant.POWDER_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant POWDER_ BLUE Copy item path

pub const POWDER_BLUE: Rgb<
u8
>;

# Constant PURPLE Copy item path

Path: led_strip/colors/constant.PURPLE.html

device_envoy_rp

::

led_strip :: colors

# Constant PURPLE Copy item path

pub const PURPLE: Rgb<
u8
>;

# Constant RED Copy item path

Path: led_strip/colors/constant.RED.html

device_envoy_rp

::

led_strip :: colors

# Constant RED Copy item path

pub const RED: Rgb<
u8
>;

# Constant ROSY_ BROWN Copy item path

Path: led_strip/colors/constant.ROSY_BROWN.html

device_envoy_rp

::

led_strip :: colors

# Constant ROSY_ BROWN Copy item path

pub const ROSY_BROWN: Rgb<
u8
>;

# Constant ROYAL_ BLUE Copy item path

Path: led_strip/colors/constant.ROYAL_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant ROYAL_ BLUE Copy item path

pub const ROYAL_BLUE: Rgb<
u8
>;

# Constant SADDLE_ BROWN Copy item path

Path: led_strip/colors/constant.SADDLE_BROWN.html

device_envoy_rp

::

led_strip :: colors

# Constant SADDLE_ BROWN Copy item path

pub const SADDLE_BROWN: Rgb<
u8
>;

# Constant SALMON Copy item path

Path: led_strip/colors/constant.SALMON.html

device_envoy_rp

::

led_strip :: colors

# Constant SALMON Copy item path

pub const SALMON: Rgb<
u8
>;

# Constant SANDY_ BROWN Copy item path

Path: led_strip/colors/constant.SANDY_BROWN.html

device_envoy_rp

::

led_strip :: colors

# Constant SANDY_ BROWN Copy item path

pub const SANDY_BROWN: Rgb<
u8
>;

# Constant SEASHELL Copy item path

Path: led_strip/colors/constant.SEASHELL.html

device_envoy_rp

::

led_strip :: colors

# Constant SEASHELL Copy item path

pub const SEASHELL: Rgb<
u8
>;

# Constant SEA_ GREEN Copy item path

Path: led_strip/colors/constant.SEA_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant SEA_ GREEN Copy item path

pub const SEA_GREEN: Rgb<
u8
>;

# Constant SIENNA Copy item path

Path: led_strip/colors/constant.SIENNA.html

device_envoy_rp

::

led_strip :: colors

# Constant SIENNA Copy item path

pub const SIENNA: Rgb<
u8
>;

# Constant SILVER Copy item path

Path: led_strip/colors/constant.SILVER.html

device_envoy_rp

::

led_strip :: colors

# Constant SILVER Copy item path

pub const SILVER: Rgb<
u8
>;

# Constant SKY_ BLUE Copy item path

Path: led_strip/colors/constant.SKY_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant SKY_ BLUE Copy item path

pub const SKY_BLUE: Rgb<
u8
>;

# Constant SLATE_ BLUE Copy item path

Path: led_strip/colors/constant.SLATE_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant SLATE_ BLUE Copy item path

pub const SLATE_BLUE: Rgb<
u8
>;

# Constant SLATE_ GRAY Copy item path

Path: led_strip/colors/constant.SLATE_GRAY.html

device_envoy_rp

::

led_strip :: colors

# Constant SLATE_ GRAY Copy item path

pub const SLATE_GRAY: Rgb<
u8
>;

# Constant SNOW Copy item path

Path: led_strip/colors/constant.SNOW.html

device_envoy_rp

::

led_strip :: colors

# Constant SNOW Copy item path

pub const SNOW: Rgb<
u8
>;

# Constant SPRING_ GREEN Copy item path

Path: led_strip/colors/constant.SPRING_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant SPRING_ GREEN Copy item path

pub const SPRING_GREEN: Rgb<
u8
>;

# Constant STEEL_ BLUE Copy item path

Path: led_strip/colors/constant.STEEL_BLUE.html

device_envoy_rp

::

led_strip :: colors

# Constant STEEL_ BLUE Copy item path

pub const STEEL_BLUE: Rgb<
u8
>;

# Constant TAN Copy item path

Path: led_strip/colors/constant.TAN.html

device_envoy_rp

::

led_strip :: colors

# Constant TAN Copy item path

pub const TAN: Rgb<
u8
>;

# Constant TEAL Copy item path

Path: led_strip/colors/constant.TEAL.html

device_envoy_rp

::

led_strip :: colors

# Constant TEAL Copy item path

pub const TEAL: Rgb<
u8
>;

# Constant THISTLE Copy item path

Path: led_strip/colors/constant.THISTLE.html

device_envoy_rp

::

led_strip :: colors

# Constant THISTLE Copy item path

pub const THISTLE: Rgb<
u8
>;

# Constant TOMATO Copy item path

Path: led_strip/colors/constant.TOMATO.html

device_envoy_rp

::

led_strip :: colors

# Constant TOMATO Copy item path

pub const TOMATO: Rgb<
u8
>;

# Constant TURQUOISE Copy item path

Path: led_strip/colors/constant.TURQUOISE.html

device_envoy_rp

::

led_strip :: colors

# Constant TURQUOISE Copy item path

pub const TURQUOISE: Rgb<
u8
>;

# Constant VIOLET Copy item path

Path: led_strip/colors/constant.VIOLET.html

device_envoy_rp

::

led_strip :: colors

# Constant VIOLET Copy item path

pub const VIOLET: Rgb<
u8
>;

# Constant WHEAT Copy item path

Path: led_strip/colors/constant.WHEAT.html

device_envoy_rp

::

led_strip :: colors

# Constant WHEAT Copy item path

pub const WHEAT: Rgb<
u8
>;

# Constant WHITE Copy item path

Path: led_strip/colors/constant.WHITE.html

device_envoy_rp

::

led_strip :: colors

# Constant WHITE Copy item path

pub const WHITE: Rgb<
u8
>;

Expand description

See https://en.wikipedia.org/wiki/Web_colors

# Constant WHITE_ SMOKE Copy item path

Path: led_strip/colors/constant.WHITE_SMOKE.html

device_envoy_rp

::

led_strip :: colors

# Constant WHITE_ SMOKE Copy item path

pub const WHITE_SMOKE: Rgb<
u8
>;

# Constant YELLOW Copy item path

Path: led_strip/colors/constant.YELLOW.html

device_envoy_rp

::

led_strip :: colors

# Constant YELLOW Copy item path

pub const YELLOW: Rgb<
u8
>;

# Constant YELLOW_ GREEN Copy item path

Path: led_strip/colors/constant.YELLOW_GREEN.html

device_envoy_rp

::

led_strip :: colors

# Constant YELLOW_ GREEN Copy item path

pub const YELLOW_GREEN: Rgb<
u8
>;

# Module colors Copy item path

Path: led_strip/colors/index.html

device_envoy_rp

::

led_strip

# Module colors Copy item path

Expand description

Predefined RGB8 color constants (CSS/Web names).

GREEN is (0, 128, 0) ; LIME is (0, 255, 0) .

## Constants §

ALICE_ BLUE ANTINQUE_ WHITE AQUA AQUAMARINE AZURE BEIGE BISQUE BLACK BLANCHED_ ALMOND BLUE BLUE_ VIOLET BROWN BURLYWOOD CADET_ BLUE CHARTREUSE CHOCOLATE CORAL CORNFLOWER_ BLUE CORNSILK CRIMSON CYAN DARK_ BLUE DARK_ CYAN DARK_ GOLDENROD DARK_ GRAY DARK_ GREEN DARK_ KHAKI DARK_ MAGENTA DARK_ OLIVE_ GREEN DARK_ ORANGE DARK_ ORCHID DARK_ RED DARK_ SALMON DARK_ SEA_ GREEN DARK_ SLATE_ BLUE DARK_ SLATE_ GRAY DARK_ TURQUOISE DARK_ VIOLET DEEP_ PINK DEEP_ SKY_ BLUE DIM_ GRAY DODGER_ BLUE FIREBRICK FLORAL_ WHITE FOREST_ GREEN FUCHSIA GAINSBORO GHOST_ WHITE GOLD GOLDENROD GRAY GREEN GREEN_ YELLOW HONEYDEW HOT_ PINK INDIAN_ RED INDIGO IVORY KHAKI LAVENDER LAVENDER_ BLUSH LAWN_ GREEN LEMON_ CHIFFON LIGHT_ BLUE LIGHT_ CORAL LIGHT_ CYAN LIGHT_ GOLDENROD_ YELLOW LIGHT_ GRAY LIGHT_ GREEN LIGHT_ PINK LIGHT_ SALMON LIGHT_ SEA_ GREEN LIGHT_ SKY_ BLUE LIGHT_ SLATE_ GRAY LIGHT_ STEEL_ BLUE LIGHT_ YELLOW LIME LIME_ GREEN LINEN MAGENTA MAROON MEDIUM_ AQUAMARINE MEDIUM_ BLUE MEDIUM_ ORCHID MEDIUM_ PURPLE MEDIUM_ SEA_ GREEN MEDIUM_ SLATE_ BLUE MEDIUM_ SPRING_ GREEN MEDIUM_ TURQUOISE MEDIUM_ VIOLET_ RED MIDNIGHT_ BLUE MINT_ CREAM MISTY_ ROSE MOCCASIN NAVAJO_ WHITE NAVY OLD_ LACE OLIVE OLIVE_ DRAB ORANGE ORANGE_ RED ORCHID PALE_ GOLDENROD PALE_ GREEN PALE_ TURQUOISE PALE_ VIOLET_ RED PAPAYA_ WHIP PEACH_ PUFF PERU PINK PLUM POWDER_ BLUE PURPLE RED ROSY_ BROWN ROYAL_ BLUE SADDLE_ BROWN SALMON SANDY_ BROWN SEASHELL SEA_ GREEN SIENNA SILVER SKY_ BLUE SLATE_ BLUE SLATE_ GRAY SNOW SPRING_ GREEN STEEL_ BLUE TAN TEAL THISTLE TOMATO TURQUOISE VIOLET WHEAT WHITE See https://en.wikipedia.org/wiki/Web_colors WHITE_ SMOKE YELLOW YELLOW_ GREEN

# Enum Current Copy item path

Path: led_strip/enum.Current.html

device_envoy_rp

::

led_strip

# Enum Current Copy item path

pub enum Current {
    Milliamps(
u32
),
    Unlimited,
}

Expand description

Current budget for a single LED strip.

Used by the led_strip! macro to derive MAX_BRIGHTNESS on the generated struct.

## Variants §

§

### Milliamps( u32 )

Current limit in milliamps.

§

### Unlimited

No limit — full brightness.

## Trait Implementations §

§

### impl Clone for Current

§

#### fn clone (&self) -> Current

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Current

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for Current

§

#### fn default () -> Current

Returns the “default value” for a type.

Read more

§

### impl PartialEq for Current

§

#### fn eq (&self, other: & Current ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for Current

§

### impl Eq for Current

§

### impl StructuralPartialEq for Current

## Auto Trait Implementations §

§

### impl Freeze for Current

§

### impl RefUnwindSafe for Current

§

### impl Send for Current

§

### impl Sync for Current

§

### impl Unpin for Current

§

### impl UnwindSafe for Current

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Enum Gamma Copy item path

Path: led_strip/enum.Gamma.html

device_envoy_rp

::

led_strip

# Enum Gamma Copy item path

pub enum Gamma {
    Linear,
    Srgb,
    SmartLeds,
}

Expand description

Gamma correction configuration for LED strips.

See the platform crate’s led_strip! macro documentation for usage and context.

## Variants §

§

### Linear

No correction; raw LED PWM values.

§

### Srgb

Perceptual sRGB semantics (gamma 2.2). Preserves named color intent.

§

### SmartLeds

Compatibility with historical smart_leds::gamma() curve (2.8).

## Trait Implementations §

§

### impl Clone for Gamma

§

#### fn clone (&self) -> Gamma

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Gamma

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for Gamma

§

#### fn default () -> Gamma

Returns the “default value” for a type.

Read more

§

### impl PartialEq for Gamma

§

#### fn eq (&self, other: & Gamma ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for Gamma

§

### impl Eq for Gamma

§

### impl StructuralPartialEq for Gamma

## Auto Trait Implementations §

§

### impl Freeze for Gamma

§

### impl RefUnwindSafe for Gamma

§

### impl Send for Gamma

§

### impl Sync for Gamma

§

### impl Unpin for Gamma

§

### impl UnwindSafe for Gamma

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module led_ strip Copy item path

Path: led_strip/index.html

device_envoy_rp

# Module led_ strip Copy item path

Source

Expand description

A device abstraction for 1-dimensional NeoPixel-style (WS2812) LED strips. For 2-dimensional panels, see the led2d module.

This page provides the primary documentation and examples for programming LED strips. The device abstraction supports pixel patterns and animation on the LED strip.

After reading the examples below, see also:

led_strip! — Macro to generate an LED strip struct type (includes syntax details).

LedStrip — Core trait defining the LED strip API surface.

LedStripGenerated — Sample generated strip type showing the constructor path.

Frame1d — 1D pixel array used to describe LED strip patterns.

led_strips! — Alternative macro to share a PIO resource with other strips or panels (includes examples).

## § Example: Write a Single 1-Dimensional Frame

In this example, we set every other LED to blue and gray. Here, the generated struct type is named LedStripSimple .

use
device_envoy_rp::{
Result
, led_strip::{LedStrip
as _
, Frame1d, colors}};
use
device_envoy_rp::led_strip;
// Define LedStripSimple, a struct type for an 8-LED strip on PIN_0.
led_strip!
{
    LedStripSimple {
        pin: PIN_0,
// GPIO pin for LED data
len:
8
,
// 8 LEDs
        // other inputs set to their defaults
}
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
// Create a LedStripSimple instance.
let
led_strip_simple = LedStripSimple::new(p.PIN_0, p.PIO0, p.DMA_CH0, spawner)
?
;
// Create and write a frame with alternating blue and gray pixels.
let
mut
frame = Frame1d::new();
for
pixel_index
in
0
..LedStripSimple::LEN {
// Directly index into the frame buffer.
frame[pixel_index] = [colors::BLUE, colors::GRAY][pixel_index %
2
];
    }
// Display the frame on the LED strip (until replaced).
led_strip_simple.write_frame(frame);

    pending().
await
// run forever
}

## § Example: Animate a Sequence

This example animates a 96-LED strip through red, green, and blue frames, cycling continuously. Here, the generated struct type is named LedStripAnimated .

use
device_envoy_rp::{
Result
, led_strip::{LedStrip
as _
, Current, Frame1d, Gamma, colors}};
use
device_envoy_rp::led_strip;
// Define LedStripAnimated, a struct type for a 96-LED strip on PIN_4.
// We change some defaults including setting a 1A power budget and disabling gamma correction.
led_strip!
{
pub
(
self
) LedStripAnimated {
// Can provide a visibility modifier
pin: PIN_4,
// GPIO pin for LED data
len:
96
,
// 96 LEDs
pio: PIO1,
// Use PIO resource 1
dma: DMA_CH3,
// Use DMA channel 3
max_current: Current::Milliamps(
1000
),
// 1A power budget
gamma: Gamma::Linear,
// No color correction
max_frames:
3
,
// Up to 3 animation frames
}
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
led_strip_animated = LedStripAnimated::new(p.PIN_4, p.PIO1, p.DMA_CH3, spawner)
?
;
// Create a sequence of frames and durations and then animate them (looping, until replaced).
let
frame_duration = embassy_time::Duration::from_millis(
300
);
    led_strip_animated.animate([
        (Frame1d::filled(colors::RED), frame_duration),
        (Frame1d::filled(colors::GREEN), frame_duration),
        (Frame1d::filled(colors::BLUE), frame_duration),
    ]);

    pending().
await
// run forever
}

## Modules §

colors Predefined RGB8 color constants (CSS/Web names). led_ strip_ generated Module containing LedStripGenerated , the sample struct type generated by the led_strip! macro.

## Macros §

led_ strip Macro to generate an LED-strip struct type (includes syntax details). led_ strips Macro to generate multiple LED strip and panel struct types that share a single PIO resource (includes syntax details).

## Structs §

Frame1d 1D pixel array used to describe LED strip patterns. Rgb888 Rgb888 color.

## Enums §

Current Current budget for a single LED strip. Gamma Gamma correction configuration for LED strips.

## Traits §

LedStrip Platform-agnostic LED strip device contract. ToRgb8 Convert a color to RGB8 for LED strip rendering. ToRgb888 Convert a color to Rgb888 for embedded-graphics rendering.

## Type Aliases §

RGB8 8-bit RGB

# Module led_ strip_ generated Copy item path

Path: led_strip/led_strip_generated/index.html

device_envoy_rp

::

led_strip

# Module led_ strip_ generated Copy item path

Source

Expand description

Module containing LedStripGenerated , the sample struct type generated by the led_strip! macro.

Auto-generated.

## Structs §

LedStrip Generated Sample struct type generated by the led_strip! macro.

# Struct LedStrip Generated Copy item path

Path: led_strip/led_strip_generated/struct.LedStripGenerated.html

device_envoy_rp

::

led_strip :: led_strip_generated

# Struct LedStrip Generated Copy item path

Source

pub struct LedStripGenerated;

Expand description

Sample struct type generated by the led_strip! macro.

This page exists to show constructor, constants, and trait methods in one place. For narrative examples, see the led_strip module.

Auto-generated.

## Implementations §

Source

§

### impl LedStripGenerated

Source

#### pub const LEN : usize = 8

Number of pixels in this strip.

Source

#### pub const MAX_FRAMES : usize = 16

Maximum number of animation frames.

Source

#### pub const MAX_BRIGHTNESS : u8

Maximum brightness after current limiting.

Source

#### pub fn new ( pin: Peri<'static, PIN_0>, pio: Peri<'static, PIO0>, dma: Peri<'static, DMA_CH0>, spawner: Spawner, ) -> Result <&'static Self>

Create a new LED strip instance of the struct type defined by led_strip! .

See the led_strip module docs for usage examples.

## Trait Implementations §

Source

§

### impl LedStrip <8> for LedStripGenerated

Source

§

#### const LEN : usize = Self::LEN

Number of LEDs in this strip.

Source

§

#### const MAX_FRAMES : usize = Self::MAX_FRAMES

Maximum number of animation frames allowed.

Source

§

#### const MAX_BRIGHTNESS : u8 = Self::MAX_BRIGHTNESS

Maximum brightness level, automatically limited by the power budget.

Source

§

#### fn write_frame (&self, frame: Frame1d <8>)

Write a frame to the LED strip.

Read more

Source

§

#### fn animate <I>(&self, frames: I) where I: IntoIterator , I:: Item : Borrow <( Frame1d <8>, Duration)>,

Animate frames on the LED strip.

Read more

## Auto Trait Implementations §

§

### impl Freeze for LedStripGenerated

§

### impl RefUnwindSafe for LedStripGenerated

§

### impl Send for LedStripGenerated

§

### impl Sync for LedStripGenerated

§

### impl Unpin for LedStripGenerated

§

### impl UnwindSafe for LedStripGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# macro.led_strip!.html

Path: led_strip/macro.led_strip!.html

Redirecting to macro.led_strip.html ...

# Macro led_ strip Copy item path

Path: led_strip/macro.led_strip.html

device_envoy_rp

::

led_strip

# Macro led_ strip Copy item path

Source

macro_rules! led_strip {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate an LED-strip struct type (includes syntax details).

See the led_strip module documentation for usage examples.

Syntax:

led_strip! {
    [<visibility>] <Name> {
        pin: <pin_ident>,
        len: <usize_expr>,
        pio: <pio_ident>,               // optional
        dma: <dma_ident>,               // optional
        max_current: <Current_expr>,    // optional
        gamma: <Gamma_expr>,            // optional
        max_frames: <usize_expr>,       // optional
    }
}

Required fields:

pin — GPIO pin for LED data

len — Number of LEDs

Optional fields:

pio — PIO resource (default: PIO0 )

dma — DMA channel (default: DMA_CH0 )

max_current — Electrical current budget (default: 250 mA)

gamma — Color curve (default: Gamma::Srgb )

max_frames — Maximum number of animation frames (default: 16 frames)

max_frames = 0 disables animation and allocates no frame storage; write_frame() is still supported.

## § Current Limiting

The max_current field automatically scales brightness to stay within your electrical current budget.

Each WS2812 LED is assumed to draw 60 mA at full brightness. For example:

16 LEDs × 60 mA = 960 mA at full brightness

With max_current: Current::Milliamps(1000) , all LEDs fit at 100% brightness

With the default electrical current limit (250 mA), the generated MAX_BRIGHTNESS limits LEDs to ~26% brightness

The electrical current limit is compiled into a lookup table at device initialization, so it has no per-frame runtime cost.

Powering LEDs from a board 5 V pin: On Pico, this is pin 40 ( VBUS ). On ESP32-C6 D6 and ESP32-S3 boards, this is usually a header pin labeled 5V or VBUS . These rails are typically USB pass-through and have practical current limits, so avoid heavy loads through the board itself. Small LED panels (a few hundred mA) are usually fine with a solid USB supply; for larger loads (around 1 A+), use a separate 5 V supply and share ground with the board. If your board labels the pin 5Vin , treat it as a power-input rail (not a general-purpose 5 V output for peripherals).

## § Color Correction (Gamma)

The gamma field applies a color response curve to make colors look more natural:

Gamma::Linear — No correction (raw values)

Gamma::Srgb — Perceptual sRGB semantics (default; preserves named color constants)

Gamma::SmartLeds — smart_leds::gamma() compatibility (2.8)

The gamma curve is compiled into a lookup table at device initialization, so it has no per-frame runtime cost.

## § Related Macros

led_strips! — Alternative macro to share a PIO resource with other strips or panels (includes examples)

led2d! — For 2-dimensional LED panels

# macro.led_strips!.html

Path: led_strip/macro.led_strips!.html

Redirecting to macro.led_strips.html ...

# Macro led_ strips Copy item path

Path: led_strip/macro.led_strips.html

device_envoy_rp

::

led_strip

# Macro led_ strips Copy item path

Source

macro_rules! led_strips {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate multiple LED strip and panel struct types that share a single PIO resource (includes syntax details).

This page provides the primary documentation and examples for configuring strip/panel groups that share a PIO resource.

Syntax:

led_strips! {
    pio: <pio_ident>, // optional
    [<visibility>] <GroupName> {
        <MemberName>: {
            pin: <pin_ident>,
            len: <usize_expr>,
            max_current: <Current_expr>,
            gamma: <Gamma_expr>,          // optional
            max_frames: <usize_expr>,     // optional
            dma: <dma_ident>,             // optional
            led2d: {                      // optional (panel mode)
                led_layout: <LedLayout_expr>,
                font: <Led2dFont_expr>,
            }
        },
        // ...more members...
    }
}

After reading the examples below, see also:

LedStrip — LED strip trait defining methods and associated constants

Led2d — LED panel trait defining methods and associated constants

led_strip! — Alternative macro to generate a single LED strip type. Consumes a PIO resource.

led2d! — Alternative macro to generate a single LED panel type. Consumes a PIO resource.

Use this macro when your project has multiple LED strips or panels that should share a single PIO resource. If you only need a single strip or panel, prefer led_strip! or led2d! for simpler configuration.

We’ll start with a complete example below, then describe the required and optional fields in detail.

## § Example: Connect Three LED Strips/Panels to One PIO Resource

This example creates three LED strips/panels on GPIO0, GPIO3, and GPIO4, all sharing PIO0. It demonstrates showing a pattern on the first two strips and animating text on the 2D panel.

use
device_envoy_rp::{
Result
, led2d::Frame2d, led2d::Led2dFont, led2d::layout::LedLayout, led_strip::{LedStrip
as _
, Current, Frame1d, Gamma, colors, led_strips}};
use
device_envoy_rp::led2d::Led2d
as _
;
use
embassy_time::Duration;
// Our 2D panel is two 12x4 panels stacked vertically.
const
LED_LAYOUT_12X4: LedLayout<
48
,
12
,
4
> = LedLayout::serpentine_column_major();
const
LED_LAYOUT_12X8: LedLayout<
96
,
12
,
8
> = LED_LAYOUT_12X4.combine_v(LED_LAYOUT_12X4);
const
LED_LAYOUT_12X8_ROTATED: LedLayout<
96
,
8
,
12
> = LED_LAYOUT_12X8.rotate_cw();
led_strips!
{
    pio: PIO0,
// Optional; defaults to PIO0.
LedStrips0 {
// Name for this group of LED strips/panels. Can provide visibility modifier
        // 1. a 8-LED strip on GPIO0
Gpio0LedStrip: {
// Exact struct name for this strip.
pin: PIN_0,
// GPIO pin for LED data signal.
len:
8
,
// 8 LEDs on this strip.
max_current: Current::Milliamps(
25
),
// Every strip/panel requires an electrical current budget.
},
// 2. a 48-LED strip on GPIO3
Gpio3LedStrip: {
            pin: PIN_3,
            len:
48
,
            max_current: Current::Milliamps(
75
),
            gamma: Gamma::Srgb,
// Optional; color correction (default, Gamma::Srgb).
max_frames:
1
,
// Optional; default 16 frames.
dma: DMA_CH11,
// Optional; auto-assigned by strip order.
},
// 3. a 96-LED 2D panel on GPIO4
Gpio4Led2d: {
            pin: PIN_4,
            len:
96
,
            max_current: Current::Milliamps(
250
),
            max_frames:
2
,
            led2d: {
// Optional panel configuration for 2D displays.
led_layout: LED_LAYOUT_12X8_ROTATED,
// Two 12×4 panels stacked and rotated.
font: Led2dFont::Font4x6Trim,
// 4x6 pixel font without the usual 1 pixel spacing.
}
        },
    }
}
async fn
example(spawner: Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
// Create instances of two LED strips and one panel.
let
(gpio0_led_strip, gpio3_led_strip, gpio4_led2d) = LedStrips0::new(
        p.PIO0, p.PIN_0, p.DMA_CH0, p.PIN_3, p.DMA_CH11, p.PIN_4, p.DMA_CH2, spawner,
    )
?
;
info!
(
"Setting GPIO0 to white, GPIO3 to alternating blue/gray, GPIO4 to Go Go animation"
);
// Turn on all-white on GPIO0 strip.
let
frame_gpio0 = Frame1d::filled(colors::WHITE);
    gpio0_led_strip.write_frame(frame_gpio0);
// Display the frame (until replaced)

    // Alternate blue/gray on GPIO3 strip.
let
mut
frame_gpio3 = Frame1d::new();
for
pixel_index
in
0
..Gpio3LedStrip::LEN {
        frame_gpio3[pixel_index] = [colors::BLUE, colors::GRAY][pixel_index %
2
];
    }
    gpio3_led_strip.write_frame(frame_gpio3);
// Display the frame (until replaced)

    // Animate "Go Go" text on GPIO4 2D panel.
let
mut
frame_go_top = Frame2d::new();
    gpio4_led2d.write_text_to_frame(
"Go"
,
&
[],
&mut
frame_go_top);
let
mut
frame_go_bottom = Frame2d::new();
    gpio4_led2d.write_text_to_frame(
"\nGo"
,
&
[colors::HOT_PINK, colors::LIME],
&mut
frame_go_bottom,
    );
let
frame_duration = Duration::from_secs(
1
);
    gpio4_led2d
        .animate([
            (frame_go_top, frame_duration),
            (frame_go_bottom, frame_duration),
        ]);
// Loop animation (until replaced)
pending().
await
// run forever
}

## § Configuration

### § Shared PIO Resource

pio — PIO peripheral to use (default: PIO0 ). This consumes one PIO resource for the group and is shared across all strips/panels in the macro invocation.

### § Required Fields per Strip/Panel

pin — GPIO pin for LED data

len — Number of LEDs (pixels)

max_current — Electrical current budget per strip (required; no default)

### § Optional Fields per Strip/Panel

dma — DMA channel (default: auto-assigned by strip order)

gamma — Gamma correction curve (default: Gamma::Srgb )

max_frames — Maximum number of animation frames (default: 16 frames)

max_frames = 0 disables animation and allocates no frame storage; write_frame() is still supported.

led2d — Marks this strip as a 2D LED panel and enables 2D rendering support (optional, see below). Detailed 2D rendering, examples, and animation support are documented in the led2d module .

### § 2D Panel Configuration ( led2d )

If a strip represents a rectangular LED panel rather than a linear strip, add a led2d configuration block to describe its geometry.

Required fields:

led_layout — Physical layout mapping (defines the panel size). See LedLayout for details.

font — Built-in font for text rendering. See Led2dFont for available fonts. Bring Led2dFont into scope or use a full path like device_envoy_rp::led2d::Led2dFont::Font4x6Trim .

The led_layout value must be a const so its dimensions can be derived at compile time.

## § Capacity and Board Capabilities

The led_strips! macro is designed to fully utilize the PIO resources of supported Pico boards.

Each led_strips! invocation can drive up to 4 LED strips or panels while sharing a single PIO resource. This lets you consolidate multiple LED outputs efficiently instead of consuming one PIO per strip.

Each invocation consumes exactly one PIO resource.

On supported boards, this enables the maximum practical LED capacity:

Pico 1 provides 2 PIO resources , allowing up to 8 LED strips or panels

Pico 2 provides 3 PIO resources , allowing up to 12 LED strips or panels

## § Current Limiting

The max_current field automatically scales brightness to stay within your electrical current budget.

Each WS2812 LED is assumed to draw 60 mA at full brightness. For example:

16 LEDs × 60 mA = 960 mA at full brightness

With max_current: Current::Milliamps(1000) , all LEDs fit at 100% brightness

With the default electrical current limit (250 mA), the generated MAX_BRIGHTNESS limits LEDs to ~26% brightness

The electrical current limit is compiled into a lookup table at device initialization, so it has no per-frame runtime cost.

Powering LEDs from a board 5 V pin: On Pico, this is pin 40 ( VBUS ). On ESP32-C6 D6 and ESP32-S3 boards, this is usually a header pin labeled 5V or VBUS . These rails are typically USB pass-through and have practical current limits, so avoid heavy loads through the board itself. Small LED panels (a few hundred mA) are usually fine with a solid USB supply; for larger loads (around 1 A+), use a separate 5 V supply and share ground with the board. If your board labels the pin 5Vin , treat it as a power-input rail (not a general-purpose 5 V output for peripherals).

## § Color Correction (Gamma)

The gamma field applies a color response curve to make colors look more natural:

Gamma::Linear — No correction (raw values)

Gamma::Srgb — Perceptual sRGB semantics (default; preserves named color constants)

Gamma::SmartLeds — smart_leds::gamma() compatibility (2.8)

The gamma curve is compiled into a lookup table at device initialization, so it has no per-frame runtime cost.

## § Related Macros

led_strip! — For a single 1-dimensional LED strip (includes examples)

led2d! — For 2-dimensional LED panels

# Struct Frame1d Copy item path

Path: led_strip/struct.Frame1d.html

device_envoy_rp

::

led_strip

# Struct Frame1d Copy item path

pub struct Frame1d<const N:
usize
>(pub [Rgb<
u8
>;
N
]);

Expand description

1D pixel array used to describe LED strip patterns.

See the platform crate’s led_strip module documentation for usage examples.

Frames deref to [RGB8; N] , so you can mutate pixels directly before passing them to the generated strip’s write_frame method.

## Tuple Fields §

§ 0: [Rgb< u8 >; N ]

## Implementations §

§

### impl<const N: usize > Frame1d <N>

#### pub const LEN : usize = N

Number of LEDs in this frame.

#### pub const fn new () -> Frame1d <N>

Create a new blank (all-black) frame.

#### pub const fn filled (color: Rgb< u8 >) -> Frame1d <N>

Create a frame filled with a single color.

## Methods from Deref <Target = [Rgb< u8 >; N ]> §

1.57.0 · Source

#### pub fn as_slice (&self) -> & [T]

Returns a slice containing the entire array. Equivalent to &s[..] .

1.57.0 · Source

#### pub fn as_mut_slice (&mut self) -> &mut [T]

Returns a mutable slice containing the entire array. Equivalent to &mut s[..] .

1.77.0 · Source

#### pub fn each_ref (&self) -> [ &T ; N ]

Borrows each element and returns an array of references with the same size as self .

#### § Example

let
floats = [
3.1
,
2.7
, -
1.0
];
let
float_refs: [
&
f64;
3
] = floats.each_ref();
assert_eq!
(float_refs, [
&
3.1
,
&
2.7
,
&
-
1.0
]);

This method is particularly useful if combined with other methods, like map . This way, you can avoid moving the original array if its elements are not Copy .

let
strings = [
"Ferris"
.to_string(),
"♥"
.to_string(),
"Rust"
.to_string()];
let
is_ascii = strings.each_ref().map(|s| s.is_ascii());
assert_eq!
(is_ascii, [
true
,
false
,
true
]);
// We can still access the original array: it has not been moved.
assert_eq!
(strings.len(),
3
);

1.77.0 · Source

#### pub fn each_mut (&mut self) -> [ &mut T ; N ]

Borrows each element mutably and returns an array of mutable references with the same size as self .

#### § Example

let
mut
floats = [
3.1
,
2.7
, -
1.0
];
let
float_refs: [
&mut
f64;
3
] = floats.each_mut();
*
float_refs[
0
] =
0.0
;
assert_eq!
(float_refs, [
&mut
0.0
,
&mut
2.7
,
&mut
-
1.0
]);
assert_eq!
(floats, [
0.0
,
2.7
, -
1.0
]);

Source

#### pub fn split_array_ref <const M: usize >(&self) -> (& [T; M] , & [T] )

🔬 This is a nightly-only experimental API. ( split_array )

Divides one array reference into two at an index.

The first will contain all indices from [0, M) (excluding the index M itself) and the second will contain all indices from [M, N) (excluding the index N itself).

#### § Panics

Panics if M > N .

#### § Examples

#![feature(split_array)]
let
v = [
1
,
2
,
3
,
4
,
5
,
6
];

{
let
(left, right) = v.split_array_ref::<
0
>();
assert_eq!
(left,
&
[]);
assert_eq!
(right,
&
[
1
,
2
,
3
,
4
,
5
,
6
]);
}

{
let
(left, right) = v.split_array_ref::<
2
>();
assert_eq!
(left,
&
[
1
,
2
]);
assert_eq!
(right,
&
[
3
,
4
,
5
,
6
]);
}

{
let
(left, right) = v.split_array_ref::<
6
>();
assert_eq!
(left,
&
[
1
,
2
,
3
,
4
,
5
,
6
]);
assert_eq!
(right,
&
[]);
}

Source

#### pub fn split_array_mut <const M: usize >(&mut self) -> (&mut [T; M] , &mut [T] )

🔬 This is a nightly-only experimental API. ( split_array )

Divides one mutable array reference into two at an index.

The first will contain all indices from [0, M) (excluding the index M itself) and the second will contain all indices from [M, N) (excluding the index N itself).

#### § Panics

Panics if M > N .

#### § Examples

#![feature(split_array)]
let
mut
v = [
1
,
0
,
3
,
0
,
5
,
6
];
let
(left, right) = v.split_array_mut::<
2
>();
assert_eq!
(left,
&mut
[
1
,
0
][..]);
assert_eq!
(right,
&mut
[
3
,
0
,
5
,
6
]);
left[
1
] =
2
;
right[
1
] =
4
;
assert_eq!
(v, [
1
,
2
,
3
,
4
,
5
,
6
]);

Source

#### pub fn rsplit_array_ref <const M: usize >(&self) -> (& [T] , & [T; M] )

🔬 This is a nightly-only experimental API. ( split_array )

Divides one array reference into two at an index from the end.

The first will contain all indices from [0, N - M) (excluding the index N - M itself) and the second will contain all indices from [N - M, N) (excluding the index N itself).

#### § Panics

Panics if M > N .

#### § Examples

#![feature(split_array)]
let
v = [
1
,
2
,
3
,
4
,
5
,
6
];

{
let
(left, right) = v.rsplit_array_ref::<
0
>();
assert_eq!
(left,
&
[
1
,
2
,
3
,
4
,
5
,
6
]);
assert_eq!
(right,
&
[]);
}

{
let
(left, right) = v.rsplit_array_ref::<
2
>();
assert_eq!
(left,
&
[
1
,
2
,
3
,
4
]);
assert_eq!
(right,
&
[
5
,
6
]);
}

{
let
(left, right) = v.rsplit_array_ref::<
6
>();
assert_eq!
(left,
&
[]);
assert_eq!
(right,
&
[
1
,
2
,
3
,
4
,
5
,
6
]);
}

Source

#### pub fn rsplit_array_mut <const M: usize >(&mut self) -> (&mut [T] , &mut [T; M] )

🔬 This is a nightly-only experimental API. ( split_array )

Divides one mutable array reference into two at an index from the end.

The first will contain all indices from [0, N - M) (excluding the index N - M itself) and the second will contain all indices from [N - M, N) (excluding the index N itself).

#### § Panics

Panics if M > N .

#### § Examples

#![feature(split_array)]
let
mut
v = [
1
,
0
,
3
,
0
,
5
,
6
];
let
(left, right) = v.rsplit_array_mut::<
4
>();
assert_eq!
(left,
&mut
[
1
,
0
]);
assert_eq!
(right,
&mut
[
3
,
0
,
5
,
6
][..]);
left[
1
] =
2
;
right[
1
] =
4
;
assert_eq!
(v, [
1
,
2
,
3
,
4
,
5
,
6
]);

## Trait Implementations §

§

### impl<const N: usize > Clone for Frame1d <N>

§

#### fn clone (&self) -> Frame1d <N>

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl<const N: usize > Debug for Frame1d <N>

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl<const N: usize > Default for Frame1d <N>

§

#### fn default () -> Frame1d <N>

Returns the “default value” for a type.

Read more

§

### impl<const N: usize > Deref for Frame1d <N>

§

#### type Target = [Rgb< u8 >; N ]

The resulting type after dereferencing.

§

#### fn deref (&self) -> &< Frame1d <N> as Deref >:: Target

Dereferences the value.

§

### impl<const N: usize > DerefMut for Frame1d <N>

§

#### fn deref_mut (&mut self) -> &mut < Frame1d <N> as Deref >:: Target

Mutably dereferences the value.

§

### impl<const N: usize > From <[Rgb< u8 >; N ]> for Frame1d <N>

§

#### fn from (array: [Rgb< u8 >; N ]) -> Frame1d <N>

Converts to this type from the input type.

§

### impl<const N: usize > From < Frame1d <N>> for [Rgb< u8 >; N ]

§

#### fn from (frame: Frame1d <N>) -> [Rgb< u8 >; N ]

Converts to this type from the input type.

§

### impl<const N: usize > Copy for Frame1d <N>

## Auto Trait Implementations §

§

### impl<const N: usize > Freeze for Frame1d <N>

§

### impl<const N: usize > RefUnwindSafe for Frame1d <N>

§

### impl<const N: usize > Send for Frame1d <N>

§

### impl<const N: usize > Sync for Frame1d <N>

§

### impl<const N: usize > Unpin for Frame1d <N>

§

### impl<const N: usize > UnwindSafe for Frame1d <N>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<P, T> Receiver for P where P: Deref <Target = T> + ? Sized , T: ? Sized ,

Source

§

#### type Target = T

🔬 This is a nightly-only experimental API. ( arbitrary_self_types )

The target type on which the method may be called.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Rgb888 Copy item path

Path: led_strip/struct.Rgb888.html

device_envoy_rp

::

led_strip

# Struct Rgb888 Copy item path

pub struct Rgb888(
/* private fields */
);

Expand description

Rgb888 color.

Use the methods provided by the [ RgbColor ] trait to access individual color channels and predefined color constants.

See the module-level documentation for more information about conversion between this type and raw data.

## Implementations §

§

### impl Rgb888 where Rgb888 : RgbColor,

#### pub const fn new (r: u8 , g: u8 , b: u8 ) -> Rgb888

Creates a new Rgb888 color. Too large channel values will be limited by setting the unused most significant bits to zero.

## Trait Implementations §

§

### impl Clone for Rgb888

§

#### fn clone (&self) -> Rgb888

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl ColorMapping for Rgb888

§

#### fn char_to_color (c: char ) -> Rgb888

Converts a char into a color of type

C

.

§

#### fn color_to_char (color: Rgb888 ) -> char

Converts a color of type

C

into a char.

§

#### const NONE_COLOR : Rgb888 = _

Color used to display

None

values when

EG_FANCY_PANIC

is enabled.

Read more

§

### impl Debug for Rgb888

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Default for Rgb888

§

#### fn default () -> Rgb888

Returns the “default value” for a type.

Read more

§

### impl From <Bgr555> for Rgb888

§

#### fn from (other: Bgr555) -> Rgb888

Converts to this type from the input type.

§

### impl From <Bgr565> for Rgb888

§

#### fn from (other: Bgr565) -> Rgb888

Converts to this type from the input type.

§

### impl From <Bgr666> for Rgb888

§

#### fn from (other: Bgr666) -> Rgb888

Converts to this type from the input type.

§

### impl From <Bgr888> for Rgb888

§

#### fn from (other: Bgr888) -> Rgb888

Converts to this type from the input type.

§

### impl From <BinaryColor> for Rgb888

§

#### fn from (color: BinaryColor) -> Rgb888

Converts to this type from the input type.

§

### impl From <Gray2> for Rgb888

§

#### fn from (other: Gray2) -> Rgb888

Converts to this type from the input type.

§

### impl From <Gray4> for Rgb888

§

#### fn from (other: Gray4) -> Rgb888

Converts to this type from the input type.

§

### impl From <Gray8> for Rgb888

§

#### fn from (other: Gray8) -> Rgb888

Converts to this type from the input type.

§

### impl From <RawU24> for Rgb888

§

#### fn from (data: RawU24) -> Rgb888

Converts to this type from the input type.

§

### impl From <Rgb555> for Rgb888

§

#### fn from (other: Rgb555) -> Rgb888

Converts to this type from the input type.

§

### impl From <Rgb565> for Rgb888

§

#### fn from (other: Rgb565) -> Rgb888

Converts to this type from the input type.

§

### impl From <Rgb666> for Rgb888

§

#### fn from (other: Rgb666) -> Rgb888

Converts to this type from the input type.

§

### impl Hash for Rgb888

§

#### fn hash <__H>(&self, state: &mut __H ) where __H: Hasher ,

Feeds this value into the given

Hasher

.

Read more

1.3.0 · Source

§

#### fn hash_slice <H>(data: &[Self], state: &mut H ) where H: Hasher , Self: Sized ,

Feeds a slice of this type into the given

Hasher

.

Read more

§

### impl Ord for Rgb888

§

#### fn cmp (&self, other: & Rgb888 ) -> Ordering

This method returns an

Ordering

between

self

and

other

.

Read more

1.21.0 · Source

§

#### fn max (self, other: Self) -> Self where Self: Sized ,

Compares and returns the maximum of two values.

Read more

1.21.0 · Source

§

#### fn min (self, other: Self) -> Self where Self: Sized ,

Compares and returns the minimum of two values.

Read more

1.50.0 · Source

§

#### fn clamp (self, min: Self, max: Self) -> Self where Self: Sized ,

Restrict a value to a certain interval.

Read more

§

### impl PartialEq for Rgb888

§

#### fn eq (&self, other: & Rgb888 ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl PartialOrd for Rgb888

§

#### fn partial_cmp (&self, other: & Rgb888 ) -> Option < Ordering >

This method returns an ordering between

self

and

other

values if one exists.

Read more

1.0.0 · Source

§

#### fn lt (&self, other: &Rhs ) -> bool

Tests less than (for

self

and

other

) and is used by the

<

operator.

Read more

1.0.0 · Source

§

#### fn le (&self, other: &Rhs ) -> bool

Tests less than or equal to (for

self

and

other

) and is used by the

<=

operator.

Read more

1.0.0 · Source

§

#### fn gt (&self, other: &Rhs ) -> bool

Tests greater than (for

self

and

other

) and is used by the

>

operator.

Read more

1.0.0 · Source

§

#### fn ge (&self, other: &Rhs ) -> bool

Tests greater than or equal to (for

self

and

other

) and is used by the

>=

operator.

Read more

§

### impl PixelColor for Rgb888

§

#### type Raw = RawU24

Raw data type.

Read more

§

### impl RgbColor for Rgb888

§

#### const MAX_R : u8

The maximum value in the red channel.

§

#### const MAX_G : u8

The maximum value in the green channel.

§

#### const MAX_B : u8

The maximum value in the blue channel.

§

#### const BLACK : Rgb888

Black color (R: 0%, G: 0%, B: 0%)

§

#### const RED : Rgb888

Red color (R: 100%, G: 0%, B: 0%)

§

#### const GREEN : Rgb888

Green color (R: 0%, G: 100%, B: 0%)

§

#### const BLUE : Rgb888

Blue color (R: 0%, G: 0%, B: 100%)

§

#### const YELLOW : Rgb888

Yellow color (R: 100%, G: 100%, B: 0%)

§

#### const MAGENTA : Rgb888

Magenta color (R: 100%, G: 0%, B: 100%)

§

#### const CYAN : Rgb888

Cyan color (R: 0%, G: 100%, B: 100%)

§

#### const WHITE : Rgb888

White color (R: 100%, G: 100%, B: 100%)

§

#### fn r (&self) -> u8

Returns the red channel value.

§

#### fn g (&self) -> u8

Returns the green channel value.

§

#### fn b (&self) -> u8

Returns the blue channel value.

§

### impl ToRgb8 for Rgb888

§

#### fn to_rgb8 (self) -> Rgb< u8 >

Convert to

RGB8

.

§

### impl ToRgb888 for Rgb888

§

#### fn to_rgb888 (self) -> Rgb888

Convert to

Rgb888

.

§

### impl WebColors for Rgb888

Named web colors.

§

#### const CSS_ALICE_BLUE : Rgb888

Alice Blue

§

#### const CSS_ANTIQUE_WHITE : Rgb888

Antique White

§

#### const CSS_AQUA : Rgb888

Aqua

§

#### const CSS_AQUAMARINE : Rgb888

Aquamarine

§

#### const CSS_AZURE : Rgb888

Azure

§

#### const CSS_BEIGE : Rgb888

Beige

§

#### const CSS_BISQUE : Rgb888

Bisque

§

#### const CSS_BLACK : Rgb888

Black

§

#### const CSS_BLANCHED_ALMOND : Rgb888

Blanched Almond

§

#### const CSS_BLUE : Rgb888

Blue

§

#### const CSS_BLUE_VIOLET : Rgb888

Blue Violet

§

#### const CSS_BROWN : Rgb888

Brown

§

#### const CSS_BURLY_WOOD : Rgb888

Burly Wood

§

#### const CSS_CADET_BLUE : Rgb888

Cadet Blue

§

#### const CSS_CHARTREUSE : Rgb888

Chartreuse

§

#### const CSS_CHOCOLATE : Rgb888

Chocolate

§

#### const CSS_CORAL : Rgb888

Coral

§

#### const CSS_CORNFLOWER_BLUE : Rgb888

Cornflower Blue

§

#### const CSS_CORNSILK : Rgb888

Cornsilk

§

#### const CSS_CRIMSON : Rgb888

Crimson

§

#### const CSS_CYAN : Rgb888

Cyan

§

#### const CSS_DARK_BLUE : Rgb888

Dark Blue

§

#### const CSS_DARK_CYAN : Rgb888

Dark Cyan

§

#### const CSS_DARK_GOLDENROD : Rgb888

Dark Goldenrod

§

#### const CSS_DARK_GRAY : Rgb888

Dark Gray

§

#### const CSS_DARK_GREEN : Rgb888

Dark Green

§

#### const CSS_DARK_KHAKI : Rgb888

Dark Khaki

§

#### const CSS_DARK_MAGENTA : Rgb888

Dark Magenta

§

#### const CSS_DARK_OLIVE_GREEN : Rgb888

Dark Olive Green

§

#### const CSS_DARK_ORANGE : Rgb888

Dark Orange

§

#### const CSS_DARK_ORCHID : Rgb888

Dark Orchid

§

#### const CSS_DARK_RED : Rgb888

Dark Red

§

#### const CSS_DARK_SALMON : Rgb888

Dark Salmon

§

#### const CSS_DARK_SEA_GREEN : Rgb888

Dark Sea Green

§

#### const CSS_DARK_SLATE_BLUE : Rgb888

Dark Slate Blue

§

#### const CSS_DARK_SLATE_GRAY : Rgb888

Dark Slate Gray

§

#### const CSS_DARK_TURQUOISE : Rgb888

Dark Turquoise

§

#### const CSS_DARK_VIOLET : Rgb888

Dark Violet

§

#### const CSS_DEEP_PINK : Rgb888

Deep Pink

§

#### const CSS_DEEP_SKY_BLUE : Rgb888

Deep Sky Blue

§

#### const CSS_DIM_GRAY : Rgb888

Dim Gray

§

#### const CSS_DODGER_BLUE : Rgb888

Dodger Blue

§

#### const CSS_FIRE_BRICK : Rgb888

Fire Brick

§

#### const CSS_FLORAL_WHITE : Rgb888

Floral White

§

#### const CSS_FOREST_GREEN : Rgb888

Forest Green

§

#### const CSS_FUCHSIA : Rgb888

Fuchsia

§

#### const CSS_GAINSBORO : Rgb888

Gainsboro

§

#### const CSS_GHOST_WHITE : Rgb888

Ghost White

§

#### const CSS_GOLD : Rgb888

Gold

§

#### const CSS_GOLDENROD : Rgb888

Goldenrod

§

#### const CSS_GRAY : Rgb888

Gray

§

#### const CSS_GREEN : Rgb888

Green

§

#### const CSS_GREEN_YELLOW : Rgb888

Green Yellow

§

#### const CSS_HONEYDEW : Rgb888

Honeydew

§

#### const CSS_HOT_PINK : Rgb888

Hot Pink

§

#### const CSS_INDIAN_RED : Rgb888

Indian Red

§

#### const CSS_INDIGO : Rgb888

Indigo

§

#### const CSS_IVORY : Rgb888

Ivory

§

#### const CSS_KHAKI : Rgb888

Khaki

§

#### const CSS_LAVENDER : Rgb888

Lavender

§

#### const CSS_LAVENDER_BLUSH : Rgb888

Lavender Blush

§

#### const CSS_LAWN_GREEN : Rgb888

Lawn Green

§

#### const CSS_LEMON_CHIFFON : Rgb888

Lemon Chiffon

§

#### const CSS_LIGHT_BLUE : Rgb888

Light Blue

§

#### const CSS_LIGHT_CORAL : Rgb888

Light Coral

§

#### const CSS_LIGHT_CYAN : Rgb888

Light Cyan

§

#### const CSS_LIGHT_GOLDENROD_YELLOW : Rgb888

Light Goldenrod Yellow

§

#### const CSS_LIGHT_GRAY : Rgb888

Light Gray

§

#### const CSS_LIGHT_GREEN : Rgb888

Light Green

§

#### const CSS_LIGHT_PINK : Rgb888

Light Pink

§

#### const CSS_LIGHT_SALMON : Rgb888

Light Salmon

§

#### const CSS_LIGHT_SEA_GREEN : Rgb888

Light Sea Green

§

#### const CSS_LIGHT_SKY_BLUE : Rgb888

Light Sky Blue

§

#### const CSS_LIGHT_SLATE_GRAY : Rgb888

Light Slate Gray

§

#### const CSS_LIGHT_STEEL_BLUE : Rgb888

Light Steel Blue

§

#### const CSS_LIGHT_YELLOW : Rgb888

Light Yellow

§

#### const CSS_LIME : Rgb888

Lime

§

#### const CSS_LIME_GREEN : Rgb888

Lime Green

§

#### const CSS_LINEN : Rgb888

Linen

§

#### const CSS_MAGENTA : Rgb888

Magenta

§

#### const CSS_MAROON : Rgb888

Maroon

§

#### const CSS_MEDIUM_AQUAMARINE : Rgb888

Medium Aquamarine

§

#### const CSS_MEDIUM_BLUE : Rgb888

Medium Blue

§

#### const CSS_MEDIUM_ORCHID : Rgb888

Medium Orchid

§

#### const CSS_MEDIUM_PURPLE : Rgb888

Medium Purple

§

#### const CSS_MEDIUM_SEA_GREEN : Rgb888

Medium Sea Green

§

#### const CSS_MEDIUM_SLATE_BLUE : Rgb888

Medium Slate Blue

§

#### const CSS_MEDIUM_SPRING_GREEN : Rgb888

Medium Spring Green

§

#### const CSS_MEDIUM_TURQUOISE : Rgb888

Medium Turquoise

§

#### const CSS_MEDIUM_VIOLET_RED : Rgb888

Medium Violet Red

§

#### const CSS_MIDNIGHT_BLUE : Rgb888

Midnight Blue

§

#### const CSS_MINT_CREAM : Rgb888

Mint Cream

§

#### const CSS_MISTY_ROSE : Rgb888

Misty Rose

§

#### const CSS_MOCCASIN : Rgb888

Moccasin

§

#### const CSS_NAVAJO_WHITE : Rgb888

Navajo White

§

#### const CSS_NAVY : Rgb888

Navy

§

#### const CSS_OLD_LACE : Rgb888

Old Lace

§

#### const CSS_OLIVE : Rgb888

Olive

§

#### const CSS_OLIVE_DRAB : Rgb888

Olive Drab

§

#### const CSS_ORANGE : Rgb888

Orange

§

#### const CSS_ORANGE_RED : Rgb888

Orange Red

§

#### const CSS_ORCHID : Rgb888

Orchid

§

#### const CSS_PALE_GOLDENROD : Rgb888

Pale Goldenrod

§

#### const CSS_PALE_GREEN : Rgb888

Pale Green

§

#### const CSS_PALE_TURQUOISE : Rgb888

Pale Turquoise

§

#### const CSS_PALE_VIOLET_RED : Rgb888

Pale Violet Red

§

#### const CSS_PAPAYA_WHIP : Rgb888

Papaya Whip

§

#### const CSS_PEACH_PUFF : Rgb888

Peach Puff

§

#### const CSS_PERU : Rgb888

Peru

§

#### const CSS_PINK : Rgb888

Pink

§

#### const CSS_PLUM : Rgb888

Plum

§

#### const CSS_POWDER_BLUE : Rgb888

Powder Blue

§

#### const CSS_PURPLE : Rgb888

Purple

§

#### const CSS_REBECCAPURPLE : Rgb888

Rebeccapurple

§

#### const CSS_RED : Rgb888

Red

§

#### const CSS_ROSY_BROWN : Rgb888

Rosy Brown

§

#### const CSS_ROYAL_BLUE : Rgb888

Royal Blue

§

#### const CSS_SADDLE_BROWN : Rgb888

Saddle Brown

§

#### const CSS_SALMON : Rgb888

Salmon

§

#### const CSS_SANDY_BROWN : Rgb888

Sandy Brown

§

#### const CSS_SEA_GREEN : Rgb888

Sea Green

§

#### const CSS_SEASHELL : Rgb888

Seashell

§

#### const CSS_SIENNA : Rgb888

Sienna

§

#### const CSS_SILVER : Rgb888

Silver

§

#### const CSS_SKY_BLUE : Rgb888

Sky Blue

§

#### const CSS_SLATE_BLUE : Rgb888

Slate Blue

§

#### const CSS_SLATE_GRAY : Rgb888

Slate Gray

§

#### const CSS_SNOW : Rgb888

Snow

§

#### const CSS_SPRING_GREEN : Rgb888

Spring Green

§

#### const CSS_STEEL_BLUE : Rgb888

Steel Blue

§

#### const CSS_TAN : Rgb888

Tan

§

#### const CSS_TEAL : Rgb888

Teal

§

#### const CSS_THISTLE : Rgb888

Thistle

§

#### const CSS_TOMATO : Rgb888

Tomato

§

#### const CSS_TURQUOISE : Rgb888

Turquoise

§

#### const CSS_VIOLET : Rgb888

Violet

§

#### const CSS_WHEAT : Rgb888

Wheat

§

#### const CSS_WHITE : Rgb888

White

§

#### const CSS_WHITE_SMOKE : Rgb888

White Smoke

§

#### const CSS_YELLOW : Rgb888

Yellow

§

#### const CSS_YELLOW_GREEN : Rgb888

Yellow Green

§

### impl Copy for Rgb888

§

### impl Eq for Rgb888

§

### impl StructuralPartialEq for Rgb888

## Auto Trait Implementations §

§

### impl Freeze for Rgb888

§

### impl RefUnwindSafe for Rgb888

§

### impl Send for Rgb888

§

### impl Sync for Rgb888

§

### impl Unpin for Rgb888

§

### impl UnwindSafe for Rgb888

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

§

### impl<C> IntoStorage for C where C: PixelColor, <C as PixelColor>::Raw: From <C>,

§

#### type Storage = <<C as PixelColor>::Raw as RawData>::Storage

The underlying storage type for the pixel color

§

#### fn into_storage (self) -> <C as IntoStorage>::Storage

Convert the

PixelColor

into its raw storage form

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

§

### impl<C> ToBytes for C where C: PixelColor + Into <<C as PixelColor>::Raw>,

§

#### type Bytes = <<C as PixelColor>::Raw as ToBytes>::Bytes

Return type of methods in this trait.

§

#### fn to_le_bytes (self) -> <C as ToBytes>::Bytes

Converts a color into a byte array with little endian byte order.

§

#### fn to_be_bytes (self) -> <C as ToBytes>::Bytes

Converts a color into a byte array with big endian byte order.

§

#### fn to_ne_bytes (self) -> <C as ToBytes>::Bytes

Converts a color into a byte array with native byte order.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait LedStrip Copy item path

Path: led_strip/trait.LedStrip.html

device_envoy_rp

::

led_strip

# Trait LedStrip Copy item path

pub trait LedStrip<const N:
usize
> {
    const
MAX_FRAMES
:
usize
;
    const
MAX_BRIGHTNESS
:
u8
;
    const
LEN
:
usize
= N;

    // Required methods
    fn
write_frame
(&self, frame:
Frame1d
<N>);
fn
animate
<I>(&self, frames: I)
where I:
IntoIterator
,
             <I as
IntoIterator
>::
Item
:
Borrow
<(
Frame1d
<N>, Duration)>
;
}

Expand description

Platform-agnostic LED strip device contract.

Platform crates implement this for their concrete LED strip types so shared logic can drive strips without knowing the underlying hardware backend.

This page serves as the definitive reference for what a generated LED strip type provides. For first-time readers, start with the led_strip module documentation in your platform crate ( device-envoy-rp or device-envoy-esp ), then return here for a complete list of available methods and associated constants.

Design intent:

Primitive operations are LedStrip::write_frame and LedStrip::animate .

This trait is intended for static dispatch on embedded targets.

## § Example: Write a Single 1-Dimensional Frame

In this example, we set every other LED to blue and gray.

use
device_envoy_core::led_strip::{Frame1d, LedStrip, colors};
fn
write_alternating_blue_gray<
const
N: usize>(led_strip:
&
impl
LedStrip<N>) {
let
mut
frame = Frame1d::new();
for
pixel_index
in
0
..N {
        frame[pixel_index] = [colors::BLUE, colors::GRAY][pixel_index %
2
];
    }
    led_strip.write_frame(frame);
}

## § Example: Animate a Sequence

This example animates a 96-LED strip through red, green, and blue frames, cycling continuously.

use
device_envoy_core::led_strip::{Frame1d, LedStrip, colors};
use
embassy_time::Duration;
fn
animate_rgb_cycle<
const
N: usize>(led_strip:
&
impl
LedStrip<N>) {
let
frame_duration = Duration::from_millis(
300
);
    led_strip.animate([
        (Frame1d::filled(colors::RED), frame_duration),
        (Frame1d::filled(colors::GREEN), frame_duration),
        (Frame1d::filled(colors::BLUE), frame_duration),
    ]);
}

## Required Associated Constants §

#### const MAX_FRAMES : usize

Maximum number of animation frames allowed.

#### const MAX_BRIGHTNESS : u8

Maximum brightness level, automatically limited by the power budget.

## Provided Associated Constants §

#### const LEN : usize = N

Number of LEDs in this strip.

## Required Methods §

#### fn write_frame (&self, frame: Frame1d <N>)

Write a frame to the LED strip.

See the LedStrip trait documentation for usage examples.

#### fn animate <I>(&self, frames: I) where I: IntoIterator , <I as IntoIterator >:: Item : Borrow <( Frame1d <N>, Duration)>,

Animate frames on the LED strip.

The duration type is embassy_time::Duration , and frames can be any iterator whose items borrow (Frame1d<N>, embassy_time::Duration) .

See the LedStrip trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl LedStrip <8> for LedStripGenerated

Source

§

#### const LEN : usize = Self::LEN

Source

§

#### const MAX_FRAMES : usize = Self::MAX_FRAMES

Source

§

#### const MAX_BRIGHTNESS : u8 = Self::MAX_BRIGHTNESS

# Trait ToRgb8 Copy item path

Path: led_strip/trait.ToRgb8.html

device_envoy_rp

::

led_strip

# Trait ToRgb8 Copy item path

pub trait ToRgb8 {
    // Required method
    fn
to_rgb8
(self) -> Rgb<
u8
>;
}

Expand description

Convert a color to RGB8 for LED strip rendering.

## Required Methods §

#### fn to_rgb8 (self) -> Rgb< u8 >

Convert to RGB8 .

## Implementations on Foreign Types §

§

### impl ToRgb8 for Rgb< u8 >

§

#### fn to_rgb8 (self) -> Rgb< u8 >

## Implementors §

§

### impl ToRgb8 for Rgb888

# Trait ToRgb888 Copy item path

Path: led_strip/trait.ToRgb888.html

device_envoy_rp

::

led_strip

# Trait ToRgb888 Copy item path

pub trait ToRgb888 {
    // Required method
    fn
to_rgb888
(self) ->
Rgb888
;
}

Expand description

Convert a color to Rgb888 for embedded-graphics rendering.

## Required Methods §

#### fn to_rgb888 (self) -> Rgb888

Convert to Rgb888 .

## Implementations on Foreign Types §

§

### impl ToRgb888 for Rgb< u8 >

§

#### fn to_rgb888 (self) -> Rgb888

## Implementors §

§

### impl ToRgb888 for Rgb888

# Type Alias RGB8 Copy item path

Path: led_strip/type.RGB8.html

device_envoy_rp

::

led_strip

# Type Alias RGB8 Copy item path

pub type RGB8 = Rgb<
u8
>;

Expand description

8-bit RGB

The colorspace is technically undefined, but generally sRGB is assumed.

## Aliased Type §

#[repr(C)]
pub struct RGB8 {
    pub r:
u8
,
    pub g:
u8
,
    pub b:
u8
,
}

## Fields §

§ r: u8

Red Component

§ g: u8

Green Component

§ b: u8

Blue Component

# Enum Rfid Event Copy item path

Path: rfid/enum.RfidEvent.html

device_envoy_rp

::

rfid

# Enum Rfid Event Copy item path

pub enum RfidEvent {
    CardDetected {
        uid: [
u8
;
10
],
    },
}

Expand description

Events received from an RFID reader.

## Variants §

§

### CardDetected

A card was detected with a 10-byte UID value.

#### Fields

§ uid: [ u8 ; 10 ]

UID bytes, padded with zeros if the physical UID is shorter than 10 bytes.

## Trait Implementations §

§

### impl Clone for RfidEvent

§

#### fn clone (&self) -> RfidEvent

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for RfidEvent

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Format for RfidEvent where [ u8 ; 10 ]: Format,

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl PartialEq for RfidEvent

§

#### fn eq (&self, other: & RfidEvent ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for RfidEvent

§

### impl Eq for RfidEvent

§

### impl StructuralPartialEq for RfidEvent

## Auto Trait Implementations §

§

### impl Freeze for RfidEvent

§

### impl RefUnwindSafe for RfidEvent

§

### impl Send for RfidEvent

§

### impl Sync for RfidEvent

§

### impl Unpin for RfidEvent

§

### impl UnwindSafe for RfidEvent

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module rfid Copy item path

Path: rfid/index.html

device_envoy_rp

# Module rfid Copy item path

Source

Expand description

A device abstraction for RFID readers using the MFRC522 chip.

See RfidRp for the primary example; helper methods link back there.

You can create up to two concurrent RfidRp instances per program; a third is expected to fail at runtime because the rfid task pool uses pool_size = 2 .

## Structs §

RfidRp A device abstraction for an RFID reader using the MFRC522 chip. Rfid Static Static resources for the RfidRp device abstraction.

## Enums §

Rfid Event Events received from an RFID reader.

## Traits §

Rfid Platform-agnostic RFID reader contract.

# Struct RfidRp Copy item path

Path: rfid/struct.RfidRp.html

device_envoy_rp

::

rfid

# Struct RfidRp Copy item path

Source

pub struct RfidRp<'a> {
/* private fields */
}

Expand description

A device abstraction for an RFID reader using the MFRC522 chip.

## § Example

use
device_envoy_rp::rfid::{Rfid, RfidEvent, RfidRp, RfidStatic};
async fn
example(
    p: embassy_rp::Peripherals,
    spawner: embassy_executor::Spawner,
) -> device_envoy_rp::Result<()> {
static
RFID_STATIC: RfidStatic = RfidRp::new_static();
let
rfid = RfidRp::new_spi0(
&
RFID_STATIC,
        p.SPI0,
        p.PIN_2,
        p.PIN_3,
        p.PIN_4,
        p.DMA_CH0,
        p.DMA_CH1,
        p.PIN_1,
        p.PIN_5,
        spawner,
    )
    .
await
?
;
loop
{
let
RfidEvent::CardDetected { uid } = rfid.wait_for_tap().
await
;
info!
(
"RFID uid: {:?}"
, uid);
    }
}

## Implementations §

Source

§

### impl RfidRp <'_>

Source

#### pub const fn new_static () -> RfidStatic

Create static channel resources for an RFID reader.

Source

#### pub async fn new_spi0 <Sck, Mosi, Miso, DmaTx, DmaRx, Cs, Rst>( rfid_static: &'static RfidStatic , spi: Peri<'static, SPI0>, sck: Peri<'static, Sck>, mosi: Peri<'static, Mosi>, miso: Peri<'static, Miso>, dma_tx: Peri<'static, DmaTx>, dma_rx: Peri<'static, DmaRx>, cs: Peri<'static, Cs>, rst: Peri<'static, Rst>, spawner: Spawner, ) -> Result <Self> where Sck: Pin + ClkPin<SPI0>, Mosi: Pin + MosiPin<SPI0>, Miso: Pin + MisoPin<SPI0>, DmaTx: ChannelInstance, DmaRx: ChannelInstance, Cs: Pin, Rst: Pin, DmaAllIrqs: Binding<DmaTx::Interrupt, InterruptHandler<DmaTx>> + Binding<DmaRx::Interrupt, InterruptHandler<DmaRx>>,

Create a new RFID reader instance using SPI0.

See the Rfid struct example for usage.

Source

#### pub async fn new_spi1 <Sck, Mosi, Miso, DmaTx, DmaRx, Cs, Rst>( rfid_static: &'static RfidStatic , spi: Peri<'static, SPI1>, sck: Peri<'static, Sck>, mosi: Peri<'static, Mosi>, miso: Peri<'static, Miso>, dma_tx: Peri<'static, DmaTx>, dma_rx: Peri<'static, DmaRx>, cs: Peri<'static, Cs>, rst: Peri<'static, Rst>, spawner: Spawner, ) -> Result <Self> where Sck: Pin + ClkPin<SPI1>, Mosi: Pin + MosiPin<SPI1>, Miso: Pin + MisoPin<SPI1>, DmaTx: ChannelInstance, DmaRx: ChannelInstance, Cs: Pin, Rst: Pin, DmaAllIrqs: Binding<DmaTx::Interrupt, InterruptHandler<DmaTx>> + Binding<DmaRx::Interrupt, InterruptHandler<DmaRx>>,

Create a new RFID reader instance using SPI1.

See the Rfid struct example for usage.

## Trait Implementations §

Source

§

### impl Rfid for RfidRp <'_>

Source

§

#### async fn wait_for_tap (&self) -> RfidEvent

Wait for the next RFID event.

Read more

## Auto Trait Implementations §

§

### impl<'a> Freeze for RfidRp <'a>

§

### impl<'a> ! RefUnwindSafe for RfidRp <'a>

§

### impl<'a> Send for RfidRp <'a>

§

### impl<'a> Sync for RfidRp <'a>

§

### impl<'a> Unpin for RfidRp <'a>

§

### impl<'a> ! UnwindSafe for RfidRp <'a>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Rfid Static Copy item path

Path: rfid/struct.RfidStatic.html

device_envoy_rp

::

rfid

# Struct Rfid Static Copy item path

Source

pub struct RfidStatic(
/* private fields */
);

Expand description

Static resources for the RfidRp device abstraction.

## Implementations §

Source

§

### impl RfidStatic

Source

#### pub const fn new () -> Self

Create static resources for an RFID reader.

## Auto Trait Implementations §

§

### impl ! Freeze for RfidStatic

§

### impl ! RefUnwindSafe for RfidStatic

§

### impl Send for RfidStatic

§

### impl Sync for RfidStatic

§

### impl Unpin for RfidStatic

§

### impl UnwindSafe for RfidStatic

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Rfid Copy item path

Path: rfid/trait.Rfid.html

device_envoy_rp

::

rfid

# Trait Rfid Copy item path

pub trait Rfid {
    // Required method
    async fn
wait_for_tap
(&self) ->
RfidEvent
;
}

Expand description

Platform-agnostic RFID reader contract.

Platform crates implement this for concrete Rfid types so shared logic can await card-tap events without depending on platform-specific modules.

## § Example

use
device_envoy_core::rfid::{Rfid, RfidEvent};
async fn
log_card_taps(rfid:
&
impl
Rfid) -> ! {
loop
{
let
RfidEvent::CardDetected { uid } = rfid.wait_for_tap().
await
;
let _
= uid;
    }
}

## Required Methods §

#### async fn wait_for_tap (&self) -> RfidEvent

Wait for the next RFID event.

See the Rfid trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Rfid for RfidRp <'_>

# Constant SERVO_ MAX_ US_ DEFAULT Copy item path

Path: servo/constant.SERVO_MAX_US_DEFAULT.html

device_envoy_rp

::

servo

# Constant SERVO_ MAX_ US_ DEFAULT Copy item path

Source

pub const SERVO_MAX_US_DEFAULT:
u16
= 2_500;

Expand description

Default maximum pulse width for hobby servos (microseconds).

# Constant SERVO_ MIN_ US_ DEFAULT Copy item path

Path: servo/constant.SERVO_MIN_US_DEFAULT.html

device_envoy_rp

::

servo

# Constant SERVO_ MIN_ US_ DEFAULT Copy item path

Source

pub const SERVO_MIN_US_DEFAULT:
u16
= 500;

Expand description

Default minimum pulse width for hobby servos (microseconds).

# Enum AtEnd Copy item path

Path: servo/enum.AtEnd.html

device_envoy_rp

::

servo

# Enum AtEnd Copy item path

pub enum AtEnd {
    Loop,
    Hold,
    Relax,
}

Expand description

Animation end behavior.

## Variants §

§

### Loop

Repeat the animation sequence indefinitely.

§

### Hold

Hold the final position when animation completes.

§

### Relax

Stop holding position after animation completes (servo relaxes).

## Trait Implementations §

§

### impl Clone for AtEnd

§

#### fn clone (&self) -> AtEnd

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for AtEnd

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl Format for AtEnd

§

#### fn format (&self, f: Formatter<'_>)

Writes the defmt representation of

self

to

fmt

.

§

### impl PartialEq for AtEnd

§

#### fn eq (&self, other: & AtEnd ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for AtEnd

§

### impl Eq for AtEnd

§

### impl StructuralPartialEq for AtEnd

## Auto Trait Implementations §

§

### impl Freeze for AtEnd

§

### impl RefUnwindSafe for AtEnd

§

### impl Send for AtEnd

§

### impl Sync for AtEnd

§

### impl Unpin for AtEnd

§

### impl UnwindSafe for AtEnd

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Enum Direction Copy item path

Path: servo/enum.Direction.html

device_envoy_rp

::

servo

# Enum Direction Copy item path

pub enum Direction {
    Forward,
    Reverse,
}

Expand description

Logical angle direction for servo mapping.

## Variants §

§

### Forward

0 maps to the minimum pulse and increases toward maximum pulse.

§

### Reverse

0 maps to the maximum pulse and increases toward minimum pulse.

## Trait Implementations §

§

### impl Clone for Direction

§

#### fn clone (&self) -> Direction

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for Direction

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl PartialEq for Direction

§

#### fn eq (&self, other: & Direction ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for Direction

§

### impl Eq for Direction

§

### impl StructuralPartialEq for Direction

## Auto Trait Implementations §

§

### impl Freeze for Direction

§

### impl RefUnwindSafe for Direction

§

### impl Send for Direction

§

### impl Sync for Direction

§

### impl Unpin for Direction

§

### impl UnwindSafe for Direction

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Function combine Copy item path

Path: servo/fn.combine.html

device_envoy_rp

::

servo

# Function combine Copy item path

pub const fn combine<const N1:
usize
, const N2:
usize
, const OUT_N:
usize
>(
    first: [(
u16
, Duration);
N1
],
    second: [(
u16
, Duration);
N2
],
) -> [(
u16
, Duration);
OUT_N
]

Expand description

Combine two animation step arrays into one larger array.

This uses embassy_time::Duration for step timing.

# Function linear Copy item path

Path: servo/fn.linear.html

device_envoy_rp

::

servo

# Function linear Copy item path

pub const fn linear<const N:
usize
>(
    start_degrees:
u16
,
    end_degrees:
u16
,
    total_duration: Duration,
) -> [(
u16
, Duration);
N
]

Expand description

Build a const linear sequence of animation steps as an array.

This function creates N evenly spaced degree targets from start_degrees to end_degrees , each using the same per-step duration.

Syntax:

linear::<N>(<start_degrees>, <end_degrees>, <total_duration>)

Behavior:

N must be greater than zero.

When N > 1 , the first step is start_degrees and the last step is end_degrees .

Each returned step uses the same duration, derived from total_duration / N .

This uses embassy_time::Duration for step timing.

See the servo module documentation for usage examples.

# Module servo Copy item path

Path: servo/index.html

device_envoy_rp

# Module servo Copy item path

Source

Expand description

A device abstraction for hobby servos.

This module provides both direct servo control ( servo! ) and servo animation ( servo_player! ).

Use the servo! macro for a keyword-driven constructor with defaults.

After reading the examples below, see also:

servo! — Direct servo control without animation support.

servo_player! — Macro to generate a servo player struct type (includes syntax details). See ServoPlayerGenerated for a sample of a generated type.

combine! & linear — Macro and function for creating complex motion sequences.

Servo — Trait defining core methods and constants for direct servo control.

ServoPlayer — Trait defining animation methods for servos and extending Servo ( set_degrees , hold , and relax ). Depending on method-resolution context, call sites may still need Servo in scope.

## § How Servos Work

When you call set_degrees(45) , the Pico starts sending a PWM control signal to the servo, telling it to move to and hold at 45 degrees. The Pico’s hardware generates this control signal automatically in the background, taking no CPU time. The control signal remains active until you change it or call relax() .

The servo itself has no idea what angle it is currently at. It simply moves as fast as it can to match whatever angle the current control signal specifies. The library does not—and cannot—wait for the servo to reach a position. This is why you must wait (for example, using Timer::after() ) after calling set_degrees() to give the servo time to physically reach the target position.

This device abstraction, servo_player , adds a background software task around the hardware control signal.

## § Controlling Multiple Servos

Supports up to eight servos, one per PWM slice resource. To calculate which PWM slice a pin uses, use the formula: PWM slice = (pin / 2) % 8 . For example, PIN_10 and PIN_11 must both use PWM_SLICE5 ((10 / 2) % 8 = 5, (11 / 2) % 8 = 5). Therefore, either of these two pins can have a servo, but not both.

## § Example: Basic Servo Control

This example demonstrates basic servo control: moving to a position, holding, relaxing, and using animation. Here, the generated struct type is named ServoPlayer11 .

use
device_envoy_rp::{
Result
, servo::{AtEnd, Servo
as _
, ServoPlayer
as _
, servo_player}};
use
embassy_time::{Duration, Timer};
// Define ServoPlayer11, a struct type for a servo on PIN_11.
servo_player!
{
    ServoPlayer11 {
        pin: PIN_11,
// GPIO pin for servo
        // other inputs set to their defaults
}
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
// PIN_11 uses PWM_SLICE5 (pin / 2) % 8 = (11 / 2) % 8 = 5 % 8 = 5)
let
servo_player11 = ServoPlayer11::new(p.PIN_11, p.PWM_SLICE5, spawner)
?
;
// Move to 90°, wait 1 second, then relax.
servo_player11.set_degrees(
90
);
    Timer::after(Duration::from_secs(
1
)).
await
;
    servo_player11.relax();
// Animate: hold at 180° for 1 second, then 0° for 1 second, then relax.
const
STEPS: [(u16, Duration);
2
] = [
        (
180
, Duration::from_secs(
1
)),
        (
0
, Duration::from_secs(
1
)),
    ];
// AtEnd::Relax quiets the servo; AtEnd::Hold keeps driving pulses to hold
    // position; AtEnd::Loop repeats.
servo_player11.animate(STEPS, AtEnd::Relax);

    pending().
await
// run forever
}

## § Example: Multi-Step Animation

This example combines 40 animation steps using linear and combine! to sweep up, hold, sweep down, hold pattern. Here, the generated struct type is named ServoSweep .

use
device_envoy_rp::{
Result
, servo::{AtEnd, Servo
as _
, ServoPlayer
as _
, combine, linear, servo_player}};
use
embassy_time::Duration;
servo_player!
{
    ServoSweep {
        pin: PIN_12,
        max_steps:
40
,
    }
}
async fn
example(spawner: embassy_executor::Spawner) ->
Result
<Infallible> {
let
p = embassy_rp::init(Default::default());
let
servo_sweep = ServoSweep::new(p.PIN_12, p.PWM_SLICE6, spawner)
?
;
const
STEPS: [(u16, Duration);
40
] =
combine!
(
        linear::<
19
>(
0
,
180
, Duration::from_secs(
2
)),
        [(
180
, Duration::from_millis(
400
))],
        linear::<
19
>(
180
,
0
, Duration::from_secs(
2
)),
        [(
0
, Duration::from_millis(
400
))]
    );

    servo_sweep.animate(STEPS, AtEnd::Loop);
    pending().
await
}

## Modules §

servo_ player_ generated Sample generated servo-player type documentation.

## Macros §

combine Combine multiple animation step arrays into one larger array. servo Create a servo with keyword arguments and default pulse widths. servo_ player Macro to generate a servo player struct type (includes syntax details).

## Structs §

ServoRp A device abstraction for hobby servos.

## Enums §

AtEnd Animation end behavior. Direction Logical angle direction for servo mapping.

## Constants §

SERVO_ MAX_ US_ DEFAULT Default maximum pulse width for hobby servos (microseconds). SERVO_ MIN_ US_ DEFAULT Default minimum pulse width for hobby servos (microseconds).

## Traits §

Servo Platform-agnostic servo device contract. Servo Player Platform-agnostic servo-player device contract.

## Functions §

combine Combine two animation step arrays into one larger array. linear Build a const linear sequence of animation steps as an array.

# macro.combine!.html

Path: servo/macro.combine!.html

Redirecting to macro.combine.html ...

# Macro combine Copy item path

Path: servo/macro.combine.html

device_envoy_rp

::

servo

# Macro combine Copy item path

Source

macro_rules! combine {
    () => { ... };
    ($single:expr) => { ... };
    ($first:expr, $second:expr) => { ... };
    ($first:expr, $($rest:expr),+ $(,)?) => { ... };
}

Expand description

Combine multiple animation step arrays into one larger array.

This macro allows combining any number of const arrays with a clean syntax.

Syntax:

combine!()
combine!(<steps_expr>)
combine!(<first_steps_expr>, <second_steps_expr>, ... )

See the servo module documentation for usage.

# macro.servo!.html

Path: servo/macro.servo!.html

Redirecting to macro.servo.html ...

# Macro servo Copy item path

Path: servo/macro.servo.html

device_envoy_rp

::

servo

# Macro servo Copy item path

Source

macro_rules! servo {
    ($($tt:tt)*) => { ... };
}

Expand description

Create a servo with keyword arguments and default pulse widths.

Syntax:

servo! {
    pin: <pin_expr>,
    slice: <pwm_slice_expr>,
    channel: A | B,             // optional
    odd: <bool_expr>,           // optional
    even: <bool_expr>,          // optional
    min_us: <u16_expr>,         // optional
    max_us: <u16_expr>,         // optional
    max_degrees: <u16_expr>,    // optional
    direction: <Direction_expr>, // optional
}

Required fields:

pin - GPIO pin for servo output

slice - PWM slice resource

Optional fields:

channel: A | B - Explicitly choose PWM output channel

odd / even - Shorthand channel selection ( odd => B , even => A )

min_us - Minimum pulse width in microseconds for 0° (default: SERVO_MIN_US_DEFAULT )

max_us - Maximum pulse width in microseconds for max_degrees (default: SERVO_MAX_US_DEFAULT )

max_degrees - Maximum servo angle in degrees (default: 180 )

direction - Logical angle direction ( Direction::Forward by default)

See the servo module documentation for details and examples.

# macro.servo_player!.html

Path: servo/macro.servo_player!.html

Redirecting to macro.servo_player.html ...

# Macro servo_ player Copy item path

Path: servo/macro.servo_player.html

device_envoy_rp

::

servo

# Macro servo_ player Copy item path

Source

macro_rules! servo_player {
    ($($tt:tt)*) => { ... };
}

Expand description

Macro to generate a servo player struct type (includes syntax details).

This page provides the primary documentation for configuring individual servo players.

See the servo module documentation for complete examples.

After reading the configuration details below, see also:

servo module — Complete examples and usage patterns

Use this macro when your project has a servo that needs scripted animation control. The macro generates a struct type and spawns a background task to execute animation sequences.

Syntax:

servo_player! {
    [<visibility>] <Name> {
        pin: <pin_ident>,
        min_us: <u16_expr>,         // optional
        max_us: <u16_expr>,         // optional
        max_degrees: <u16_expr>,    // optional
        direction: <Direction_expr>,    // optional
        max_steps: <usize_expr>,    // optional
    }
}

## § Configuration

Required fields:

pin — GPIO pin for servo

Optional fields:

min_us — Minimum pulse width in microseconds for 0° (default: 500)

max_us — Maximum pulse width in microseconds for max_degrees (default: 2500)

max_degrees — Maximum servo angle in degrees (default: 180)

direction — Logical direction mapping ( Direction::Forward by default)

max_steps — Maximum number of animation steps (default: 16)

max_steps = 0 disables animation and allocates no step storage; set_degrees() , hold() , and relax() are still supported.

# Module servo_ player_ generated Copy item path

Path: servo/servo_player_generated/index.html

device_envoy_rp

::

servo

# Module servo_ player_ generated Copy item path

Source

Expand description

Sample generated servo-player type documentation.

## Structs §

Servo Player Generated Sample struct type generated by the servo_player! macro, showing all methods and constants.

# Struct Servo Player Generated Copy item path

Path: servo/servo_player_generated/struct.ServoPlayerGenerated.html

device_envoy_rp

::

servo :: servo_player_generated

# Struct Servo Player Generated Copy item path

Source

pub struct ServoPlayerGenerated;

Expand description

Sample struct type generated by the servo_player! macro, showing all methods and constants.

This page serves as the definitive reference for what a generated servo player type provides. For first-time readers, start with the examples in the servo module documentation, then return here for a complete list of available methods and associated constants.

Auto-generated.

## Implementations §

Source

§

### impl ServoPlayerGenerated

Source

#### pub const MAX_STEPS : usize = 16

Maximum number of animation steps allowed.

Specified in the servo_player! macro.

Source

#### pub fn new ( pin: Peri<'static, PIN_11>, slice: Peri<'static, PWM_SLICE5>, spawner: Spawner, ) -> Result <&'static Self>

Create a new servo player instance of the struct type defined by servo_player! .

See the servo module docs for usage.

The pin and slice parameters must correspond to the GPIO pin specified in the macro.

#### § Parameters

pin : GPIO pin for servo

slice : PWM slice for servo control ( slice = (pin / 2) % 8 )

spawner : Task spawner for background operations

## Trait Implementations §

Source

§

### impl Servo for ServoPlayerGenerated

Source

§

#### fn set_degrees (&self, degrees: u16 )

Set the target angle. The most recent command always wins.

See the servo module docs for usage.

Source

§

#### fn hold (&self)

Hold the servo at its current position.

See the servo module docs for usage.

Source

§

#### fn relax (&self)

Relax the servo. Servo can move freely.

See the servo module docs for usage.

Source

§

#### const DEFAULT_MAX_DEGREES : u16 = 180

Default maximum rotation range in degrees.

Source

§

### impl ServoPlayer <16> for ServoPlayerGenerated

Source

§

#### fn animate <I>(&self, steps: I, at_end: AtEnd ) where I: IntoIterator , I:: Item : Borrow <( u16 , Duration)>,

Animate the servo through a sequence of angles with per-step hold durations.

Each step is a tuple (degrees, duration) . Accepts both owned iterators and references to collections.

This uses embassy_time::Duration for step timing.

See the servo module docs for usage.

Source

§

#### const MAX_STEPS : usize = Self::MAX_STEPS

Maximum number of animation steps accepted by

ServoPlayer::animate

.

## Auto Trait Implementations §

§

### impl Freeze for ServoPlayerGenerated

§

### impl RefUnwindSafe for ServoPlayerGenerated

§

### impl Send for ServoPlayerGenerated

§

### impl Sync for ServoPlayerGenerated

§

### impl Unpin for ServoPlayerGenerated

§

### impl UnwindSafe for ServoPlayerGenerated

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct ServoRp Copy item path

Path: servo/struct.ServoRp.html

device_envoy_rp

::

servo

# Struct ServoRp Copy item path

Source

pub struct ServoRp<'d> {
/* private fields */
}

Expand description

A device abstraction for hobby servos.

Use servo! for direct, immediate control when you want to manually manage servo positioning. Use servo_player instead when you need background animation sequences or want motion to continue while your code does other work.

## § How Servos Work

When you call set_degrees(45) , the Pico starts sending a PWM control signal to the servo, telling it to move to and hold at 45 degrees. The Pico’s hardware generates this control signal automatically in the background, taking no CPU time. The control signal remains active until you change it or call relax() .

The servo itself has no idea what angle it is currently at. It simply moves as fast as it can to match whatever angle the current control signal specifies. The library does not—and cannot—wait for the servo to reach a position. This is why you must wait (for example, using Timer::after() ) after calling set_degrees() to give the servo time to physically reach the target position.

## § Examples

use
device_envoy_rp::{servo, servo::Servo
as _
};
use
embassy_time::{Duration, Timer};
async fn
example(p: embassy_rp::Peripherals) {
// Create a servo on GPIO 11.
    // GPIO 11 → (11/2) % 8 = 5 → PWM_SLICE5
let
mut
servo =
servo!
{
        pin: p.PIN_11,
        slice: p.PWM_SLICE5,
    };

    servo.set_degrees(
45
);
// Move to 45 degrees and hold.
Timer::after(Duration::from_secs(
1
)).
await
;
// Give servo reasonable time to reach position
servo.set_degrees(
90
);
// Move to 90 degrees and hold.
Timer::after(Duration::from_secs(
1
)).
await
;
// Give servo reasonable time to reach position
servo.relax();
// Let the servo relax. It will re-enable on next set_degrees()
}

## Implementations §

Source

§

### impl<'d> ServoRp <'d>

Source

#### pub const DEFAULT_MAX_DEGREES : u16 = <Self as Servo>::DEFAULT_MAX_DEGREES

Default maximum rotation range in degrees (180°).

## Trait Implementations §

Source

§

### impl<'d> Servo for ServoRp <'d>

Source

§

#### fn set_degrees (&self, degrees: u16 )

Set position in degrees 0..=max_degrees mapped into [min_us, max_us].

Automatically enables the servo if it was disabled.

See the servo module documentation for usage examples.

Source

§

#### fn relax (&self)

Stop sending control signals to the servo.

This allows the servo to relax and move freely, reducing power consumption and mechanical stress.

See the servo module documentation for usage examples.

Source

§

#### fn hold (&self)

Resume sending control signals to the servo.

The servo will move back to its last commanded position.

Source

§

#### const DEFAULT_MAX_DEGREES : u16 = 180

Default maximum rotation range in degrees.

## Auto Trait Implementations §

§

### impl<'d> ! Freeze for ServoRp <'d>

§

### impl<'d> ! RefUnwindSafe for ServoRp <'d>

§

### impl<'d> Send for ServoRp <'d>

§

### impl<'d> ! Sync for ServoRp <'d>

§

### impl<'d> Unpin for ServoRp <'d>

§

### impl<'d> ! UnwindSafe for ServoRp <'d>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Servo Copy item path

Path: servo/trait.Servo.html

device_envoy_rp

::

servo

# Trait Servo Copy item path

pub trait Servo {
    const
DEFAULT_MAX_DEGREES
:
u16
;

    // Required methods
    fn
set_degrees
(&self, degrees:
u16
);
fn
hold
(&self);
fn
relax
(&self);
}

Expand description

Platform-agnostic servo device contract.

Platform crates implement this trait for their concrete servo types so direct servo operations resolve through trait methods instead of inherent methods.

This page serves as the definitive reference for direct servo control across platforms.

## § Example

This example demonstrates basic servo control: move to 45 degrees, then 90 degrees, then relax.

use
device_envoy_core::servo::Servo;
use
embassy_time::{Duration, Timer};
async fn
move_and_relax(servo:
&
impl
Servo) {
    servo.set_degrees(
45
);
// Move to 45 degrees and hold.
Timer::after(Duration::from_secs(
1
)).
await
;
// Give servo reasonable time to reach position
servo.set_degrees(
90
);
// Move to 90 degrees and hold.
Timer::after(Duration::from_secs(
1
)).
await
;
// Give servo reasonable time to reach position
servo.relax();
// Let the servo relax. It will re-enable on next set_degrees()
}

## Required Associated Constants §

#### const DEFAULT_MAX_DEGREES : u16

Default maximum rotation range in degrees.

## Required Methods §

#### fn set_degrees (&self, degrees: u16 )

Set position in degrees 0..=max_degrees .

See the Servo trait documentation for usage examples.

#### fn hold (&self)

Keep driving pulses at the last commanded angle.

#### fn relax (&self)

Stop driving pulses.

See the Servo trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl Servo for ServoPlayerGenerated

Source

§

#### const DEFAULT_MAX_DEGREES : u16 = 180

Source

§

### impl<'d> Servo for ServoRp <'d>

Source

§

#### const DEFAULT_MAX_DEGREES : u16 = 180

# Trait Servo Player Copy item path

Path: servo/trait.ServoPlayer.html

device_envoy_rp

::

servo

# Trait Servo Player Copy item path

pub trait ServoPlayer<const MAX_STEPS:
usize
>:
Servo
{
    const
MAX_STEPS
:
usize
;

    // Required method
    fn
animate
<I>(&self, steps: I, at_end:
AtEnd
)
where I:
IntoIterator
,
             <I as
IntoIterator
>::
Item
:
Borrow
<(
u16
, Duration)>
;
}

Expand description

Platform-agnostic servo-player device contract.

Platform crates implement this trait for generated servo player types so servo operations resolve through trait methods instead of inherent methods.

This trait extends Servo , so a servo player supports all Servo methods ( set_degrees , hold , and relax ) in addition to ServoPlayer::animate . Depending on method-resolution context, call sites may still need Servo in scope.

## § Example: Basic Servo Control

This example demonstrates basic servo control: moving to a position, relaxing, and using animation.

use
device_envoy_core::servo::{AtEnd, ServoPlayer};
use
embassy_time::{Duration, Timer};
async fn
basic_servo_control<
const
MAX_STEPS: usize>(servo_player:
&
impl
ServoPlayer<MAX_STEPS>) {
// Move to 90 degrees, wait 1 second, then relax.
servo_player.set_degrees(
90
);
    Timer::after(Duration::from_secs(
1
)).
await
;
    servo_player.relax();
// Animate: hold at 180 degrees for 1 second, then 0 degrees for 1 second, then relax.
const
STEPS: [(u16, Duration);
2
] = [
        (
180
, Duration::from_secs(
1
)),
        (
0
, Duration::from_secs(
1
)),
    ];
// AtEnd::Relax quiets the servo; AtEnd::Hold keeps driving pulses to hold
    // position; AtEnd::Loop repeats.
servo_player.animate(STEPS, AtEnd::Relax);
}

## § Example: Multi-Step Animation

This example combines 40 animation steps using linear and combine to sweep up, hold, sweep down, hold.

use
device_envoy_core::servo::{AtEnd, ServoPlayer, combine, linear};
use
embassy_time::Duration;
async fn
run_sweep_animation(servo_player:
&
impl
ServoPlayer<
40
>) {
// Combine 40 animation steps into one array.
const
STEPS_UP_AND_HOLD: [(u16, Duration);
20
] = combine::<
19
,
1
,
20
>(
        linear::<
19
>(
0
,
180
, Duration::from_secs(
2
)),
// 19 steps from 0 degrees to 180 degrees
[(
180
, Duration::from_millis(
400
))],
// Hold at 180 degrees for 400 ms
);
const
STEPS_DOWN_AND_HOLD: [(u16, Duration);
20
] = combine::<
19
,
1
,
20
>(
        linear::<
19
>(
180
,
0
, Duration::from_secs(
2
)),
// 19 steps from 180 degrees to 0 degrees
[(
0
, Duration::from_millis(
400
))],
// Hold at 0 degrees for 400 ms
);
const
STEPS: [(u16, Duration);
40
] =
        combine::<
20
,
20
,
40
>(STEPS_UP_AND_HOLD, STEPS_DOWN_AND_HOLD);

    servo_player.animate(STEPS, AtEnd::Loop);
// Loop the sweep animation

    // Let it run in the background for 10 seconds, then relax.
embassy_time::Timer::after(Duration::from_secs(
10
)).
await
;
    servo_player.relax();
}

## Required Associated Constants §

#### const MAX_STEPS : usize

Maximum number of animation steps accepted by ServoPlayer::animate .

## Required Methods §

#### fn animate <I>(&self, steps: I, at_end: AtEnd ) where I: IntoIterator , <I as IntoIterator >:: Item : Borrow <( u16 , Duration)>,

Animate through a sequence of angles with per-step hold durations.

This uses embassy_time::Duration for step timing.

See the ServoPlayer trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl ServoPlayer <16> for ServoPlayerGenerated

Source

§

#### const MAX_STEPS : usize = Self::MAX_STEPS

# struct.ServoPlayerGenerated.html

Path: servo_player/servo_player_generated/struct.ServoPlayerGenerated.html

Redirecting to ../../../device_envoy_rp/servo/servo_player_generated/struct.ServoPlayerGenerated.html ...

# Type Alias Result Copy item path

Path: type.Result.html

device_envoy_rp

# Type Alias Result Copy item path

Source

pub type Result<T, E =
Error
> =
Result
<T, E>;

Expand description

A specialized Result where the error is this crate’s Error type.

## Aliased Type §

pub enum Result<T, E =
Error
> {
    Ok(T),
    Err(E),
}

## Variants §

§

1.0.0

### Ok(T)

Contains the success value

§

1.0.0

### Err(E)

Contains the error value

# Enum Wifi Auto Event Copy item path

Path: wifi_auto/enum.WifiAutoEvent.html

device_envoy_rp

::

wifi_auto

# Enum Wifi Auto Event Copy item path

pub enum WifiAutoEvent {
    CaptivePortalReady,
    Connecting {
        try_index:
u8
,
        try_count:
u8
,
    },
    ConnectionFailed,
}

Expand description

Events emitted while connecting.

See WifiAuto::connect for usage examples.

## Variants §

§

### CaptivePortalReady

Captive portal is ready and waiting for user configuration.

§

### Connecting

Attempting to connect to WiFi network.

#### Fields

§ try_index: u8

Current attempt number (0-based).

§ try_count: u8

Total number of attempts that will be made.

§

### ConnectionFailed

Connection failed after all attempts, device will reset.

## Trait Implementations §

§

### impl Clone for WifiAutoEvent

§

#### fn clone (&self) -> WifiAutoEvent

Returns a duplicate of the value.

Read more

1.0.0 · Source

§

#### fn clone_from (&mut self, source: &Self)

Performs copy-assignment from

source

.

Read more

§

### impl Debug for WifiAutoEvent

§

#### fn fmt (&self, f: &mut Formatter <'_>) -> Result < () , Error >

Formats the value using the given formatter.

Read more

§

### impl PartialEq for WifiAutoEvent

§

#### fn eq (&self, other: & WifiAutoEvent ) -> bool

Tests for

self

and

other

values to be equal, and is used by

==

.

1.0.0 · Source

§

#### fn ne (&self, other: &Rhs ) -> bool

Tests for

!=

. The default implementation is almost always sufficient, and should not be overridden without very good reason.

§

### impl Copy for WifiAutoEvent

§

### impl Eq for WifiAutoEvent

§

### impl StructuralPartialEq for WifiAutoEvent

## Auto Trait Implementations §

§

### impl Freeze for WifiAutoEvent

§

### impl RefUnwindSafe for WifiAutoEvent

§

### impl Send for WifiAutoEvent

§

### impl Sync for WifiAutoEvent

§

### impl Unpin for WifiAutoEvent

§

### impl UnwindSafe for WifiAutoEvent

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> CloneToUninit for T where T: Clone ,

Source

§

#### unsafe fn clone_to_uninit (&self, dest: *mut u8 )

🔬 This is a nightly-only experimental API. ( clone_to_uninit )

Performs copy-assignment from

self

to

dest

.

Read more

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module fields Copy item path

Path: wifi_auto/fields/index.html

device_envoy_rp

::

wifi_auto

# Module fields Copy item path

Source

Expand description

A device abstraction for extra setup fields used by WifiAutoRp .

See the WifiAutoRp struct example for the full setup.

This module provides ready-to-use field types that can be passed to WifiAutoRp::new for collecting additional configuration beyond WiFi credentials.

There are two levels of customization:

Use built-in helpers like TextField and TimezoneField .

Define your own field type by implementing WifiAutoField . See example there.

## § Example

use
device_envoy_rp::{
    Error,
Result
,
    button::PressedTo,
    button_watch,
    flash_block::FlashBlockRp,
    wifi_auto::{WifiAuto
as _
, WifiAutoEvent, WifiAutoRp},
    wifi_auto::fields::{TextField, TextFieldStatic, TimezoneField, TimezoneFieldStatic},
};
button_watch!
{
    ButtonWatch13 {
        pin: PIN_13,
    }
}
async fn
example(
    spawner: embassy_executor::Spawner,
    p: embassy_rp::Peripherals,
) ->
Result
<()> {
let
[wifi_flash, website_flash, timezone_flash] = FlashBlockRp::new_array::<
3
>(p.FLASH)
?
;
static
WEBSITE_STATIC: TextFieldStatic<
32
> = TextField::new_static();
let
website_field = TextField::new(
&
WEBSITE_STATIC,
        website_flash,
"website"
,
"Website"
,
"google.com"
,
    );
static
TIMEZONE_STATIC: TimezoneFieldStatic = TimezoneField::new_static();
let
timezone_field = TimezoneField::new(
&
TIMEZONE_STATIC, timezone_flash);
let
button_watch13 = ButtonWatch13::new(p.PIN_13, PressedTo::Ground, spawner).
await
?
;
let
wifi_auto = WifiAutoRp::new(
        p.PIN_23,
        p.PIN_24,
        p.PIN_25,
        p.PIN_29,
        p.PIO0,
        p.DMA_CH0,
        wifi_flash,
"DeviceEnvoySetup"
,
        [website_field, timezone_field],
        spawner,
    )
?
;
let
_stack = wifi_auto
        .connect(
&mut *
button_watch13, |wifi_auto_event|
async move
{
match
wifi_auto_event {
                WifiAutoEvent::CaptivePortalReady => {}
                WifiAutoEvent::Connecting { .. } => {}
                WifiAutoEvent::ConnectionFailed => {}
            }
Ok
(())
        })
        .
await
?
;
let
_website = website_field.text()
?
.unwrap_or_default();
let
_offset_minutes = timezone_field
        .offset_minutes()
?
.ok_or(Error::MissingCustomWifiAutoField)
?
;
Ok
(())
}

## Structs §

Text Field A generic text input field for collecting user input during WiFi provisioning. Text Field Static Static for TextField . Timezone Field A timezone selection field for WiFi provisioning. Timezone Field Static Static for TimezoneField .

# Struct Text Field Copy item path

Path: wifi_auto/fields/struct.TextField.html

device_envoy_rp

::

wifi_auto :: fields

# Struct Text Field Copy item path

Source

pub struct TextField<const N:
usize
> {
/* private fields */
}

Expand description

A generic text input field for collecting user input during WiFi provisioning.

Presents a customizable text input box in the captive portal that validates and stores user-provided text in memory or flash.

Multiple TextField instances can be created with different labels and field names to collect additional configuration beyond WiFi credentials.

See the wifi_auto::fields module example for usage.

## Implementations §

Source

§

### impl<const N: usize > TextField <N>

Source

#### pub const fn new_static () -> TextFieldStatic <N>

Create static resources for TextField .

See the wifi_auto::fields module example for usage.

Source

#### pub fn new_in_memory ( text_field_static: &'static TextFieldStatic <N>, field_name: &'static str , label: &'static str , default_value: &'static str , ) -> &'static Self

Initialize a text field backed by in-memory state.

The maximum text length is determined by the generic parameter N .

Parameters:

text_field_static : Static resources for initialization.

field_name : HTML form field name (for example, device_name ).

label : HTML label text shown in the captive portal form.

default_value : Initial value if nothing has been configured yet.

See the wifi_auto::fields module example for usage.

Source

#### pub fn new_with_flash ( text_field_static: &'static TextFieldStatic <N>, text_flash_block: FlashBlockRp , field_name: &'static str , label: &'static str , default_value: &'static str , ) -> &'static Self

Initialize a text field backed by a flash block.

The maximum text length is determined by the generic parameter N .

Parameters:

text_field_static : Static resources for initialization.

text_flash_block : Flash block for persistent storage.

field_name : HTML form field name (for example, device_name ).

label : HTML label text shown in the captive portal form.

default_value : Initial value if nothing has been configured yet.

See the wifi_auto::fields module example for usage.

Source

#### pub fn text (&self) -> Result < Option <String<N>>, Error >

Return the current text, if present.

Returns None when no text has been configured yet.

See the wifi_auto::fields module example for usage.

Source

#### pub fn set_text (&self, text: & str ) -> Result < () , Error >

Set the current text.

This allows programmatic updates to the field value.

The text must be non-empty and fit within the maximum length N .

See the wifi_auto::fields module example for usage.

Source

#### pub fn clear (&self) -> Result < () , Error >

Clear the current text.

Source

§

### impl<const N: usize > TextField <N>

Source

#### pub fn new ( text_field_static: &'static TextFieldStatic <N>, text_flash_block: FlashBlockRp , field_name: &'static str , label: &'static str , default_value: &'static str , ) -> &'static Self

Initialize a text field backed by a flash block.

See the WifiAutoRp struct example for usage.

## Trait Implementations §

Source

§

### impl<const N: usize > WifiAutoField for TextField <N>

Source

§

#### type Error = Error

Platform crate error type used by field implementations.

Source

§

#### fn render (&self, page: &mut HtmlBuffer) -> Result < () , Error >

Render form HTML elements.

Source

§

#### fn parse (&self, form: &FormData<'_>) -> Result < () , Error >

Parse submitted form data and typically persist the field value.

Source

§

#### fn is_satisfied (&self) -> Result < bool , Error >

Whether this field currently has valid configured data.

Source

§

### impl<const N: usize > Sync for TextField <N>

## Auto Trait Implementations §

§

### impl<const N: usize > ! Freeze for TextField <N>

§

### impl<const N: usize > ! RefUnwindSafe for TextField <N>

§

### impl<const N: usize > Send for TextField <N>

§

### impl<const N: usize > Unpin for TextField <N>

§

### impl<const N: usize > ! UnwindSafe for TextField <N>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Text Field Static Copy item path

Path: wifi_auto/fields/struct.TextFieldStatic.html

device_envoy_rp

::

wifi_auto :: fields

# Struct Text Field Static Copy item path

Source

pub struct TextFieldStatic<const N:
usize
> {
/* private fields */
}

Expand description

Static for TextField .

See the wifi_auto::fields module example for usage.

## Auto Trait Implementations §

§

### impl<const N: usize > ! Freeze for TextFieldStatic <N>

§

### impl<const N: usize > ! RefUnwindSafe for TextFieldStatic <N>

§

### impl<const N: usize > Send for TextFieldStatic <N>

§

### impl<const N: usize > Sync for TextFieldStatic <N>

§

### impl<const N: usize > Unpin for TextFieldStatic <N>

§

### impl<const N: usize > ! UnwindSafe for TextFieldStatic <N>

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Timezone Field Copy item path

Path: wifi_auto/fields/struct.TimezoneField.html

device_envoy_rp

::

wifi_auto :: fields

# Struct Timezone Field Copy item path

Source

pub struct TimezoneField {
/* private fields */
}

Expand description

A timezone selection field for WiFi provisioning.

Allows users to select their timezone from a dropdown during captive-portal setup. The selected offset (in minutes from UTC) is persisted in memory or flash and can be retrieved later by application code.

See the wifi_auto::fields module example for usage.

## Implementations §

Source

§

### impl TimezoneField

Source

#### pub const fn new_static () -> TimezoneFieldStatic

Create static resources for TimezoneField .

See the wifi_auto::fields module example for usage.

Source

#### pub fn new_in_memory ( timezone_field_static: &'static TimezoneFieldStatic , ) -> &'static Self

Initialize a timezone field backed by in-memory state.

See the wifi_auto::fields module example for usage.

Source

#### pub fn new_with_flash ( timezone_field_static: &'static TimezoneFieldStatic , timezone_flash_block: FlashBlockRp , ) -> &'static Self

Initialize a timezone field backed by a flash block.

See the wifi_auto::fields module example for usage.

Source

#### pub fn offset_minutes (&self) -> Result < Option < i32 >, Error >

Load the stored timezone offset in minutes from UTC.

Returns None if no timezone has been configured yet.

See the wifi_auto::fields module example for usage.

Source

#### pub fn set_offset_minutes (&self, offset_minutes: i32 ) -> Result < () , Error >

Save a timezone offset in minutes from UTC.

This allows programmatic updates to timezone settings.

For flash-backed fields, this only writes when the value changes to reduce unnecessary flash wear.

See the wifi_auto::fields module example for usage.

Source

#### pub fn clear (&self) -> Result < () , Error >

Clear the stored timezone offset.

See the wifi_auto::fields module example for usage.

Source

§

### impl TimezoneField

Source

#### pub fn new ( timezone_field_static: &'static TimezoneFieldStatic , timezone_flash_block: FlashBlockRp , ) -> &'static Self

Initialize a timezone field backed by a flash block.

See the WifiAutoRp struct example for usage.

## Trait Implementations §

Source

§

### impl WifiAutoField for TimezoneField

Source

§

#### type Error = Error

Platform crate error type used by field implementations.

Source

§

#### fn render (&self, page: &mut HtmlBuffer) -> Result < () , Error >

Render form HTML elements.

Source

§

#### fn parse (&self, form: &FormData<'_>) -> Result < () , Error >

Parse submitted form data and typically persist the field value.

Source

§

#### fn is_satisfied (&self) -> Result < bool , Error >

Whether this field currently has valid configured data.

Source

§

### impl Sync for TimezoneField

## Auto Trait Implementations §

§

### impl ! Freeze for TimezoneField

§

### impl ! RefUnwindSafe for TimezoneField

§

### impl Send for TimezoneField

§

### impl Unpin for TimezoneField

§

### impl ! UnwindSafe for TimezoneField

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Struct Timezone Field Static Copy item path

Path: wifi_auto/fields/struct.TimezoneFieldStatic.html

device_envoy_rp

::

wifi_auto :: fields

# Struct Timezone Field Static Copy item path

Source

pub struct TimezoneFieldStatic {
/* private fields */
}

Expand description

Static for TimezoneField .

See the wifi_auto::fields module example for usage.

## Auto Trait Implementations §

§

### impl ! Freeze for TimezoneFieldStatic

§

### impl ! RefUnwindSafe for TimezoneFieldStatic

§

### impl Send for TimezoneFieldStatic

§

### impl Sync for TimezoneFieldStatic

§

### impl Unpin for TimezoneFieldStatic

§

### impl ! UnwindSafe for TimezoneFieldStatic

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Module wifi_ auto Copy item path

Path: wifi_auto/index.html

device_envoy_rp

# Module wifi_ auto Copy item path

Source

Expand description

A device abstraction that connects a Pico with WiFi to the Internet and, when needed, creates a temporary WiFi network to enter credentials.

See WifiAutoRp for the main struct and usage examples.

## Modules §

fields A device abstraction for extra setup fields used by WifiAutoRp .

## Structs §

Wifi Auto Rp A device abstraction that connects a Pico with WiFi to the Internet and, when needed, creates a temporary WiFi network to enter credentials.

## Enums §

Wifi Auto Event Events emitted while connecting.

## Traits §

Wifi Auto Platform-agnostic Wi-Fi auto-connect contract. Wifi Auto Field Shared trait for custom setup fields rendered and parsed by the captive portal.

## Type Aliases §

Wifi Stack Canonical network stack type returned by WifiAuto::connect .

# Struct Wifi Auto Rp Copy item path

Path: wifi_auto/struct.WifiAutoRp.html

device_envoy_rp

::

wifi_auto

# Struct Wifi Auto Rp Copy item path

Source

pub struct WifiAutoRp {
/* private fields */
}

Expand description

A device abstraction that connects a Pico with WiFi to the Internet and, when needed, creates a temporary WiFi network to enter credentials.

WifiAutoRp handles WiFi connections end-to-end. It normally connects using a saved WiFi network name (SSID) and password. If those values are missing or invalid, it temporarily creates its own WiFi network (a “captive portal”) and hosts a web form where the user can enter the local WiFi ssid and password.

WifiAutoRp works on the Pico 1 W and Pico 2 W, which include the CYW43 WiFi chip.

The typical usage pattern is:

Ensure your hardware includes a button wired to a GPIO. The button can be used during boot to force captive-portal mode.

Construct a ButtonWatch to control the physical button.

Construct a FlashBlockRp to store WiFi credentials.

Use WifiAutoRp::new to construct a WifiAutoRp .

Use WifiAuto::connect to connect to WiFi while optionally showing status.

Let’s look at an example. Following the example, we’ll explain the details. (For additional examples, see the wifi_auto::fields module example and the WifiAuto::connect docs.)

### § Example: Connect with logging

This example connects to WiFi and logs progress.

use
device_envoy_rp::{
Result
,
    button::PressedTo,
    button_watch,
    flash_block::FlashBlockRp,
    wifi_auto::{WifiAuto
as _
, WifiAutoEvent, WifiAutoRp},
};
use
embassy_time::Duration;
button_watch!
{
    ButtonWatch13 {
        pin: PIN_13,
    }
}
async fn
connect_wifi(
    spawner: embassy_executor::Spawner,
    p: embassy_rp::Peripherals,
) ->
Result
<()> {
// Set up ButtonWatch to control the physical button.
let
button_watch13 = ButtonWatch13::new(p.PIN_13, PressedTo::Ground, spawner).
await
?
;
// Set up flash storage for WiFi credentials.
let
[wifi_flash] = FlashBlockRp::new_array::<
1
>(p.FLASH)
?
;
// Construct WifiAutoRp
let
wifi_auto = WifiAutoRp::new(
        p.PIN_23,
// CYW43 power
p.PIN_24,
// CYW43 clock
p.PIN_25,
// CYW43 chip select
p.PIN_29,
// CYW43 data
p.PIO0,
// WiFi PIO
p.DMA_CH0,
// WiFi DMA
wifi_flash,
"DeviceEnvoySetup"
,
// Captive-portal SSID
[],
// Any extra fields
spawner,
    )
?
;
// Connect (logging status as we go)
let
stack = wifi_auto
        .connect(
&mut *
button_watch13, |event|
async move
{
match
event {
                WifiAutoEvent::CaptivePortalReady =>
info!
(
"Captive portal ready"
),
                WifiAutoEvent::Connecting { .. } =>
info!
(
"Connecting to WiFi"
),
                WifiAutoEvent::ConnectionFailed =>
info!
(
"WiFi connection failed"
),
            }
Ok
(())
        })
        .
await
?
;
info!
(
"WiFi connected"
);
loop
{
if let
Ok
(addresses) = stack.dns_query(
"google.com"
, embassy_net::dns::DnsQueryType::A).
await
{
info!
(
"google.com: {:?}"
, addresses);
        }
else
{
info!
(
"google.com: lookup failed"
);
        }
        embassy_time::Timer::after(Duration::from_secs(
15
)).
await
;
    }
}

### § What happens during connection

While connect is running:

The WiFi chip may reset as it switches between normal WiFi operation and hosting its own temporary WiFi network.

Your code should tolerate these resets. Initializing LEDs or displays before WiFi is fine; just be aware they may be momentarily disrupted during mode changes.

### § WiFi limitations

Only standard SSID/password 2.4 GHz WiFi networks are supported.

### § Hardware model

On the Pico W, the CYW43 WiFi chip is wired to fixed GPIOs.

## Implementations §

Source

§

### impl WifiAutoRp

Source

#### pub fn new <const N: usize , PIO: WifiPio, DMA: ChannelInstance, FlashBlockType>( pin_23: Peri<'static, PIN_23>, pin_24: Peri<'static, PIN_24>, pin_25: Peri<'static, PIN_25>, pin_29: Peri<'static, PIN_29>, pio: Peri<'static, PIO>, dma: Peri<'static, DMA>, wifi_credentials_flash_block: FlashBlockType, captive_portal_ssid: &'static str , custom_fields: [&'static (dyn WifiAutoField <Error = Error > + Sync ); N ], spawner: Spawner, ) -> Result <Self> where FlashBlockType: FlashBlock <Error = Error > + Into < FlashBlockRp >, DmaAllIrqs: Binding<DMA::Interrupt, InterruptHandler<DMA>>,

Initialize WiFi auto-provisioning with custom configuration fields.

#### § Parameters

pin_23 , pin_24 , pin_25 , pin_29 : the internal GPIO pins for the CYW43 WiFi chip.

pio : PIO resource used for WiFi.

dma : DMA resource for WiFi.

wifi_credentials_flash_block : a flash block implementing FlashBlock for WiFi credentials.

captive_portal_ssid : SSID shown when the device starts setup mode.

custom_fields : Extra fields collected in the setup page. See the wifi_auto::fields module example for usage.

spawner : Embassy task spawner for background work.

See the WifiAutoRp struct example for a complete example.

Source

#### pub async fn connect <OnEvent, OnEventFuture>( self, button: &mut impl Button , on_event: OnEvent, ) -> Result < WifiStack > where OnEvent: FnMut ( WifiAutoEvent ) -> OnEventFuture, OnEventFuture: Future <Output = Result < () >>,

Connect to Wi-Fi using a caller-provided button reference.

## Trait Implementations §

Source

§

### impl WifiAuto for WifiAutoRp

Source

§

#### async fn connect <OnEvent, OnEventFuture>( self, button: &mut impl Button , on_event: OnEvent, ) -> Result < WifiStack > where OnEvent: FnMut ( WifiAutoEvent ) -> OnEventFuture, OnEventFuture: Future <Output = Result < () >>,

Connects to WiFi (if possible), reports status, and returns the network stack, consuming the WifiAutoRp .

See the WifiAutoRp struct example for a usage example.

This method does not return until WiFi is connected. It may briefly restart the Pico while switching between normal WiFi operation and hosting its temporary setup network.

This connect method reports progress by calling a user-provided async handler whenever the WiFi state changes. The handler receives a WifiAutoEvent . The handler is called sequentially for each event and may await .

The three events are:

CaptivePortalReady : The device is hosting a captive portal and waiting for user input.

Connecting : The device is attempting to connect to the WiFi network.

ConnectionFailed : All connection attempts failed. The device will reset and re-enter setup mode (for example, if the password is incorrect).

The first example uses a handler that does nothing. The second example shows how to use an LED panel to display status messages. The example on the WifiAutoRp struct shows simple logging.

#### § Example 1: No-op event handler

let
_stack = wifi_auto
    .connect(
&mut *
button_watch13, |_event|
async move
{
Ok
(()) })
    .
await
?
;

#### § Example 2: Using a display to show status

// Keep a reference so the handler can reuse the display across events.
let
led8x12_ref =
&
led8x12;
let
stack = wifi_auto
    .connect(
&mut *
button_watch13, |event|
async move
{
match
event {
            WifiAutoEvent::CaptivePortalReady => {
                led8x12_ref.write_text(
"JO\nIN"
, COLORS);
            }
            WifiAutoEvent::Connecting { .. } => {
                show_animated_dots(led8x12_ref).
await
?
;
            }
            WifiAutoEvent::ConnectionFailed => {
                led8x12_ref.write_text(
"FA\nIL"
, COLORS);
            }
        }
Ok
(())
    })
    .
await
?
;

Source

§

#### type Error = Error

Platform-specific error type.

## Auto Trait Implementations §

§

### impl Freeze for WifiAutoRp

§

### impl ! RefUnwindSafe for WifiAutoRp

§

### impl ! Send for WifiAutoRp

§

### impl ! Sync for WifiAutoRp

§

### impl Unpin for WifiAutoRp

§

### impl ! UnwindSafe for WifiAutoRp

## Blanket Implementations §

Source

§

### impl<T> Any for T where T: 'static + ? Sized ,

Source

§

#### fn type_id (&self) -> TypeId

Gets the

TypeId

of

self

.

Read more

Source

§

### impl<T> Az for T

Source

§

#### fn az <Dst>(self) -> Dst where T: Cast <Dst>,

Casts the value.

Source

§

### impl<T> Borrow <T> for T where T: ? Sized ,

Source

§

#### fn borrow (&self) -> &T

Immutably borrows from an owned value.

Read more

Source

§

### impl<T> BorrowMut <T> for T where T: ? Sized ,

Source

§

#### fn borrow_mut (&mut self) -> &mut T

Mutably borrows from an owned value.

Read more

Source

§

### impl<Src, Dst> CastFrom <Src> for Dst where Src: Cast <Dst>,

Source

§

#### fn cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> CheckedAs for T

Source

§

#### fn checked_as <Dst>(self) -> Option <Dst> where T: CheckedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> CheckedCastFrom <Src> for Dst where Src: CheckedCast <Dst>,

Source

§

#### fn checked_cast_from (src: Src) -> Option <Dst>

Casts the value.

Source

§

### impl<T> From <T> for T

Source

§

#### fn from (t: T) -> T

Returns the argument unchanged.

Source

§

### impl<T, U> Into <U> for T where U: From <T>,

Source

§

#### fn into (self) -> U

Calls U::from(self) .

That is, this conversion is whatever the implementation of From <T> for U chooses to do.

Source

§

### impl<Src, Dst> LosslessTryInto <Dst> for Src where Dst: LosslessTryFrom <Src>,

Source

§

#### fn lossless_try_into (self) -> Option <Dst>

Performs the conversion.

Source

§

### impl<Src, Dst> LossyInto <Dst> for Src where Dst: LossyFrom <Src>,

Source

§

#### fn lossy_into (self) -> Dst

Performs the conversion.

Source

§

### impl<T> OverflowingAs for T

Source

§

#### fn overflowing_as <Dst>(self) -> (Dst, bool ) where T: OverflowingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> OverflowingCastFrom <Src> for Dst where Src: OverflowingCast <Dst>,

Source

§

#### fn overflowing_cast_from (src: Src) -> (Dst, bool )

Casts the value.

Source

§

### impl<T> Same for T

Source

§

#### type Output = T

Should always be

Self

Source

§

### impl<T> SaturatingAs for T

Source

§

#### fn saturating_as <Dst>(self) -> Dst where T: SaturatingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> SaturatingCastFrom <Src> for Dst where Src: SaturatingCast <Dst>,

Source

§

#### fn saturating_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> StrictAs for T

Source

§

#### fn strict_as <Dst>(self) -> Dst where T: StrictCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> StrictCastFrom <Src> for Dst where Src: StrictCast <Dst>,

Source

§

#### fn strict_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T, U> TryFrom <U> for T where U: Into <T>,

Source

§

#### type Error = Infallible

The type returned in the event of a conversion error.

Source

§

#### fn try_from (value: U) -> Result <T, <T as TryFrom <U>>:: Error >

Performs the conversion.

Source

§

### impl<T, U> TryInto <U> for T where U: TryFrom <T>,

Source

§

#### type Error = <U as TryFrom <T>>:: Error

The type returned in the event of a conversion error.

Source

§

#### fn try_into (self) -> Result <U, <U as TryFrom <T>>:: Error >

Performs the conversion.

Source

§

### impl<T> UnwrappedAs for T

Source

§

#### fn unwrapped_as <Dst>(self) -> Dst where T: UnwrappedCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> UnwrappedCastFrom <Src> for Dst where Src: UnwrappedCast <Dst>,

Source

§

#### fn unwrapped_cast_from (src: Src) -> Dst

Casts the value.

Source

§

### impl<T> WrappingAs for T

Source

§

#### fn wrapping_as <Dst>(self) -> Dst where T: WrappingCast <Dst>,

Casts the value.

Source

§

### impl<Src, Dst> WrappingCastFrom <Src> for Dst where Src: WrappingCast <Dst>,

Source

§

#### fn wrapping_cast_from (src: Src) -> Dst

Casts the value.

# Trait Wifi Auto Copy item path

Path: wifi_auto/trait.WifiAuto.html

device_envoy_rp

::

wifi_auto

# Trait Wifi Auto Copy item path

pub trait WifiAuto {
    type
Error
;

    // Required method
    async fn
connect
<OnEvent, OnEventFuture>(
        self,
        button: &mut impl
Button
,
        on_event: OnEvent,
    ) ->
Result
<&'static Stack<'static>, Self::
Error
>
where OnEvent:
FnMut
(
WifiAutoEvent
) -> OnEventFuture,
             OnEventFuture:
Future
<Output =
Result
<
()
, Self::
Error
>>
;
}

Expand description

Platform-agnostic Wi-Fi auto-connect contract.

Platform crates implement this trait for their concrete runtime handles. Constructors remain inherent on platform types.

WifiAuto::connect handles Wi-Fi setup end-to-end. It usually tries saved credentials first, and if credentials are missing or invalid it can run a captive portal so the user can submit new credentials.

While connect runs, on_event receives progress updates:

WifiAutoEvent::CaptivePortalReady : captive portal is ready for user input.

WifiAutoEvent::Connecting : a Wi-Fi connection attempt is in progress.

WifiAutoEvent::ConnectionFailed : all connection attempts failed.

## § Example

use
core::future::Future;
use
core::convert::Infallible;
use
device_envoy_core::{
    button::Button,
    wifi_auto::{WifiAuto, WifiAutoEvent, WifiStack},
};
async fn
connect_with_status(
    wifi_auto:
impl
WifiAuto<Error = Infallible>,
) ->
Result
<WifiStack, Infallible> {
    wifi_auto
        .connect(
&mut
ButtonMock, |wifi_auto_event|
async move
{
match
wifi_auto_event {
                WifiAutoEvent::CaptivePortalReady => {
// Captive portal is ready for Wi-Fi credential entry.
}
                WifiAutoEvent::Connecting { .. } => {
// A Wi-Fi connection attempt is in progress.
}
                WifiAutoEvent::ConnectionFailed => {
// All connection attempts failed.
}
            }
Ok
(())
        })
        .
await
}

## Required Associated Types §

#### type Error

Platform-specific error type.

## Required Methods §

#### async fn connect <OnEvent, OnEventFuture>( self, button: &mut impl Button , on_event: OnEvent, ) -> Result <&'static Stack<'static>, Self:: Error > where OnEvent: FnMut ( WifiAutoEvent ) -> OnEventFuture, OnEventFuture: Future <Output = Result < () , Self:: Error >>,

Connect to Wi-Fi, emitting progress events to on_event .

See the WifiAuto trait documentation for usage examples.

## Dyn Compatibility §

This trait is not dyn compatible .

In older versions of Rust, dyn compatibility was called "object safety", so this trait is not object safe.

## Implementors §

Source

§

### impl WifiAuto for WifiAutoRp

Source

§

#### type Error = Error

# Trait Wifi Auto Field Copy item path

Path: wifi_auto/trait.WifiAutoField.html

device_envoy_rp

::

wifi_auto

# Trait Wifi Auto Field Copy item path

pub trait WifiAutoField {
    type
Error
;

    // Required methods
    fn
render
(&self, page: &mut HtmlBuffer) ->
Result
<
()
, Self::
Error
>;
fn
parse
(&self, form: &FormData<'_>) ->
Result
<
()
, Self::
Error
>;

    // Provided method
    fn
is_satisfied
(&self) ->
Result
<
bool
, Self::
Error
> { ... }
}

Expand description

Shared trait for custom setup fields rendered and parsed by the captive portal.

This is the platform-agnostic contract used by both RP and ESP ports.

To define your own field type, implement:

WifiAutoField::render to write the HTML form UI.

WifiAutoField::parse to read submitted form values and typically persist them.

WifiAutoField::is_satisfied when setup should require the field.

For ready-to-use field types, see platform modules: device_envoy_rp::wifi_auto::fields and device_envoy_esp::wifi_auto::fields .

## § Example

This example mirrors the platform example wifi_auto_custom_checkbox.rs , but keeps everything core-only. It shows a checkbox field that:

renders a checkbox in the captive portal,

parses submitted form data,

stores the latest value in field-local state, and

requires at least one explicit user choice before setup is considered complete.

use
core::{cell::Cell, convert::Infallible, fmt::Write
as _
};
use
device_envoy_core::wifi_auto::{
    FormData, HtmlBuffer, WifiAutoField, parse_post,
};
// Custom field type used by WifiAuto setup.
struct
CheckboxField {
// HTML form key.
field_name:
&
'static
str,
// Label shown next to the checkbox.
label:
&
'static
str,
// Render-only fallback before the first submit.
default_checked: bool,
// Tri-state: None = never configured, Some(true/false) = explicit user choice.
checked: Cell<
Option
<bool>>,
}
impl
CheckboxField {
const fn
new(
        field_name:
&
'static
str,
        label:
&
'static
str,
        default_checked: bool,
    ) ->
Self
{
Self
{
            field_name,
            label,
            default_checked,
            checked: Cell::new(
None
),
        }
    }
fn
checked(
&
self
) ->
Option
<bool> {
self
.checked.get()
    }
// UI fallback only; setup completeness is driven by `checked()`.
fn
checked_or_default(
&
self
) -> bool {
self
.checked().unwrap_or(
self
.default_checked)
    }
}
impl
WifiAutoField
for
CheckboxField {
type
Error = Infallible;
fn
render(
&
self
, page:
&mut
HtmlBuffer) ->
Result
<(),
Self
::Error> {
// render() controls what appears on the captive-portal page.
let
checked_attribute =
if
self
.checked_or_default() {
" checked"
}
else
{
""
};
let _
=
write!
(
            page,
"<p><label><input type=\"checkbox\" name=\"{}\" value=\"1\"{}> {}</label></p>"
,
self
.field_name, checked_attribute,
self
.label
        );
Ok
(())
    }
fn
parse(
&
self
, form:
&
FormData<
'_
>) ->
Result
<(),
Self
::Error> {
// parse() reads the submitted field value.
let
checked =
matches!
(
            form.get(
self
.field_name),
Some
(
"1"
) |
Some
(
"on"
) |
Some
(
"true"
)
        );
// This example persists to in-memory field state; platform fields can
        // persist to flash instead.
self
.checked.set(
Some
(checked));
Ok
(())
    }
fn
is_satisfied(
&
self
) ->
Result
<bool,
Self
::Error> {
// Require at least one explicit submit before provisioning is complete.
Ok
(
self
.checked().is_some())
    }
}
fn
example() {
// Instantiate the custom field.
let
checkbox_field = CheckboxField::new(
"share_telemetry"
,
"Share anonymous telemetry"
,
false
,
    );
// Pass it to the same parse pipeline used by WifiAuto internals.
let
fields: [
&
dyn
WifiAutoField<Error = Infallible>;
1
] = [
&
checkbox_field];
// Simulate a captive-portal form submission.
let
request =
"POST / HTTP/1.1\r\nHost: 192.168.4.1\r\n\r\nssid=Office&password=abc123&share_telemetry=1"
;
let
_wifi_credentials = parse_post::<Infallible>(request,
None
,
&
fields);
}

## Required Associated Types §

#### type Error

Platform crate error type used by field implementations.

## Required Methods §

#### fn render (&self, page: &mut HtmlBuffer) -> Result < () , Self:: Error >

Render form HTML elements.

#### fn parse (&self, form: &FormData<'_>) -> Result < () , Self:: Error >

Parse submitted form data and typically persist the field value.

## Provided Methods §

#### fn is_satisfied (&self) -> Result < bool , Self:: Error >

Whether this field currently has valid configured data.

## Implementors §

Source

§

### impl WifiAutoField for TimezoneField

Source

§

#### type Error = Error

Source

§

### impl<const N: usize > WifiAutoField for TextField <N>

Source

§

#### type Error = Error

# Type Alias Wifi Stack Copy item path

Path: wifi_auto/type.WifiStack.html

device_envoy_rp

::

wifi_auto

# Type Alias Wifi Stack Copy item path

pub type WifiStack = &'static Stack<'static>;

Expand description

Canonical network stack type returned by WifiAuto::connect .